3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf               	R4-1808897

Montreal, Canada, 2 July – 6 July 2018

Agenda item:	4.1.1.1

Source:	Samsung

Title:	Simulation results for SPUSCH

Document for:	Discussion

# 1	Introduction

During RAN4#86b meeting, WF about BS demodulation performance requirements was approved [1]. As for the sPUSCH performance part, the simulation assumption as follows in Table1:

Table 1: Simulation assumption for sPUSCH

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| Fraction of maximum throughput | 70% |
| Maximum number of HARQ transmissions | 4 |
| DMRS configuration | IFDMA is not configured |
| Number of HARQ processes | 16 |
| RV sequence | 0, 2, 3, 1, 0, 2, 3, 1 |
| Modulation | 16QAM |

As for the open issue about DMRS sharing pattern, the option 1 have been agreed in last meeting as follow []:

  - Option 1:

    - With DMRS sharing  RDD DD DD RD DD RDD

In this contribution, based on above agreements, we provide our simulation results on sPUSCH for alignment, the impairment results are also provided.

# 2	Simulation Results

Based on the agreed WF and simulation assumption and the DMRS sharing pattern “RDD DD DD RD DD RDD”, performance of sPUSCH is provided, where the alignment results and impairment results are indicated in Table 2 and Table 3.

Table 2: Alignment results for sPUSCH

| Tx | Rx | Propagation condition | Number of RB | SNR@70% of max throughput |
| --- | --- | --- | --- | --- |
| 1 | 2 | EPA 5Hz | 24 | 10.25 |
|  |  |  | 48 | 10.16 |
|  |  |  | 72 | 10.15 |
|  |  |  | 100 | 10.51 |
| 1 | 4 | EPA 5Hz | 24 | 6.41 |
|  |  |  | 48 | 6.47 |
|  |  |  | 72 | 6.37 |
|  |  |  | 100 | 6.65 |

Table 3: Impairment results for sPUSCH

| Tx | Rx | Propagation condition | Number of RB | SNR@70% of max throughput |
| --- | --- | --- | --- | --- |
| 1 | 2 | EPA 5Hz | 24 | 12.25 |
|  |  |  | 48 | 12.16 |
|  |  |  | 72 | 12.15 |
|  |  |  | 100 | 12.51 |
| 1 | 4 | EPA 5Hz | 24 | 8.41 |
|  |  |  | 48 | 8.47 |
|  |  |  | 72 | 8.37 |
|  |  |  | 100 | 8.65 |

For Table 2, we can observe that the target SNR value at the 70% maximum throughput for different number of RB is similar. Compared with 4Rx, there is about 3.8dB performance gain with 2Rx.

# 3	Conclusion

In this contrition, both alignment results and impairment results of sPUSCH with DMRS sharing pattern “RDD DD DD RD DD RDD” are provided.

# References

R4-1805494, WF on sTTI BS demodulation requirements, Ericsson, Huawei, Nokia

R4-1805495, Simulation assumption for BS demodulation requirements, Ericsson

R4-1807977, Adhoc meeting minutes for LTE UE/BS demodulation for multiple features, Ericsson