3GPP TSG-RAN WG4 Meeting AH-1807                                                        R4-180xxxx

Montreal ,Canada, 2nd  – 6th July, 2018

Agenda Item:   5.4.1.5

Source:        Samsung

Title:          Views on CSI test scope

Document for:  Discussion

Introduction

A dedicated specification was assigned for NR UE performance requirements as 38.101-4. The target competition time for Rel-15 NR performance part is RAN#82 in Dec 2018. In this contribution, preliminary views for UE CSI test scope were provided.

Discussion

Following RAN1 feature list, below features agreed as mandatory for basic CSI feedback:

| # in RAN1 feature list | Feature group | Components |
| --- | --- | --- |
| 2-32 | Basic CSI feedback | 1. Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) |
|  |  | 2. 2Tx codebook for FR1 and FR2 |
|  |  | 3. 4Tx codebook for FR1 |
|  |  | 4. 8Tx codebook for FR1 when configured as wideband CSI report |
|  |  | 5. p-CSI on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) |
|  |  | 6. p-CSI report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) |
|  |  | 7. a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately) |
|  |  | further check a-CSI on p-CSI-RS and/or SP-CSI-RS from component-7 |

It’s unrealistic to verify all the features, functionality and different configurations specified in CSI framework considering work load and tight timeline for Rel-15 NR performance requirements. Also considering diverse capability values for different feature groups, it’s impractical to introduce test cases covering all UE capabilities.

P1: Introducing CSI performance requirements focused on the mandatory and some fundamental features of NR CSI framework under Rel-15 timeframe.

CSI framework

NR_MIMO was designed to further improve performance and enable deployment in high frequency i.e. above 3GHz in FR1 and mmWave bands in FR2 with multi beam management. CSI framework in NR can be considered as an enhancement based on the design for Rel-14 eFD-MIMO CSI to facilitate hybrid beam operation of. Modular CSI framework was introduced in a future proof manner which consists of CSI-RS resources settings, CSI reporting settings and links configurations between them for measurement list.

CSI reporting/CSI-RS resource setting

For CSI reporting setting/CSI-RS resource setting, ‘aperiodic’, and ‘periodic’ and ‘semi-persistent’ types are supported for NR.

For CSI-RS resource setting, both CSI-IM resources and NZP CSI-RS resources can be configured for interference measurement.

Considering Semi-persistent CSI report and semi-persistent CSI-RS resource are optional in Rel-15, meanwhile from network scheduling flexibility perspective, supporting flexible and configurable CSI reporting types and CSI-RS resource setting is important, it’s better to cover all the types in Rel-15.

P2-1: CSI report types: In Rel-15, besides periodic CSI report and aperiodic CSI report, additional test case for semi-persistent report also need to be included.

P2-3: CSI-RS resource Type: In Rel-15, besides periodic aperiodic CSI-RS resources configuration, additional test case for semi-persistent CSI-RS resource need to be included.

P2-3: CSI-RS resources setting: Rel-15 focused on 1 NZP CSI-RS resource and 1 CSI-IM resource configuration for CSI report. CSI-IM used for interference measurement.

For number of configured CSI report, below capability signalling agreed:

P2-4: Number of CSI report and BWP: Rel-15 focused on single BWP/CC with 1 CSI reporting configured.

CSI reporting content

In NR, CSI consists of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), layer indication (LI), rank indication (RI) and/or L1-RSRP.

For CRI, it’s quite beneficial for Hybrid beam forming operation:

- Operation 1: Used for hybrid PMI reporting, combined with PMI report

- Operation 2: Used for beam management, combined with L1-RSRP report

In order to facilitate hybrid beam operation, we preferred to introduce both CRI/CSI test case and CRI/L1-RSRP test cases.

P3-1: Introduce both CRI/CSI test case and CRI/L1-RSRP test cases in Rel-15.

L1-RSRP

L1-RSPR was introduced in NR CSI framework for beam management. Unlike RSRP reporting in RRM, the reporting mechanism and trigger mechanism are different between RRM measurement and CSI measurement. L1-RSRP should be included in the scope of CSI performance requirements.

L1-RSRP reporting is essential feature al to facilitate beam management (Tx/Rx beam switching) for data and control channel transmission during RRC connected state. Follow agreed UE feature list, periodic and aperiodic L1-RSRP report is mandatory at least for FR2.

Proposal 3-2 : L1-RSRP belong to NR CSI framework, new requirements need to be introduced under CSI performance requirements agenda in Rel-15.

Conclusion

In this contribution, preliminary views for the scope of UE CSI requirements were provided.

Test scope

P1: Introducing CSI performance requirements focused on the mandatory and some fundamental features of NR CSI framework under Rel-15 timeframe.

CSI frame work

P2: In Rel-15, NR CSI test cases focused on below configurations:

- P2-1: CSI report types: In Rel-15, besides periodic CSI report and aperiodic CSI report, additional test case for semi-persistent report also need to be included.

- P2-3: CSI-RS resource Type: In Rel-15, besides periodic aperiodic CSI-RS resources configuration, additional test case for semi-persistent CSI-RS resource need to be included.

- P2-3: CSI-RS resources setting: Rel-15 focused on 1 NZP CSI-RS resource and 1 CSI-IM resource configuration for CSI report. CSI-IM used for interference measurement.

- P2-4: Number of CSI report and BWP: Rel-15 focused on single BWP/CC with 1 CSI reporting configured.

CSI reporting content

P3-1: Introduce both CRI/CSI test case and CRI/L1-RSRP test cases in Rel-15.

P3-2: L1-RSRP belong to NR CSI framework, new requirements need to be introduced under CSI performance requirements agenda in Rel-15.

Reference

[1] R4-1808027, “Way Forward on NR UE performance requirements”, Ericsson

[2] R1-1807891, “LS on NR UE feature list”