3GPP TSG-RAN WG4 Meeting RAN4#87                                                 R4-1808911

Monterall ,Korean, 21 -25 May, 2018

Agenda Item:   5.4.1.2

Source:        Samsung

Title:          Initial simulation results for NR PDSCH

Document for:  Discussion

Introduction

In last RAN4 meeting, several parameters for PDSCH demodulation test cases have been discussed and candidate options have been summarized in WF [1]. Through email discussion, below test cases were recommended for initial alignment purpose.

Table 1 List of simulation cases for FR1 FDD with 10 MHz + 15 kHz

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Channel Model |
| --- | --- | --- | --- | --- | --- |
| 1a,1b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |
| 2a,2b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 256QAM MCS 24 | Layer 1 | TDL-A 30ns, 10Hz |
| 3a,3b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 16QAM MCS 24 | Layer 2 | TDL-A 30ns, 10Hz |
| 4a,4b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 64QAM MCS 24 | Layer 2 | TDL-A 30ns, 10Hz |
| 5a | 10MHz/15kHz | 4Tx 2Rx Low | 16QAM MCS 24 | Layer 2 | TDL-A 30ns, 10Hz |
| 5b | 10MHz/15kHz | 4Tx 4Rx Low | 16QAM MCS 24 | Layer 4 | TDL-A 30ns, 10Hz |

Table 2 List of simulation cases for FR1 TDD covering different combinations of MIMO Tx Rx, MCS and layers

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Channel Model |
| --- | --- | --- | --- | --- | --- |
| 6a,6b | 40MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns,100Hz |
| 7a,7b | 40MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 256QAM MCS 24 | Layer 1 | TDL-A 30ns, 10Hz |
| 8a,8b | 40MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 16QAM MCS 13 | Layer 2 | TDL-A 30ns, 10Hz |
| 9a,9b | 40MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 64QAM MCS 24 | Layer 2 | TDL-A 30ns, 10Hz |
| 10a,10b | 40MHz/30kHz | 4Tx 2Rx Low | 16QAM MCS 13 | Layer 2 | TDL-A 30ns, 10Hz |
| 11 | 40MHz/30kHz | 4Tx 4Rx Low | 16QAM MCS 13 | Layer 4 | TDL-A 30ns, 10Hz |
| Note: TDD with 40MHz/30kHz and DL-UL configuration as DDDSU (default),S = 12D+1Gp+1U |  |  |  |  |  |

Table 3 List of simulation cases for FR1 TDD covering different CHBW and SCS, and different DL-UL configurations

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Channel Model |
| --- | --- | --- | --- | --- | --- |
| 12a,12b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |
| 13a,13b | 20MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |
| 14a,14b | 40MHz/30kHz with DL-UL 7D1S2U | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |
| 15a,15b | 40MHz/30kHz With DL-UL DSSU | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |

Table 4 List of simulation cases for FR2 TDD covering different MIMO Tx Rx, MCS and layers, CHBW and SCS, and different DL-UL configurations

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Channel Model |
| --- | --- | --- | --- | --- | --- |
| 16 | 100MHz/120kHz | 1Tx 2Rx Low | QPSK MCS 4 | Layer 1 | TBD |
| 17 | 100MHz/120kHz | 1Tx 2Rx Low | 64QAM MCS 24 | Layer 1 | TBD |
| 18 | 100MHz/120kHz | 2Tx 2Rx Low | 16QAM MCS 13 | Layer 2 | TBD |
| 19 | 100MHz/60kHz | 1Tx 2Rx Low | QPSK MCS 4 | Layer 1 | TBD |
| 20 | 200MHz/120kHz | 1Tx 2Rx Low | QPSK MCS 4 | Layer 1 | TBD |
| 21 | 100MHz/120kHz With DSDSU, S =10D+2Gp+2U | 1Tx 2Rx Low | QPSK MCS 4 | Layer 1 | TBD |
| Note 1:Except case 20 in table, all other cases as DDDSU |  |  |  |  |  |

Discussion

Followed above list tables, we provided initial simulation results with below assumption:

- HARQ process: 4 for FDD, 8 for TDD

- Non FDM for data and DMRS of layer 1 and layer 2 test

- MMSE-IRC applied

Simulation results for FDD in FR1

To be updated

Simulation results for TDD in FR1

To be updated

Simulation results for FR2

To be updated

Reference

[1] R4-1808027, “Way Forward on NR UE performance requirements”, Ericsson