3GPP TSG-RAN WG4 Meeting #AH1807	R4-1808927

Montreal, Canada, July 2nd –6th, 2018

Agenda item:			5.3.3.4

Source:	MediaTek Inc.

Title:	Discussion on RRM measurement accuracy performance and RF impairment in NR wide bandwidth

Document for:			Discussion

# 1	Introduction

In [1], the link-level simulation assumptions for SSB-based measurement accuracy was agreed. The measurement accuracy to be studied are listed as follows:

- SS-RSRP based on NR SSS,

- SS-RSRQ based on NR SSS, and

- SS-SINR based on NR SSS.

Besides, the SS-RSRP absolute and relative accuracies of intra-frequency measurement were also agreed in last meeting [2]. In FR1, similar with LTE spec, ±[4.5] dB RSRP intra-frequency absolute accuracy when side condition Es/IoT = -6 dB and ±[2] dB RSRP intra-frequency relative accuracy when side condition Es/IoT = -3 dB are re-used; however, the corresponding operating band groups are not decided yet.

In this contribution, the RSRP and SINR accuracies for both sub-6 and mmWave bands are investigated. The operating band groups of agreed RSRP intra-frequency absolute and relative accuracies are discussed. Considering the RF impairment in wider bandwidth (BW), the accuracies should be further studied when NR BW ≥ 20MHz.

# 2	RSRP simulation results

In this section, we compare the measurement accuracy of NR-SSS based RSRP on different sub-scarrier spacing (SCS), sample number (N) and channel type, with detail setting parameters shown in Section 7. Here, Table 1 and Table 3 summarizes the absolute accuracies and values of 5%, 50%,95% points on the CDF curves of Delta RSRP in FR1 and FR2, respectively. Table 2 and Table 4 summarizes the relative accuracies and values of 5%, 50%,95% points on the CDF curves of Delta RSRP in FR1 and FR2, respectively.

Table 1: NR-SSS based RSRP measurement absolute accuracy in FR1, side condition Es/IoT = -6 dB

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 15 kHz |  |  |  | SCS = 30 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy | 5% | 50% | 95% | Absolute accuracy |
| AWGN | 1 | -0.99 | 0.0 | 0.95 | 0.96 | -1.06 | -0.07 | 0.91 | 0.99 |
|  | 3 | -0.8 | -0.12 | 0.57 | 0.73 | -0.87 | -0.16 | 0.53 | 0.76 |
|  | 5 | -0.74 | -0.15 | 0.48 | 0.68 | -0.81 | -0.18 | 0.17 | 0.74 |
| EPA5 | 1 | -1.42 | -0.26 | 0.98 | 1.39 | -1.74 | -0.51 | 0.77 | 1.52 |
|  | 3 | -1.16 | -0.26 | 0.59 | 1.01 | -1.41 | -0.51 | 0.43 | 1.27 |
|  | 5 | -1.02 | -0.26 | 0.45 | 0.89 | -1.34 | -0.49 | 0.33 | 1.22 |
| ETU30 | 1 | -1.53 | -0.27 | 1.11 | 1.36 | N/A | N/A | N/A | N/A |
|  | 3 | -1.15 | -0.29 | 0.84 | 1.08 | N/A | N/A | N/A | N/A |
|  | 5 | -1.09 | -0.29 | 0.77 | 1 | N/A | N/A | N/A | N/A |
| ETU70 | 1 | -1.5 | -0.19 | 1.06 | 1.34 | N/A | N/A | N/A | N/A |
|  | 3 | -1.22 | -0.23 | 0.77 | 1.04 | N/A | N/A | N/A | N/A |
|  | 5 | -1.09 | -0.24 | 0.68 | 0.97 | N/A | N/A | N/A | N/A |

Table 2: NR-SSS based RSRP measurement relative accuracy in FR1, side condition Es/IoT = -3 dB

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 15 kHz |  |  |  | SCS = 30 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Relative accuracy | 5% | 50% | 95% | Relative accuracy |
| AWGN | 1 | -1.08 | 0.14 | 1.41 | 1.3 | -1.01 | 0.26 | 1.46 | 1.32 |
|  | 3 | -0.71 | 0.13 | 1.02 | 0.92 | -0.6 | 0.23 | 1.08 | 0.98 |
|  | 5 | -0.58 | 0.14 | 0.93 | 0.82 | -0.49 | 0.22 | 0.94 | 0.86 |
| EPA5 | 1 | -1.59 | 0.10 | 1.95 | 1.81 | -1.33 | 0.34 | 1.91 | 1.7 |
|  | 3 | -1.04 | 0.10 | 1.31 | 1.18 | -0.7 | 0.34 | 1.37 | 1.2 |
|  | 5 | -0.89 | 0.12 | 1.12 | 1.03 | -0.56 | 0.36 | 1.17 | 1.06 |
| ETU30 | 1 | -1.53 | 0.23 | 2.05 | 1.88 | N/A | N/A | N/A | N/A |
|  | 3 | -1.16 | 0.26 | 1.78 | 1.52 | N/A | N/A | N/A | N/A |
|  | 5 | -1.01 | 0.26 | 1.62 | 1.42 | N/A | N/A | N/A | N/A |
| ETU70 | 1 | -1.66 | 0.21 | 2.03 | 1.89 | N/A | N/A | N/A | N/A |
|  | 3 | -1.25 | 0.23 | 1.63 | 1.49 | N/A | N/A | N/A | N/A |
|  | 5 | -1.03 | 0.275 | 1.46 | 1.34 | N/A | N/A | N/A | N/A |

Table 3: NR-SSS based RSRP measurement absolute accuracy in FR2, side condition Es/IoT = -6 dB

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS = 120 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy |
| TDL-A delay spread: 30 ns UE speed: 3 km/h | 1 | -1.52 | -0.28 | 0.78 | 1.04 |
|  | 3 | -1.14 | -0.29 | 0.48 | 0.83 |
|  | 5 | -1.02 | -0.28 | 0.36 | 0.79 |
| TDL-B delay spread: 30 ns UE speed: 3 km/h | 1 | -1.58 | -0.3 | 1.06 | 1.1 |
|  | 3 | -1.19 | -0.28 | 0.63 | 0.84 |
|  | 5 | -1.06 | -0.30 | 0.45 | 0.78 |
| TDL-C delay spread: 30 ns UE speed: 3 km/h | 1 | -1.42 | -0.28 | 0.88 | 1.02 |
|  | 3 | -1.12 | -0.29 | 0.5 | 0.81 |
|  | 5 | -1.04 | -0.28 | 0.4 | 0.74 |
| TDL-A delay spread: 300 ns UE speed: 30 km/h | 1 | -2.46 | -0.67 | 0.96 | 1.57 |
|  | 3 | -2.04 | -0.37 | 0.5 | 1.41 |
|  | 5 | -1.8 | -0.68 | 0.365 | 1.32 |
| TDL-B delay spread: 300 ns UE speed: 30 km/h | 1 | -2.67 | -0.80 | 0.73 | 1.77 |
|  | 3 | -2.25 | -0.84 | 0.26 | 1.62 |
|  | 5 | -2.02 | -0.88 | 0.11 | 1.56 |
| TDL-C delay spread: 300 ns UE speed: 30 km/h | 1 | -2.05 | -0.56 | 0.65 | 1.35 |
|  | 3 | -1.52 | -0.55 | 0.42 | 1.15 |
|  | 5 | -1.4 | -0.53 | 0.28 | 1.06 |

Table 4: NR-SSS based RSRP measurement relative accuracy in FR2, side condition Es/IoT = -3 dB

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS = 120 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Relative accuracy |
| TDL-A delay spread: 30 ns UE speed: 3 km/h | 1 | -1.4 | 0.23 | 1.98 | 1.77 |
|  | 3 | -1.07 | 0.19 | 1.62 | 1.36 |
|  | 5 | -0.92 | 0.22 | 1.38 | 1.18 |
| TDL-B delay spread: 30 ns UE speed: 3 km/h | 1 | -1.6 | 0.22 | 2.05 | 1.81 |
|  | 3 | -1 | 0.17 | 1.61 | 1.37 |
|  | 5 | -0.88 | 0.17 | 1.38 | 1.12 |
| TDL-C delay spread: 30 ns UE speed: 3 km/h | 1 | -1.6 | 0.21 | 1.91 | 1.75 |
|  | 3 | -0.91 | 0.19 | 1.34 | 1.2 |
|  | 5 | -0.77 | 0.21 | 1.25 | 1.05 |
| TDL-A delay spread: 300 ns UE speed: 30 km/h | 1 | -1.51 | 0.20 | 1.95 | 1.72 |
|  | 3 | -1.04 | 0.19 | 1.38 | 1.29 |
|  | 5 | -0.93 | 0.17 | 1.23 | 1.13 |
| TDL-B delay spread: 300 ns UE speed: 30 km/h | 1 | -1.38 | 0.29 | 2.24 | 1.9 |
|  | 3 | -0.85 | 0.32 | 1.65 | 1.43 |
|  | 5 | -1.68 | 0.35 | 1.44 | 1.23 |
| TDL-C delay spread: 300 ns UE speed: 30 km/h | 1 | -1.61 | 0.14 | 2.01 | 1.86 |
|  | 3 | -1.15 | 0.16 | 1.49 | 1.34 |
|  | 5 | -0.93 | 0.18 | 1.26 | 1.14 |

# 3	SINR simulation results

In this section, we compare the measurement accuracy of NR-SSS based SINR on different sub-scarrier spacing (SCS), sample number (N) and channel type, with detail setting parameters shown in Section 7. Here, Table 5 and Table 6 summarizes the absolute accuracies and values of 5%, 50%,95% points on the CDF curves of Delta SINR in FR1 and FR2, respectively.

Table 5: NR-SSS based SINR measurement absolute accuracy in FR1, side condition Es/IoT = -6 dB

| Channel | N | Distribution of delta SINR [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 15 kHz |  |  |  | SCS = 30 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy | 5% | 50% | 95% | Absolute accuracy |
| AWGN | 1 | -1.12 | -0.08 | 1 | 1.06 | -1.14 | -0.12 | 0.96 | 1.06 |
|  | 3 | -0.94 | -0.20 | 0.55 | 0.85 | -0.95 | -0.23 | 0.51 | 0.86 |
|  | 5 | -0.88 | -0.23 | 0.45 | 0.8 | -0.89 | -0.36 | 0.36 | 0.81 |
| EPA5 | 1 | -1.72 | -0.25 | 1.12 | 1.44 | -1.65 | -0.35 | 0.87 | 1.45 |
|  | 3 | -1.28 | -0.29 | 0.59 | 1.1 | -1.33 | -0.43 | 0.44 | 1.18 |
|  | 5 | -1.13 | -0.33 | 0.48 | 1 | -1.25 | -0.45 | 0.29 | 1.09 |
| ETU30 | 1 | -1.62 | -0.20 | 1.17 | 1.42 | N/A | N/A | N/A | N/A |
|  | 3 | -1.33 | -0.29 | 0.76 | 1.18 | N/A | N/A | N/A | N/A |
|  | 5 | -1.25 | -0.30 | 0.67 | 1.12 | N/A | N/A | N/A | N/A |
| ETU70 | 1 | -1.59 | -0.20 | 1.11 | 1.4 | N/A | N/A | N/A | N/A |
|  | 3 | -1.38 | -0.26 | 0.66 | 1.16 | N/A | N/A | N/A | N/A |
|  | 5 | -1.25 | -0.28 | 0.55 | 1.08 | N/A | N/A | N/A | N/A |

Table 6: NR-SSS based SINR measurement absolute accuracy in FR2, side condition Es/IoT = -6 dB

| Channel | N | Distribution of delta SINR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS = 120 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy |
| TDL-A delay spread: 30 ns UE speed: 3 km/h | 1 | -1.53 | -0.27 | 0.94 | 1.34 |
|  | 3 | -1.3 | -0.34 | 0.55 | 1.11 |
|  | 5 | -1.17 | -0.34 | 0.44 | 1.02 |
| TDL-B delay spread: 30 ns UE speed: 3 km/h | 1 | -1.6 | -0.28 | 1.08 | 1.4 |
|  | 3 | -1.24 | -0.32 | 0.62 | 1.11 |
|  | 5 | -1.13 | -0.34 | 0.49 | 1.02 |
| TDL-C delay spread: 30 ns UE speed: 3 km/h | 1 | -1.54 | -0.30 | 0.93 | 1.37 |
|  | 3 | -1.19 | -0.33 | 0.48 | 1.06 |
|  | 5 | -1.1 | -0.35 | 0.37 | 0.96 |
| TDL-A delay spread: 300 ns UE speed: 30 km/h | 1 | -2.35 | -0.72 | 0.82 | 2.02 |
|  | 3 | -1.97 | -0.77 | 0.38 | 1.69 |
|  | 5 | -1.73 | -0.80 | 0.2 | 1.59 |
| TDL-B delay spread: 300 ns UE speed: 30 km/h | 1 | -2.52 | -0.80 | 0.72 | 2.2 |
|  | 3 | -2.21 | -0.89 | 0.15 | 1.94 |
|  | 5 | -1.99 | -0.94 | -0.04 | 1.84 |
| TDL-C delay spread: 300 ns UE speed: 30 km/h | 1 | -2.05 | -0.55 | 0.72 | 1.7 |
|  | 3 | -1.67 | -1.62 | 0.46 | 1.48 |
|  | 5 | -1.48 | -0.61 | 0.31 | 1.38 |

# 4	RF impairment in NR wide bandwidth

In this section, the SS-RSRP absolute and relative accuracies of intra-frequency measurement agreed in last meeting [2] are discussed. In LTE, the bandwidth of PSS, SSS and the measurement bandwidth for CRS are aligned with the carrier center. This makes the RF calibration easier. However, in NR the SSB can be located at any possible frequency location within a carrier bandwidth. The more effort for RF calibration is needed, and it is more difficult to achieve the same accuracy as LTE.

It was mentioned in [3] that the calibration errors in FR2 are higher than that in FR1. Therefore, a higher margin is recommended for SS-based RSRP measurements absolute accuracy in FR2. The calibration errors mainly come from the dramatic variation of the cascade ripple response from UE antenna connector to baseband. A mis-match occurs when UE calibrates its RF impairment by utilizing an average gain of a wide BW to approximate the gain in specific sub-band. The wider the bandwidth is used, larger the error is expected. Considering that NR FR1 also has wide bandwidth now, we suggest that the agreed ±[4.5] dB RSRP intra-frequency absolute accuracy when side condition Es/IoT = -6 dB and ±[2] dB RSRP intra-frequency relative accuracy when side condition Es/IoT = -3 dB in [2] should be confined within the operating band groups with NR BW ≤ 20MHz.

Proposal 1: In FR1, the agreed ±[4.5] dB RSRP intra-frequency absolute accuracy when side condition Es/IoT = -6 dB and ±[2] dB RSRP intra-frequency relative accuracy when side condition Es/IoT = -3 dB in [2] should be confined within the operating band groups with NR BW ≤ 20MHz.

Figure 9. Cascade ripple response from UE antenna connector to baseband at n41.

In Figure 9, an example of cascade ripple response from UE antenna connector to baseband at n41 is shown. Based on the numerical result, we evaluate the maximal gain variations of BW 20MHz, BW 40MHz, BW 60MHz, BW 80MHz, and BW 100MHz, accordingly. The results are listed in below table:

Table 5: Maximal gain variations of cascade ripple response at n41.

|  | BW 20MHz | BW 40MHz | BW 60MHz | BW 80MHz | BW 100MHz |
| --- | --- | --- | --- | --- | --- |
| maximal gain variations [dB] | 1.61 | 2.84 | 3.44 | 4.10 | 4.87 |

The gain variation difference between larger BW (40, 68, 80, and 100MHz) and BW 20MHz can also be calculated and it represents the margin should be added when RAN4 evaluates the accuracy of larger BW. The gain variation differences are shown in Table 6.

Table 6: Gain variation differences comparing with BW 20MHz at n41.

|  | BW 40MHz | BW 60MHz | BW 80MHz | BW 100MHz |
| --- | --- | --- | --- | --- |
| gain variation differences comparing with BW 20MHz [dB] | 1.23 | 1.83 | 2.49 | 3.26 |

Therefore, we suggest to extend the accuracy margin based on these gain variation differences. For example, the RSRP intra-frequency absolute accuracy at BW 100MHz should be extended to ±[7.76] = ±[4.5 + 3.26], when side condition Es/IoT = -6 dB.

Proposal 2: In FR1, the RF impairment should be considered when specify the corresponding measurement accuracies. When NR BW > 20MHz, the accuracy margins should be extended based on gain variations differences provided in Table 4.

# 5	Summary

In this contribution, the simulation results for NR measurement accuracy are presented. It is observed that

And we propose

Proposal 1: In FR1, the agreed ±[4.5] dB RSRP intra-frequency absolute accuracy when side condition Es/IoT = -6 dB and ±[2] dB RSRP intra-frequency relative accuracy when side condition Es/IoT = -3 dB in [2] should be confined within the operating band groups with NR BW ≤ 20MHz.

Proposal 2: In FR1, the RF impairment should be considered when specify the corresponding measurement accuracies. When NR BW > 20MHz, the accuracy margins should be extended based on gain variations differences provided in Table 4.

# 6	Reference

[1] R4-1806551, Link-level simulation assumptions for SSB-based measurement accuracy, Ericsson, May 2018.

[2] R4-1808473, Modification on section 10 measurement performance requirements, Huawei, May 2018.

[3] R4-1807080, SS RSRP Measurements Requirements in NR, Qualcomm Incorporated, May 2018.

# 7	Simulation Assumption

Table 1: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 5 MHz; 10 MHz | 50 MHz; 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx (with rx beamforming Note1) | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx (with rx beamforming Note1) |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 5 | 5 |
| SS burst set configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set | 1 | 1 |
| - SS burst periodicity | 5 ms | 5 ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - Actual SS block transmissions | always transmitted | always transmitted |
| NOTE1: For Case B, companies are encouraged to state their assumptions on beamforming and rx antenna configuration together with the provided simulation results |  |  |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| SNR (beamforming gain is included for Case B) | dB | 6 dB 3.3 dB | 1 dB |
| Es/IoT (calculated from SNR) | dB | N/A | -5.97 dB -3.97 dB |
| Propagation conditions | - | * For 4 GHz: AWGN, EPA5, ETU30 (for 15 kHz SCS), ETU70 (for 15 kHz SCS), TDL-A/B/C Note (delay spread: 30 ns and 300 ns; UE speed: 3 km/h and 30 km/h) * For 30 GHz: CDL-C, optional TDL-A/B/C Note (delay spread: 30 ns and 300 ns; UE speed: 3 km/h and 30 km/h) |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |