3GPP TSG RAN WG4 Meeting NR-AH#1807		R4-1808929

Montreal, Canada, 2nd – 6th July, 2018

Source: 	NTT DOCOMO, INC.

Title: 	Remaining issues on gap sharing

Agenda item:	5.2.3.4

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 reached the consensus on introducing measurement gap sharing factor and send the LS to RAN2 to ask introducing the corresponding signalling [1]. In parallel, RAN2 introduced the corresponding IE for measurement gap sharing scheme [1][2]. However the detailed definition of each index of gap sharing scheme are up to RAN4 and still under discussion. In this contribution, we provide our views on the gap sharing scheme definition.

- Discussion

According to TS36.331 and TS38.331, the IE for measurement gap sharing configuration was introduced as follows. It means that the appropriate values of X for calculating the gap sharing factor can be discussed in RAN4, i.e., the way of utilizing this configuration and the gap sharing factor for per-FR gap can be configured separately between FR1 including LTE and FR2.

For TS36.331: MeasGapSharingConfig information element -- ASN1START MeasGapSharingConfig-r14 ::= CHOICE { release NULL, setup SEQUENCE { measGapSharingScheme-r14 ENUMERATED {scheme00, scheme01, scheme10, scheme11} } } -- ASN1STOP MeasGapSharingConfig field descriptions. measGapSharingScheme Indicates the measurement gaps sharing scheme for BL UEs in CE mode A and CE mode B and for EN-DC (for the measurement gap configured by E-UTRAN). For BL UEs, see TS 36.133 [16, Table 8.13.2.1.1.1-2 and Table 8.13.3.1.1.1-3]. For EN-DC, see TS 36.133 [16, Table FFS]. Value scheme00 corresponds to "00", value scheme01 corresponds to "01", and so on.. For TS38.331: MeasGapSharingConfig information element -- ASN1START --TAG-MEAS-GAP-SHARING-CONFIG-START MeasGapSharingConfig ::= SEQUENCE { gapSharingFR2 SetupRelease { MeasGapSharingScheme } OPTIONAL, -- Need M ... } MeasGapSharingScheme ::= ENUMERATED { scheme00, scheme01, scheme10, scheme11 } --TAG-MEAS-GAP-SHARING-CONFIG-STOP -- ASN1STOP MeasGapSharingConfig field descriptions. gapSharingFR2 Indicates the measurement gaps sharing scheme, see TS 38.133 [14]. Value scheme00 corresponds to "00", value scheme01 corresponds to "01", and so on..

According to the RAN4 discussion so far, current RRM specification defines followings.

Measurement gap sharing shall be applies when UE requires measurement gaps to identify and measure intra-frequency cells or when SMTC configured for intra-frequency measurement are fully overlapping with measurement gaps, and when UE is configured to identify and measure cells on inter-frequency carriers and inter-RAT carriers.When network signals “01”, “10” or “11”, where X is a signalled RRC parameter TBD and is defined as in Table 9.1.2-5, - the performance of intra-frequency measurements with no measurement gaps as specified in section 9.2.5, when SMTC configured for intra-frequency measurement are fully overlapping with measurement gaps, shall consider the factor Kintra = 1 / X * 100, - the performance of intra-frequency measurements with measurement gaps as specified in section 9.2.6 shall consider the factor Kintra = 1 / X * 100, - the performance of inter-frequency measurement as specified in section 9.3 and the performance of inter-RAT measurement as specified in section 9.4 shall consider the factor Kinter = 1 / (100 – X) * 100, When network signals “00” indicating equal splitting gap sharing, X is not applied and the performance of intra-frequency measurements as specified in section 9.2.5 and section 9.2.6, the performance of inter-frequency measurement as specified in section 9.3 and the performance of inter-RAT measurement as specified in section 9.4 are FFS. Table 9.1.2-5: Value of parameter X Network signaling ParameterName (to be determined by RAN2); Value of X (%). ‘00’; Equal splitting. ‘01’; [25]. ‘10’; [50]. ‘11’; [75].

In our understanding, indicated gap sharing scheme would determine the gap utilization ratio between intra-frequency measurement and inter-frequency measurement without considering number of carriers to be measured, except for “Equal splitting”. For FR1, although multiple intra-frequency carriers may be measured within a gap, we can assume that up to two intra-frequency carriers can be measured simultaneously within a gap as Kca = 1 for PCell/PSCell. Since the number of intra-frequency FR1 carriers to be measured within a gap would not be so large, scaling factor Kintra would not be too large even when there are multiple inter-frequency carriers to be measured. On the other hand, measurement delay requirements for inter-frequency carriers are scaled by not only Kinter but also Nfreq. There may be a case where scaling for a certain inter-frequency carrier may be too large due to number of inter-frequency carriers to be measured and value of X. For FR2, it is still discussing whether or not multiple cell searchers can be assumed to derive scaling factor in case of FR2 CA. Therefore, although it may not be a typical case, scaling factor for FR2 intra-frequency measurement within a gap may be too large in case of multiple intra-frequency carriers to be measured.

Generally, frequency carriers can be classified into two types; frequency carrier for mobility (i.e., for PCell/PSCell) and frequency carrier for capacity (i.e., for SCell). Therefore, it may be beneficial to consider carrier type for determining scaling factor.

Observation 1:

It may be beneficial if different scaling factor is applied to different carrier types for measurement delay requirements in case of gap sharing.

- Conclusion

In this contribution, we provided our view on the principle of the gap sharing. Based on the discussion, we made following observation.

Observation 1:

It may be beneficial if different scaling factor is applied to different carrier types for measurement delay requirements in case of gap sharing.

- Reference

[1] TS36.331 v15.3.0, June 2018.

[2] TS38.331 v15.2.0, June 2018.