3GPP TSG-RAN WG4 Meeting AH-1807	R4-1808995

## Montreal, Canada, 2 – 6 July, 2018

Source: 	Huawei

Title:	TP to TS 38.141-1: Environmental requirements for the BS equipment (Annex B)

Agenda Item:	5.1.3 (Conducted conformance testing (38.141-1))

Document for:	Approval

# Introduction

In this TP to TS 38.141-1 annex B for the environmental requirements for the BS equipment is provided for BS type 1-C and BS type 1-H.

Cross-references within the TP which require to be verified (e.g. referring to sections which are still empty in the latest TS 38.141-1) were marked highlighted.

# Conclusions

Based on the above introduction, the following proposal is formulated:

Proposal 1:  approve the attached TP to TS 38.141-1 on annex B.

# References

[1] 		Draft TS 38.141-1 v 0.3.0, shared after RAN4#87 on the RAN4 e-mail reflector

# TP to TS 38.141-1

------------------------------ Modified section ------------------------------

- Annex B (normative):
Environmental requirements for the BS equipment

# B.1	General

For each test in the present document, the environmental conditions under which the BS is to be tested are defined.

# B.2	Normal test environment

When a normal test environment is specified for a test, the test should be performed within the minimum and maximum limits of the conditions stated in table B.1.

Table B.1: Limits of conditions for normal test environment

| Condition | Minimum | Maximum |
| --- | --- | --- |
| Barometric pressure | 86 kPa | 106 kPa |
| Temperature | 15 C | 30 C |
| Relative humidity | 20 % | 85 % |
| Power supply | Nominal, as declared by the manufacturer |  |
| Vibration | Negligible |  |

The ranges of barometric pressure, temperature and humidity represent the maximum variation expected in the uncontrolled environment of a test laboratory. If it is not possible to maintain these parameters within the specified limits, the actual values shall be recorded in the test report.

NOTE:	This may, for instance, be the case for measurements of radiated emissions performed on an open field test site.

# B.3	Extreme test environment

The manufacturer shall declare one of the following:

1)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-3 [x];

2)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-4 [x];

3)	The equipment that does not comply with the mentioned classes, the relevant classes from IEC 60 721 [x] documentation for temperature, humidity and vibration shall be declared.

NOTE:	Reduced functionality for conditions that fall outside of the standard operational conditions is not tested in the present document. These may be stated and tested separately.

## B.3.1	Extreme temperature

When an extreme temperature test environment is specified for a test, the test shall be performed at the standard minimum and maximum operating temperatures defined by the manufacturer's declaration for the equipment under test.

Minimum temperature:

The test shall be performed with the environment test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-1 [x].

Maximum temperature:

The test shall be performed with the environmental test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-2 [x].

NOTE:	It is recommended that the equipment is made fully operational prior to the equipment being taken to its lower operating temperature.

# B.4	Vibration

When vibration conditions are specified for a test, the test shall be performed while the equipment is subjected to a vibration sequence as defined by the manufacturer’s declaration for the equipment under test. This shall use the environmental test equipment and methods of inducing the required environmental phenomena in to the equipment, conforming to the test procedure of IEC 60 068-2-6 [x]. Other environmental conditions shall be within the ranges specified in annex B.2.

NOTE:	The higher levels of vibration may induce undue physical stress in to equipment after a prolonged series of tests. The testing body should only vibrate the equipment during the RF measurement process.

# B.5	Power supply

When extreme power supply conditions are specified for a test, the test shall be performed at the standard upper and lower limits of operating voltage defined by manufacturer's declaration for the equipment under test.

Upper voltage limit:

The equipment shall be supplied with a voltage equal to the upper limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [x] Test Ab/Ad and IEC 60 068-2-2 [11] Test Bb/Bd: Dry heat.

Lower voltage limit:

The equipment shall be supplied with a voltage equal to the lower limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [x] Test Ab/Ad and IEC 60 068-2-2 [11] Test Bb/Bd: Dry heat.

# B.6	Measurement of test environments

The measurement accuracy of the BS test environments defined in annex B shall be:

Pressure:	5 kPa

Temperature:	2 degrees

Relative humidity:	5 %

DC voltage:	1.0 %

AC voltage:	1.5 %

Vibration:	10 %

Vibration frequency:	0.1 Hz

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

------------------------------ End of modified section ------------------------------