3GPP TSG-RAN WG4 Meeting AH-1807							   R4-1809010

Montreal, CA, 2 - 6 July, 2018

Title: 	Discussion on Rx beam selection for RRM measurements

Source: 		Huawei, Hisilicon

Agenda item:		5.2.4

Document for:			Discussion

Introduction

At RAN1 #92bis meeting a LS is sent to RAN4 to ask RAN4’s opinion on Rx beam selection for RRM measurements:

RAN1 LS R1-1805760 RAN1 has discussed following proposals regarding Rx beam selection for RRM measurements in the context of requiring the UE to provide more stable measurements. - Measurement to be reported is the best among the measurements based on each RX beam in the selected set - Measurement to be reported shall be greater than average of measurements based on each RX beam in the selected set - The selection of Rx beam set to perform measurement on carrier is left to the UE implementation with the limitation that the same Rx beam set is used to measure the same carrier RAN1 have not concluded on a specific proposal and believes that the selection and/or definition of the Rx beam for measurement should be determined by RAN4. The following are agreements from RAN1 2017 Adhoc #3 that may be relevant for RAN4: Agreements: - For a SS-RSRQ measurement, the same RX beam shall be applied between RSSI measurement and RSRP measurement - For a CSI-RSRQ measurement, the same RX beam shall be applied between RSSI measurement and RSRP measurement - It is up to UE implementation how to select a set of RX beams to perform RRM measurement on a carrier - Different sets of RX beams can be used in measurements based on different measurement objects - Same set of RX beams shall be used in measurement of each TX beam based on a measurement object - FFS - Alt.1: Measurement to be reported shall be greater than average of measurements based on each RX beam in the selected set - Alt.2: Measurement to be reported shall be the best among measurements based on each RX beam in the selected set - Other alternatives are not precluded. RAN1 would like kindly ask RAN4 to consider the above in their further work.

Companies has discussed this topic at RAN4 #87. However, no agreements are reached. So in this contribution we continue to discuss this issue.

Discussion on Selecting Rx beams for RRM measurements.

The main concern in RAN1’s discussion is how to ensure the stability of the measurement results. In our opinion, deriving measurement result based on the average of measurements based on each RX beam in the selected set may be more numerically stable but cannot reflect the real channel quality between NW and UE. In RAN4’s discussion on RRM measurement delay requirements the impact of Rx beam sweeping has been taken into account. To be more specific, PSS/SSS sync time, SSB index acquisition time and measurement period in FR2 shall be scaled by Rx beam scaling factor N1, N2 and N3 respectively.

|  |  | FR2 |
| --- | --- | --- |
| No DRX | TPSS/SSS_sync | max600ms,Kp_intra×5×N1×SMTC period×KCA |
|  | TSSB_time_index | max200ms,Kp_intra×5×N2×SMTC period×KCA |
|  | Tmeasurement_period | max400ms,Kp_intra×5×N3×SMTC period×KCA |
| DRX cycle≤ 320ms | TPSS/SSS_sync | max600ms,Kp_intra×Kmis×5×N1×maxDRX cycle, SMTC period×KCA |
|  | TSSB_time_index | max200ms,Kp_intra×Kmis×5×N2×maxDRX cycle, SMTC period×KCA |
|  | Tmeasurement_period | max400ms,Kp_intra×Kmis×5×N3×maxDRX cycle, SMTC period×KCA |
| DRX cycle> 320ms | TPSS/SSS_sync | Kp_intra×5×N1×DRX cycle×KCA |
|  | TSSB_time_index | Kp_intra×5×N2×DRX cycle×KCA |
|  | Tmeasurement_period | Kp_intra×5×N3×DRX cycle×KCA |

Table 1. Agreed RRM measurement delay requirements for FR2

So one can assume enough time has been reserved for UE to perform measurements with every Rx beam in candidate beam set and find the best beam for reception in set. So the channel quality reflected by measurement results derived by averaging measurement results on each RX beam in set differs from the channel quality when UE using the best Rx beam for reception. So we think that measurement to report should be the best among the measurements based on each RX beam in the selected set

Proposal 1: Measurement to be reported is the best among the measurements based on each RX beam in the selected set

Corresponding draft reply LS is given in [2].

Conclusion

In this contribution Rx beam selection for RRM measurements is discussed and the following proposal is given.

Proposal 1: Measurement to be reported is the best among the measurements based on each RX beam in the selected set

Reference

[1] R1-1805760

[2] R4-1809011 draft reply LS on Rx beam selection for RRM measurements