3GPP TSG-RAN WG4 Meeting-AH-1807                 	R4-1809076

Montreal, Canada, July 2nd – 6th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	UE measurement scaling factor discussion for FR2

Agenda item:	5.2.4.1

Document for:	Discussion

# 1	Introduction

RAN4 has been discussing UE measurement capability for a number of meetings with good and positive progress in the Busan meeting. However, some final agreements are still lacking.

In this paper we discuss, based on system level simulation results from earlier, the UE measurement scaling factors for FR2.

Address the scaling factor N and propose N for intra-f and inter-f: N1, N2 and N3 (intra) as well as N4, N5 and N6 (inter). Use section to and include 2.4.1

2	Discussion

Latest agreements in RAN4 related to UE measurement capability ended with the following outcome at the Busan meeting in May:

Based on the agreements made in Busan meeting the UE shall be able to monitor:

|  | FR1 |  | FR2 Main CC |  | FR2 other CCs than Main CC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cells | SSB Beams | Cells | SSB Beams | Cells | SSB Beams |
| Intra-frequency | 8 | [14] | [6] | [24] | 1 | [1 ~ 4] |
| Inter-frequency | 4 | [7] | 4 | [10] | N/A | N/A |

Additionally, following was agreed concerning cell detection, SSB detection and measurement periods for FR2 for no DRX:

- TPSS/SSS_sync:		max[ 600ms, ceil( [5] x Kp) x N1 x SMTC period ]

- TSSB_time_index:		max[ 200ms, ceil( [5] x Kp) x N2 x SMTC period ]

- T SSB_measurement_period:	max[ 400ms, ceil( 5 x Kp) x N3 x SMTC period ]

In earlier meetings we have provided SLS results targeted at helping the decision related to number of cells per carrier the UE would need to be able to monitor [1, 5 and 6]. Based on these results we propose in [2, 5] following for FR2:

Proposal 5: Monitoring of 2 beam/SSB per cell in FR2 is not sufficient.

Proposal 6: The number of beams the UE need to track per cell in FR2 is higher than 2.

Next, we look at the UE Rx beam forming relaxation factors N1, N2 and N3. We use earlier provided system level simulation results performed in a fully dynamic system simulator, and based on the results we also provide our view on suitable values for the UE Rx beam scaling factors, Nx.

## 2.1	Simulation setup

The simulator is a fully dynamic system simulator in which all the UEs are moving dynamically in a grid of cells with 30 km/h. There are 28 UEs active at a time with keep alive traffic (one UE per sector). SSB burst periodicities of 20ms or 40ms have been used which are also used as the UE sampling rate (L1 sampling interval). I.e. the UE measurement rate is 20ms or 40ms. This emulates SMTC periods of 20ms and 40ms without any measurement restrictions due to performing any other measurements. In the simulations we use an SSB transmission offset to reduce the interference seen by the UE when performing SSB based measurements. More details related to the simulation assumptions can be found in Appendix A.

In the simulations L1 measurement filter of 5 samples is used. For inter-cell mobility (between cells), the modeling of measurement report, handover command and handover complete transmissions are error-free and there is no radio link monitoring active. Measurement reporting to handover command delay (handover preparation delay) is included in the simulation and modeled as a static 50 ms delay. Additionally, the handover command to handover complete delay is modeled using a latency of 20 ms.

For intra-cell inter-beam mobility, i.e. beam change or beam management, the beam detection delay is 400ms and no Index reading latency is assumed. Cell change, or normal inter-cell mobility, is done using handover procedure, including a cell detection delay (600ms), measurement delay (400ms), measurement reportinging to handover command delay (50ms), and handover command to handover complete delay (20ms).

In the simulations an intra-cell beam change is performed without delay (except from the beam detection and measurement delay). An inter-cell beam change is done by handover procedure accounting all the handover delays described.

The simulations are based on the baseline simulation assumptions as agreed in [3] changing the necessary parameters to apply cell and SSB settings according to [4]. The network layout is illustrated in figure 1.

Figure 1 Illustration of the network layout used in the simulation.

We have used an Urban Macro layout with 7x4-sector sites, 28 sectors total and 200ms ISD. In the simulation setup we have used a more realistic deployment setup accounting the necessary DL beam gain to ensure cell coverage. Additionally, the simulation setup is accounting the limitation that there can be no more than 64 SSB based beams per cell.

Accounting these aspects, the sector aperture in the simulations is 90 degrees. The results are based on using 48 SSBs per cell in 3 elevations. Note: the hexagonal grid is only for illustrative purposes and have no real meaning.

## 2.2	Analyzing the impact from Nx

It has been discussed at length and it is clear that there is a need to allow certain relaxations to the UE measurement requirements in FR2 due to UE Rx beam forming. In general, when discussing mobility robustness in FR2, at least the following parameters impact the performance:

- Cell detection delay (including measurement delay and possible Index reading)

- SMTC periodicity

- Nx (relaxation from UE Rx beamforming)

- Measurement gaps

Next, we have investigated the impact from cell detection delay and N. The SMTC period, which is a deployment factor, is used as baseline to investigate the impact of UE Rx scaling factor, N. Impact from the measurement gap sharing is not analyzed.

We have used a fixed cell detection delay of 600ms. To analyze the effect of the UE RX beam forming factor, N, we scale the UE measurement interval assuming that the baseline SMTC (L1 measurement period without scaling) is kept fixed at 20ms or 40ms (20ms SMTC is included to analyze the effect of extreme N factor values). The principle is illustrated in following table:

| SMTC/L1 measurement period [ms] | N3 | MeasInterval [ms] | T SSB_measurement_period [ms] |
| --- | --- | --- | --- |
| 20 | 1 | 20 | 400 |
| 20 | 16 | 320 | 1600 |
| 40 | 1 | 40 | 400 |
| 40 | 2 | 80 | 400 |
| 40 | 8 | 320 | 1600 |
| 80 | 1 | 80 | 400 |
| 80 | 4 | 320 | 1600 |

As an example: the UE is configured with SMTC of 80ms while the UE Rx beam relaxation factor is 4, this leads to an effective measurements period of 320ms per SMTC.

The results will enable analyzing the effect of the scaling factor, Nx. E.g. if SMTC=80ms and N3=1 compared to SMTC=80 and N3=4.

In the simulations we have been tracking 4-6 cells, and a number of beams per carrier equivalent to 1-4 beams per cell – i.e. between 4 and 24 beams per carrier

## 2.3	Simulation results

In the following we look at simulation results from a fully dynamic simulator for each of the listed metrics.

2.3.1	Effect of the measurement interval in serving cell SS-RSRP (effect of UE Rx beam forming)

In order to analyze the effect of the measurement intervals on the UE ability to track cells and ensure that UE is not camped on a cell/beam which is much worse that another cell/beam, we look at the effect of the measurement interval on the SS-RSRP of the serving cell. The SS-RSRP is the SS-RSRP after 5 sample L1 measurement filter (i.e. T SSB_measurement_period).

Figure 2 Effect of the measurement interval on serving Cell SS-RSRP.

From the result it can be observed that the measurement period does have an effect on the SS-RSRP. We see that a longer measurement period leads to a lowered serving SS-RSRP result with an amount of about 2dB.

E.g. if we have SMTC of 20ms while relaxing the UE requirements with an N3 factor of 16, this will have impact on system level in terms of a capacity or coverage drop of 2dB. Additionally, it can also be concluded that if we have two different devices in the field with different N3-factor, these devices will likely not measure same absolute SS-RSRP under same conditions.

Increased measurement period leads to lowered absolute serving cell SS-RSRP results.

Higher UE RX beam forming scaling factor increases the measurement latency and leads to lowered absolute serving cell SS-RSRP results.

In order to enable some flexibility in the UE Rx beamforming as well as meeting some reasonable UE latency requirements for ensuring robust mobility and beam management and network configuration opportunities, we see it problematic with long relaxation factors and based on this we propose a N3=4.

For FR2 intra-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N3 =4.

In principle RAN4 could discuss and decide on different UE Rx beam forming scaling factors, N1 and N3, for the different procedures: cell detection and measurement period. However, as UE movement is rather unpredictable and for simplicity, we propose same scaling factor for both scenario. I.e. N1= N3=4.

For FR2 intra-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N1= N3=4.

As discussed in Busan meeting, the need for Index is conditional and could be assumed to be part of the cell detection. I.e. once the cell is detected and the UE has acquired an initial Index, the UE would not need to read Index from other beams. Based on this we propose N2=1.

For FR2 intra-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N2= 1.

## 2.4	Scaling factors for Inter-frequency requirements

In addition to the UE Rx beam forming scaling for intra-frequency, RAN4 also need to decide on the need for a similar scaling factor for inter-frequency measurement requirements.

As such we do not see any immediate reason why there would be any difference in the scaling factor needed for inter-frequency measurement compared to what is needed by the UE for intra-frequency measurements. Based on this we propose following scaling factors to be defined for inter-frequency measurements:

For FR2 inter-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N4= N6=4.

For FR2 inter-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N5= 1.

3	Conclusion

In this paper we have provided new system level simulation results as input to the discussion related to UE measurement capability in terms UE beam forming scaling factor, Nx, for FR2.

Based on the simulation results and the observations from the results in this paper (and earlier papers) and the system level results provided in earlier meetings, we propose following UE numbers for the UE Rx beam forming scaling factor for FR2:

- For FR2 intra-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N3 =4.

For FR2 intra-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N1= N3=4.

For FR2 intra-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N2= 1.

For FR2 inter-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N4= N6=4.

For FR2 inter-frequency measurements, UE is allowed a UE Rx beam forming scaling factor, N5= 1.

References

- R4-1802798, Dynamic simulation results for NR Mobility in FR2, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1802799, Summary discussion on number of cells and beams in FR2, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1709903, System level simulation assumptions in NR RRM, Ericsson

- R4-1713151, Network Cell and Beam Dimensioning discussion, Nokia, Nokia Shanghai Bell

- R4-1805265, Additional SLS on UE measurement capability, Nokia, Nokia Shanghai Bell

- R4-1807549, UE measurement capability, Nokia, Nokia Shanghai Bel

# A	Simulation assumptions