3GPP TSG-RAN WG4 Meeting AH-1807                 	R4-1809102

Montreal, Canada, July 2nd – 6th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On remaining issues for NR PRACH demodulation

Agenda item:	5.4.2.4

Document for:	Discussion

- Introduction

NR BS demodulation work has been full started in RAN4#87, and the initial agreements for test scope and setup are made regarding the scope of the work in Rel-15 and the simulation assumptions for alignment have been agreed, and some open issues on PRACH demodulation are listed for further discussion [1].

The general open issues in [1] are copied below, and there are also other open issues which were not addressed in RAN4#87.

- PRACH - Test metric - NR PRACH performance metric is the SNR for False alarm probability and Missed detection probability - The requirements on timing estimation error should be further studied - PRACH - FFS: the test for Restricted sets in Rel-15 - Preamble Format - For long sequence, requirements are defined for: FFS for format 1/2/3 - For short sequence, requirements are defined for which PRACH format(s) are FFS

In this paper, we will provide our views on the open issues for NR PRACH demodulation work.

- Discussion

PRACH is the UL random access channel. In 36.104, the PRACH requirements are defined in section 8.4 which can be used as baseline for the work in NR. Compared to LTE there are some new aspects in NR PRACH design and they need to be specifically discussed and taken into account when defining the performance tests for NR.

## 2.1	Test scope and performance metric

In [1], it’s agreed that NR PRACH performance metric is the SNR for False alarm probability and Missed detection probability. From the simulation result, False alarm probability < 0.1% and Missed detection probability <1% can be used as performance metrics for AWGN channel.

The estimation error of the timing is dependent on the channel models and the discussion for channel model is stilling ongoing. We need to further study the suitable condition of timing estimation error occurring after propagation channels for BS are decided in RNA4.

The requirements on timing estimation error should be further studied after propagation channels for BS are decided in RNA4

## 2.2	Test for Restricted sets

For high speed mode, High-frequency offset impact on PRACH causes problems and the restricted sets are introduced in LTE to resolve them. In NR, the same restricted sets, type A and type B, are defined in 38.211. As per section 6.3.3.1 for PRACH preamble formats, we can know restricted sets are supported for formats 0~3 with long sequence. For preamble formats with short sequence, restricted sets are not supported. In our view, restricted sets are only used for the high speed mode and RAN4 should at this stage focus on the essential normal mode. Thus, we propose not to define test for restricted set in Release 15.

RAN4 should at this stage focus on the essential normal mode and the test for restricted sets can be considered in later release

2.3	Format specific parameters

For long sequence, it’s agreed to define the requirements for format 0. For short sequence, there are 9 PRACH preamble formats to consider. In order to avoid too many test cases for large payloads, we think RAN4 should consider these preamble formats for that PRACH performance may be different. Several factors may impact on PRACH performance, e.g. the time of preamble sequence duration, sequence length and SCS. Our proposal for the combinations of preamble formats and SCS is provided in Table 1.

Table 1 Recommended the combinations of preamble formats and SCS

|  | Burst format | SCS(kHz) |
| --- | --- | --- |
| FR1 | 0 | 1.25 |
|  | A2 | 15/30 |
|  | B4 | 15/30 |
|  | C2 | 15/30 |
| FR2 | A2 | 60/120 |
|  | B4 | 60/120 |
|  | C2 | 60/120 |

The parameters of PRACH test preambles that are defined in 36.104 can be reused in NR. Since the number of root sequences required and the cyclic shift length (Ncs) are dependent on the cell size, we design test preambles for different cell radius. An example of the test parameters is provided in Table 2.

Table 2 Recommended test preambles

|  | Burst format | Ncs | Logical sequence index |
| --- | --- | --- | --- |
| FR1 | 0 | 13 | 22 |
|  | A2 | 10 | 0 |
|  | B4 | 15 | 0 |
|  | C2 | 34 | 0 |
| FR2 | A2 | 10 | 0 |
|  | B4 | 15 | 0 |
|  | C2 | 34 | 0 |

- The proposal of the format specific parameters for NR PRACH performance tests

- Conclusion

In this paper, we provided our initial views on the NR PRACH performance requirements, and gave our suggestion on the simulation assumptions.

- The requirements on timing estimation error should be further studied after propagation channels for BS are decided in RNA4

RAN4 should at this stage focus on the essential normal mode and the test for restricted sets can be considered in later release

The proposal of the format specific parameters for NR PRACH performance tests

- References

- R4-1808026, Way forward on NR PRACH demodulation requirements, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, Samsung