3GPP TSG-RAN WG4 Meeting AH-1807                 	R4-1809105

Montreal, Canada, July 2nd – 6th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for NR PRACH

Agenda item:	5.4.2.4

Document for:	Discussion

Introduction

In this contribution we introduce simulation results using the parameters as agreed in [1].

Simulation assumptions

Simulation assumptions as agreed for NR PRACH in [1] are shown below.

Table 1: Simulation assumptions for NR PRACH

| Preamble format | 0 | A2 | B4 | C2 |
| --- | --- | --- | --- | --- |
| Number of Tx | 1 | 1 | 1 | 1 |
| Number of Rx | 2 | 2 | 2 | 2 |
| SCS [KHz] | 1.25 | 15,30,60,120 | 15,30,60,120 | 15,30,60,120 |
| CBW (MHz) | 20, 40, 100 | 15kHz SCS: 10 30kHz SCS: 20, 40, 100 60/120kHz SCS: 100 | 15kHz SCS: 10 30kHz SCS: 20, 40, 100 60/120kHz SCS: 100 | 15kHz SCS: 10 30kHz SCS: 20, 40, 100 60/120kHz SCS: 100 |
| Ncs | 13 | 10 | 15 | 34 |
| Logical sequence index | 22 | 0 | 0 | 0 |
| Carrier Frequency [GHz] | 4 | 4,30 | 4,30 | 4,30 |
| Propagation Channel | AWGN | AWGN | AWGN | AWGN |

Simulation results

Simulation results for NR PRACH using the parameters from Table 1, and False alarm probability less than 0.1% are shown below in Table 2 for preamble format 0, Table 3 for preamble format A2, Table 4 for preamble format B4, and Table 5 for preamble format C2.

Table 2: Initial simulation results for PRACH format 0

| SCS [KHz] | CBW (MHz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- |
| 1.25 | 20 | -16.2 |
| 1.25 | 40 | -16.3 |
| 1.25 | 100 | -16.3 |

Table 3: Initial simulation results for PRACH format A2

| SCS [KHz] | CBW (MHz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- |
| 15 | 10 | -14.7 |
| 30 | 20 | -14.8 |
| 30 | 40 | -15.0 |
| 30 | 100 | -14.8 |
| 60 | 100 | -15.0 |
| 120 | 100 | -14.7 |

Table 4: Initial simulation results for PRACH format B4

| SCS [KHz] | CBW (MHz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- |
| 15 | 10 | -18.6 |
| 30 | 20 | -18.6 |
| 30 | 40 | -18.9 |
| 30 | 100 | -18.7 |
| 60 | 100 | -18.9 |
| 120 | 100 | -18.6 |

Table 5: Initial simulation results for PRACH format C2

| SCS [KHz] | CBW (MHz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- |
| 15 | 10 | -14.5 |
| 30 | 20 | -14.4 |
| 30 | 40 | -14.7 |
| 30 | 100 | -14.5 |
| 60 | 100 | -14.7 |
| 120 | 100 | -14.4 |

Conclusion

In this contribution we have presented simulation results for PRACH with format 0, format A2, format B4 and format C2.

- References

- R4-1808026, Way forward on NR PRACH demodulation requirements, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, Samsung