3GPP TSG-RAN WG4 Meeting AH-1807                 	R4-1809106

Montreal, Canada, July 2nd – 6th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for NR PUCCH

Agenda item:	5.4.2.3

Document for:	Discussion

Introduction

In this contribution we introduce simulation results using the parameters as agreed in [1].

Simulation assumptions

Simulation assumptions as agreed for NR PUCCH in [1] are shown below.

Table 1: Simulation assumptions for NR PUCCH

| PUCCH format | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Number of Tx | 1 | 1 | 1 | 1 |
| Number of Rx | 2 | 2 | 2 | 2 |
| SCS [KHz] | 15 | 15,30,60,120 | 30 | 15,30,60,120 |
| CBW (MHz) | 10 | 15kHz SCS: 10 30kHz SCS: 20, 40, 100 60/120kHz SCS: 100 | 100 | 15kHz SCS: 10 30kHz SCS: 20, 40, 100 60/120kHz SCS: 100 |
| Number of OFDM-symbols | 1,2 | 14 | 1 | 14 |
| UCI bit number | 1 | 2 | 4 | 16 |
| Number of PRB | 1 | 1 | 4 | 1 |
| Carrier Frequency [GHz] | 5.4 | 5.4, 26 | 5.4 | 5.4, 26 |
| Propagation Channel | AWGN | AWGN | AWGN | AWGN |

Simulation results

Simulation results for NR PUCCH using the parameters from Table 1, and the DTX to ACK probability less than 1% are shown below in Table 2 for format 0, Table 3 for format 1, Table 4 for format 2, Table 5 and Table 6 for format 3.

Table 2: Initial simulation results for PUCCH format 0

| SCS [KHz] | CBW (MHz) | Number of OFDM-symbols | SNR [dB] with ACK missed detection probability 1 % |
| --- | --- | --- | --- |
| 15 | 10 | 1 | -3.6 |
| 15 | 10 | 2 | -6.0 |

Table 3: Initial simulation results for PUCCH format 1

| SCS [KHz] | CBW (MHz) | SNR [dB] with ACK missed detection probability 1 % |
| --- | --- | --- |
| 15 | 10 | -11.1 |
| 30 | 20 | -11.1 |
| 30 | 40 | -11.1 |
| 30 | 100 | -11.1 |
| 60 | 100 | -11.1 |
| 120 | 100 | -11.1 |

Table 4: Initial simulation results for PUCCH format 2

| SCS [KHz] | CBW (MHz) | SNR [dB] with ACK missed detection probability 1 % | SNR [dB] with BLER 1 % |
| --- | --- | --- | --- |
| 30 | 100 | -7.9 | -9.4 |

Table 5: Initial simulation results for PUCCH format 3 and no additional DMRS

| SCS [KHz] | CBW (MHz) | SNR [dB] with ACK missed detection probability 1 % | SNR [dB] with BLER 1 % |
| --- | --- | --- | --- |
| 15 | 10 | -2.5 | -7.2 |
| 30 | 20 | -2.5 | -7.2 |
| 30 | 40 | -2.5 | -7.2 |
| 30 | 100 | -2.5 | -7.2 |
| 60 | 100 | -2.5 | -7.2 |
| 120 | 100 | -2.5 | -7.2 |

Table 6: Initial simulation results for PUCCH format 3 with additional DMRS

| SCS [KHz] | CBW (MHz) | SNR [dB] with ACK missed detection probability 1 % | SNR [dB] with BLER 1 % |
| --- | --- | --- | --- |
| 15 | 10 | -4.9 | -8.0 |
| 30 | 20 | -4.9 | -8.0 |
| 30 | 40 | -4.9 | -8.0 |
| 30 | 100 | -4.9 | -8.0 |
| 60 | 100 | -4.9 | -8.0 |
| 120 | 100 | -4.9 | -8.0 |

Conclusion

In this contribution we have presented simulation results for PUCCH with format 0, format 1, format 2 and format 3.

- References

- R4-1808025, Way forward on NR PUCCH demodulation performance in Rel-15, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, Samsung