3GPP TSG-RAN WG4 Meeting AH-1807                 	R4-1809107

Montreal, Canada, July 2nd – 6th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for NR PUSCH

Agenda item:	5.4.2.2

Document for:	Discussion

Introduction

In this contribution we introduce simulation results using the parameters as agreed in [1].

Simulation assumptions

Simulation assumptions as agreed for NR PUSCH in [1] are shown below.

Table 1: Simulation assumptions for NR PUSCH

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Transmission scheme | Codebook based | Codebook based |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| DMRS type | Type-1; Type-2 | Type-1; Type-2 |
| Number of DMRS symbols | 1 front-loaded | 1 front-loaded |
| Time domain resource allocation type | Type A and Type B | Type A and Type B |
| PUSCH symbol length | RA Type A: [14] OFDM symbols RA Type B: [11] OFDM symbols | RA Type A: [14] OFDM symbols RA Type B: [11] OFDM symbols |
| MCS index | QPSK: [2] 16QAM: [13] 64QAM: [25] | QPSK: [2] 16QAM: [13] 64QAM: [25] |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | AWGN | AWGN |
| Fraction of maximum throughput | 70% | 70% |
| Number of LDPC iterations | 15 | 15 |

Simulation results

Simulation results for NR PUSCH using the parameters from Table 1 are shown below in Table 2 for MCS index 2, Table 3 for MCS index 13 and Table 4 MCS index 25.

Table 2: Initial simulation results with MCS index 2.

| MCS index | SCS [KHz] | CBW [MHz] | DMRS type | Time domain resource allocation type | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 15 | 10 | Type-1 | Type A | 70% | -4.1 |
|  |  |  |  | Type B | 70% | -4.0 |
|  |  |  | Type-2 | Type A | 70% | -4.2 |
|  |  |  |  | Type B | 70% | -4.1 |
|  | 30 | 100 | Type-1 | Type A | 70% | -4.5 |
|  |  |  |  | Type B | 70% | -4.5 |
|  |  |  | Type-2 | Type A | 70% | -4.8 |
|  |  |  |  | Type B | 70% | -4.7 |
|  | 60 | 100 | Type-1 | Type A | 70% | -4.4 |
|  |  |  |  | Type B | 70% | -4.4 |
|  |  |  | Type-2 | Type A | 70% | -4.3 |
|  |  |  |  | Type B | 70% | -4.5 |
|  | 120 | 100 | Type-1 | Type A | 70% | -4.1 |
|  |  |  |  | Type B | 70% | -4.2 |
|  |  |  | Type-2 | Type A | 70% | -4.2 |
|  |  |  |  | Type B | 70% | -4.2 |

Table 3: Initial simulation results with MCS index 13.

| MCS index | SCS [KHz] | CBW [MHz] | DMRS type | Time domain resource allocation type | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 15 | 10 | Type-1 | Type A | 70% | 4.5 |
|  |  |  |  | Type B | 70% | 4.5 |
|  |  |  | Type-2 | Type A | 70% | 4.4 |
|  |  |  |  | Type B | 70% | 4.6 |
|  | 30 | 100 | Type-1 | Type A | 70% | 4.2 |
|  |  |  |  | Type B | 70% | 4.2 |
|  |  |  | Type-2 | Type A | 70% | 4 |
|  |  |  |  | Type B | 70% | 4.2 |
|  | 60 | 100 | Type-1 | Type A | 70% | 4.2 |
|  |  |  |  | Type B | 70% | 4.3 |
|  |  |  | Type-2 | Type A | 70% | 4.1 |
|  |  |  |  | Type B | 70% | 4.4 |
|  | 120 | 100 | Type-1 | Type A | 70% | 4.3 |
|  |  |  |  | Type B | 70% | 4.3 |
|  |  |  | Type-2 | Type A | 70% | 4.1 |
|  |  |  |  | Type B | 70% | 4.4 |

Table 4: Initial simulation results with MCS index 25.

| MCS index | SCS [KHz] | CBW [MHz] | DMRS type | Time domain resource allocation type | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 25 | 15 | 10 | Type-1 | Type A | 70% | 14.7 |
|  |  |  |  | Type B | 70% | 14.8 |
|  |  |  | Type-2 | Type A | 70% | 15.5 |
|  |  |  |  | Type B | 70% | 16.1 |
|  | 30 | 100 | Type-1 | Type A | 70% | 14.4 |
|  |  |  |  | Type B | 70% | 14.7 |
|  |  |  | Type-2 | Type A | 70% | 14.2 |
|  |  |  |  | Type B | 70% | 14.7 |
|  | 60 | 100 | Type-1 | Type A | 70% | 14.8 |
|  |  |  |  | Type B | 70% | 14.6 |
|  |  |  | Type-2 | Type A | 70% | 14.6 |
|  |  |  |  | Type B | 70% | 14.7 |
|  | 120 | 100 | Type-1 | Type A | 70% | 14.6 |
|  |  |  |  | Type B | 70% | 14.6 |
|  |  |  | Type-2 | Type A | 70% | 14.2 |
|  |  |  |  | Type B | 70% | 14.5 |

Conclusion

In this contribution we have presented simulation results for PUSCH with MCS index 2, 13 and 25.

- References

- R4-1808024, WF on NR PUSCH demodulation requirements in Rel-15, Nokia, Nokia Shanghai Bell