3GPP TSG-RAN WG4 Meeting AH-1807	R4-1809226

## Montreal, Canada, 2 – 6 July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 37.145-2: OTA AAS BS demod requirements, annexes (C, D) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-06-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In this Cat. B DraftCR, annexes for the OTA AAS BS demodulation requirements are completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of OTA AAS BS demodulation requirements. - Annex C.4: deriviation of test requirements for OTA AAS BS demodulation - Annex D.3: OTA test setups for radiated performance requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Rel-15 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.4, D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This DraftCR is an updated version of the R4-1807735. Updated version was drafted on top of the latest TS 37.145-2 agreed after RAN4#87 in R4-1808502. |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

Annex C (informative):
Test tolerances and derivation of test requirements

# C.1	General

The test requirements explicitly defined in this specification have been calculated by relaxing the minimum requirements of the core specification using the Test Tolerances defined here. When the Test Tolerance is non-zero, the test requirements will differ from the minimum requirements, and the formula used for this relaxation is given in the following tables.

The Test Tolerances are derived from Test System uncertainties.

The Test Tolerances should not be modified for any reason e.g. to take account of commonly known test system errors (such as mismatch, cable loss, etc.).

Note that a formula for applying Test Tolerances is provided for all tests.

NOTE:	OTA test requirements for AAS BS are specified for Normal conditions, only.

# C.2	Measurement of transmitter (OTA)

Table C.2-1: Derivation of Test Requirements (OTA transmitter tests)

| Test | Minimum Requirement in 3GPP TS 37.105 [6] | Test Tolerance (TT) | Test Requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See 3GPP TS 37.105 [6], subclause 9.2 | 1.0 dB, f ≤ 3.0 GHz 1.2 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Upper limit + TT, Lower limit – TT |

# C.3	Measurement of receiver (OTA)

Table C.2-1: Derivation of Test Requirements (OTA receiver tests)

| Test | Minimum Requirement in 3GPP TS 37.105 [6] | Test Tolerance | Test Requirement in the present document |
| --- | --- | --- | --- |
| 7.2 OTA sensitivity | See 3GPP TS 37.105 [6], subclause 10.2 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Declared Minimum EIS + TT |

# C.4	Measurement of performance requirements (OTA)

Radiated test requirements for BS demodulation are the same as the conducted test requirements.

For derivation of conducted UTRA test requirements for BS demodulation requirements, refer to TS 25.141 [10]:

- Subclause 4.1.4 for maximum test system uncertainties for performance requirements,

- Subclause 4.2.3 for test tolerances of performance requirements.

For derivation of conducted E-UTRA test requirements for BS demodulation requirements, refer to TS 36.141 [12]:

- Subclause 4.1.2.3 for maximum test system uncertainties for performance requirements,

- Annex G.3 for derivation of test requirements.

------------------------------ Next modified section ------------------------------

# D.3	Radiated performance requirements

Test requirements for BS demodulation requirements for OTA AAS BS shall be tested in OTA test setups which are based on the conducted test setups, as captured in TS 25.141 [10] and TS 36.141 [12] for UTRA FDD and E-UTRA, with the OTA modifications being introduced to the test setups.

For reference purposes, an examples OTA test setup for the radiated performance requirements for PUSCH in multipath fading conditions is presented in annex D.3.1. In this example OTA test setup, the connected 2Rx test setup has been modified into an OTA equivalent of the conducted 1TX – 2RX setup, based on the polarization separation assumption. Two test antennas with orthogonal polarizations were used in this “cabel replacement” setup.

Other OTA test setups for the radiated performance requirements shall be modified from the conducted into OTA test setups in similar manner.

## D.3.1	Radiated performance requirements for PUSCH in multipath fading conditions

BS under test

Rx

A

Rx

B

F

ar

field region

BS tester

AWGN generator

HARQ feedback (only for PUSCH)

Tx

A, polarization 1

Channel simulator

Channel simulator

Tx

A, polarization 2

AWGN generator

Figure D3-1: Basic OTA test setup for radiated PUSCH performance requirements in multipath fading conditions, and for High Speed Train conditions for OTA AAS BS with Rx diversity (equivalent to conducted 1TX – 2RX setup)

------------------------------ End of modified sections ------------------------------