3GPP TSG-RAN WG4 Meeting AH-1807	R4-1809232

## Montreal, Canada, 2 – 6 July, 2018

Source: 	Huawei

Title:	TP to TS 38.141-2: Wideband operation and related declarations

Agenda Item:	5.1.4.3.6 (Declarations)

Document for:	Discussion

# Introduction

During RAN4#87, DraftCR to TS 38.104 on improvement of declarations for wide NR bands was agreed in [2].

Based on the agreement on improvements of declarations for wide NR bands and the fractional bandwidth definition in TS 38.104, in this contribution we are looking into required improvements to the NR OTA testing specification. TP to TS 38.141-2 is attached for approval.

# Discussion

Referring to the DraftCR in [2], it was agreed to introduce the elements:

- “fractional bandwidth” definition:

fractional bandwidth: fractional bandwidth FBW is defined as

- EIRPFBWmax, EIRPFBWmin, FBWmin, FBWmax symbols

- Related requirement text in 9.2 subclause (Radiated transmit power):

For operating bands where the supported fractional bandwidth (FBW) is larger than 6%, EIRP is declared as EIRPFBWmin at minimum supported frequency FBWmin, and as EIRPFBWmax at maximum supported frequency FBWmax. For frequencies in between the FBWmin and FBWmax frequency the EIRP is: - EIRPFBWmin, for the frequency range FBWmin ≤ f < (FBWmin + FBWmax) / 2, - EIRPFBWmax, for the frequency range (FBWmin + FBWmax) / 2 ≤ f ≤ FBWmax.

The usual arrangement in BS specifications is such that the declarations are captured in the BS test specification. Therefore, below we are discussing on the related implication on the manufacturer declarations for radiated testing in TS 38.141-2.

It shall be noted, that the FBW definition and the related declaration are applicable not only to the BS type 1-O and BS type 2-O, but also to the BS type 1-H. This is due to the fact that FBW definition is directly related to the EIRP requirement (i.e. the EIRP requirement from Rel-13/14 AAS BS specification).

In order to address the above agreements, the following manufacturer declarations are proposed to be formulated in TS 38.141-2:

| Declaration identifier | Declaration | Description | Applicability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H | BS type 1-O | BS type 2-O |
| Dx.y1 | Operating bands subject to fractional bandwidth | List of supported operating bands with the fractional bandwidth (FBW) larger than 6%. NOTE: This list is subset of the NR operating bands in Dx.x. | x | x | x |
| Dx.y2 | Supported frequency range of the operating band | Declaration of the supported part of the operating band. Declared as the frequency range (FBWmin - FBWmax) for all operating bands declared in Dx.y1. | x | x | x |
| Dx.y3 | EIRPFBWmin | Declared for all operating bands in Dx.y1. NOTE: if Dx.y3 is declared for certain operating band (Dx.y1), there shall be no “Rated beam EIRP” value declaration (D9.10) for that operating band. | x | x | x |
| Dx.y4 | EIRPFBWmax | Declared for all operating bands in Dx.y1. NOTE: if Dx.y4 is declared for certain operating band (Dx.y1), there shall be no “Rated beam EIRP” value declaration (D9.10) for that operating band. | x | x | x |

Please note, that manufacturer declarations for radiated testing of the NR BS were submitted in [3], based on the latest TS 38.141-1 [1].

# Conclusions

Based on the above introduction, the following proposal is formulated:

Proposal 1: agree on the attached TP to TS 38.141-2, for the introduction of the FBW related terms and manufacturer declarations.

# References

[1] 		Draft TS 38.141-2 v 0.2.0, shared after RAN4#87 on the RAN4 e-mail reflector

[2]		R4-1808484	Draft CR for TS 38.104: Improvement of declarations for wide NR bands in sub-clause 9.2, RAN4#87

[3] 		R4-180xxxx	TP to TS 38.141-2: manufacturer's declaration for OTA tests NR#1

# TP to TS 38.141-2

------------------------------ Modified section ------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

antenna connector: connector at the conducted interface of the BS type 1-C

aggregated BS channel bandwidth: the RF bandwidth in which a Base Station transmits and receives multiple contiguously aggregated carriers. The aggregated BS channel bandwidth is measured in MHz

Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [5] used for the formulation of unwanted emission requirements for FR1

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain BS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is found

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink

NOTE 1:	The BS channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	It is possible for the BS to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the BS transmission bandwidth configuration, in any part of the BS transmission bandwidth configuration.

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

channel edge: lowest or highest frequency of the NR carrier, separated by the BS channel bandwidth

carrier aggregation: aggregation of two or more component carriers in order to support wider transmission bandwidths

carrier aggregation configuration: a set of one or more operating bands across which the BS aggregates carriers with a specific set of technical requirements

co-location reference antenna: a passive antenna used as reference for base station to base station co-location requirements

contiguous carriers: set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block

contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s)

directional requirement: requirement which is applied in a specific direction within the OTA coverage range for the Tx and when the AoA of the incident wave of a received signal is within the OTA REFSENS RoAoA or the minSENS RoAoA as appropriate for the receiver

Editor’s note: This definition is aligned with the definition of Single directional requirement for AAS BS. The naming should be further aligned between NR and AAS specifications.

fractional bandwidth: fractional bandwidth FBW is defined as

highest carrier: the carrier with the highest carrier frequency transmitted/received in a specified frequency band

inter-band carrier aggregation: carrier aggregation of component carriers in different operating bands

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

intra-band contiguous carrier aggregation: contiguous carriers aggregated in the same operating band

intra-band non-contiguous carrier aggregation: non-contiguous carriers aggregated in the same operating band

[Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands]

lowest Carrier: the carrier with the lowest carrier frequency transmitted/received in a specified frequency band

lower sub-block edge: frequency at the lower edge of one sub-block

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

maximum carrier TRP output power: mean power level measured per RIB during the transmitter ON period for a specific carrier in a specified reference condition and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

maximum total TRP output power: mean power level measured per RIB during the transmitter ON period in a specified reference condition and corresponding to the declared rated total TRP output power (Prated,t,TRP)

minSENS: the lowest declared EIS value for the OSDD’s declared for OTA sensitivity requirement.

minSENS RoAoA: The reference RoAoA associated with the OSDD with the lowest declared EIS

multi-band RIB: operating band specific RIB which is paired with one or more additional operating band specific RIBs where the multiple bands are supported through common active electronic component(s)

multi-carrier transmission configuration: set of one or more contiguous or non-contiguous carriers that a NR BS is able to transmit simultaneously according to the manufacturer’s specification

non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s)

NR BS receiver: composite receiver function of a NR BS receiving in an operating band

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

NOTE:	The operating band(s) for a BS is declared by the manufacturer according to the designations in TS 38.104 [2].

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE:     The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

OTA REFSENS RoAoA: Is the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction

NOTE:	This contour will be related to the average element/sub-array radiation pattern 3dB beam width.

OTA sensitivity directions declaration: set of manufacturer declarations comprising at least one set of declared minimum EIS values (with related RAT and BS channel bandwidth), and related directions over which the EIS applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction

radiated interface boundary: operating band specific reference point for radiated requirements, defined in the near field or in the far field depending on the radiated requirement’s testability feasibility

rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period

rated carrier TRP output power: mean power level declared by the manufacturer per carrier, for BS operating in single carrier, multi-carrier, or carrier aggregation configurations that the manufacturer has declared to be available at the RIB during the transmitter ON period

rated total TRP output power: mean power level declared by the manufacturer, that the manufacturer has declared to be available at the RIB during the transmitter ON period

reference beam direction pair: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the EIRP accuracy compliance directions set

receiver target: AoA in which reception is performed by BS types 1-H, 1-O and BS types 2-O

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to particular OSDD

receiver target reference direction: direction inside the OTA sensitivity directions declaration declared by the manufacturer for conformance testing. For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA

reference RoAoA: the sensitivity RoAoA associated with the receiver target reference direction for each OSDD

requirement set:	one of the NR base station requirement’s set as defined for BS type 1-C, BS type 1-H, BS type 1-O, and BS type 2-O

sensitivity RoAoA: RoAoA within the OTA sensitivity directions declaration, within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific BS direction setting

single-band RIB: operating band specific RIB without any common active electronic component(s) shared with other operating bands

sub-block: one contiguous allocated block of spectrum for transmission and reception by the same base station

NOTE:	There may be multiple instances of sub-blocks within a Base Station RF Bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within a Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

TAB connector: transceiver array boundary connector

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: RF Bandwidth of an instantaneous transmission from a UE or BS, measured in resource block units

total radiated power: the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.  Total radiated power is defined in both the near-field region and the far-field region.

upper sub-block edge: frequency at the upper edge of one sub-block

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BeWθ,REFSENS 	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees

BWChannel	BS channel bandwidth

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12

f 	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax 	f_offsetmax minus half of the bandwidth of the measuring filter

ΔFGlobal 	Global frequency raster granularity

ΔfOBUE 	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔminSENS 	Difference between conducted reference sensitivity and EISminsens

ΔOTAREFSENS 	Difference between conducted reference sensitivity and OTA REFSENS

ΔFRaster 	Channel raster granularity

ΔSUL	Channel raster offset for SUL

EIRPFBWhigh	Declared EIRP value for the upper frequency range of the part of the operating band for which fractional bandwidth support was declared

EIRPFBWlow	Declared EIRP value for the lower frequency range of the part of the operating band for which fractional bandwidth support was declared

EISminsens		The EIS declared for the minSENS RoAoA

EISREFSENS	OTA REFSENS EIS value

EISREFSENS_50M	Declared OTA reference sensitivity basis level for FR2 based on a reference measurement channel with 50MHz BS channel bandwidth

FBWhigh	Upper frequency range of the part of the operating band for which fractional bandwidth support was declared

FBWlow	Lower frequency range of the part of the operating band for which fractional bandwidth support was declared

FC	RF reference frequency on the channel raster

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC_low 	The Fc of the lowest carrier, expressed in MHz

FC_high 	The Fc of the highest carrier, expressed in MHz

Fedge_low 	The lower edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset_low

Fedge_high 	The upper edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset_high

Fedge,block,low 	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset_low

Fedge,block,high 	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset_high

Foffset_high 	Frequency offset from FC_high to the upper Base Station RF Bandwidth edge, or from F C,block, high to the upper sub-block edge

Foffset_low 	Frequency offset from FC_low to the lower Base Station RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

f_offset	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF,SUL		RF reference frequency for Supplementary Uplink (SUL) bands

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

NRB	Transmission bandwidth configuration, expressed in resource blocks

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

Pmax,c,EIRP 	The maximum carrier EIRP when the NR BS is configured at the maximum rated carrier output TRP (PRated,c,TRP)

Pmax,c,TRP	Maximum carrier TRP output power measured at the RIB(s), and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

Prated,c,TRP	Rated carrier TRP output power declared per RIB

Prated,t,TRP	Rated total TRP output power declared per RIB

PREFSENS	Conducted reference Sensitivity power levelSSREF	SS block reference frequency position

TTOTA	Test tolerance for OTA requirements

Wgap	Sub-block gap or Inter RF Bandwidth gap size

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AA	Antenna Array

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

CA	Carrier Aggregation

CACLR	Cumulative ACLR

DUT	Device Under Test

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

FBW	Fractional Bandwidth

FR	Frequency Range

GSCN	Global Synchronization Channel Number

ICS	In-Channel Selectivity

NR	New Radio

OSDD	OTA Sensitivity Directions Declaration

OTA	Over The Air

RDN	Radio Distribution Network

REFSENS	Reference Sensitivity

RIB	Radiated Interface Boundary

RoAoA	Range of Angles of Arrival

SCS	Sub-Carrier Spacing

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TRP	Total Radiated Power

TT	Test Tolerance

------------------------------ End of modified section ------------------------------

## 4.6	Manufacturer’s declarations

The following manufacturer’s declarations are required for radiated requirements for BS type 1-H, BS type 1-O and BS type 2-O.

Table 4.6-1 Manufacturers declarations common for BS type 1-H, BS type 1-O and BS type 2-O

| Declaration identifier | Declaration | Description | Applicability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H | BS type 1-O | BS type 2-O |
| Dx.y1 | Operating bands subject to fractional bandwidth | List of supported operating bands with the fractional bandwidth (FBW) larger than 6%. NOTE: This list is subset of the NR operating bands in Dx.x. | x | x | x |
| Dx.y2 | Supported frequency range of the operating band | Declaration of the supported part of the operating band. Declared as the frequency range (FBWmin - FBWmax) for all operating bands declared in Dx.y1. | x | x | x |
| Dx.y3 | EIRPFBWmin | Declared for all operating bands in Dx.y1. NOTE: if Dx.y3 is declared for certain operating band (Dx.y1), there shall be no “Rated beam EIRP” value declaration (D9.10) for that operating band. | x | x | x |
| Dx.y4 | EIRPFBWmax | Declared for all operating bands in Dx.y1. NOTE: if Dx.y4 is declared for certain operating band (Dx.y1), there shall be no “Rated beam EIRP” value declaration (D9.10) for that operating band. | x | x | x |

------------------------------ End of modified section ------------------------------