3GPP TSG RAN WG4 Meeting #AH-1807	R4-1809241

Montreal, Canada, July 2 – 6, 2018

Agenda Item:	5.1.2

Source:	Huawei

Title:	Efficient parameterization of test models

Document for:	Discussion

# Introduction

In 36.141 [1], there are many tables indicating which RBs in a subframe are boosted for several test models. For example:

- E-TM1.2 (section 6.1.1.2), E-TM3.2 (section 6.1.1.5), E-TM3.3 (section 6.1.1.6): for FDD, there are 60 entries resulting from ten 1-ms subframes  6 bandwidths [1.4, 3, 5, 10, 15, 20 MHz]. For TDD, there are 84 entries (two frames  ten 1-ms subframes  6 bandwidths).

As a result of this complexity in the specification, in RAN#87, WF [3] described

[…]

- For each TM specific list of parameters should be create. There may be different list of parameters for each frequency range.

- Efficient way to introduce list of parameters for TM (e.g. boosted PRBs and allocated PRBs for given test models) should be consider.

[…]

Given the increased number of bandwidths and inclusion of other features, a methodical approach to generating the lists is preferred from a maintainability and specification perspective. The contribution first illustrates the complexity and potential limitations for the existing 36.141 tables. An alternative approach using quadratic permutation polynomials (i.e. the basis for turbo code interleavers [2]) is proposed.

# Discussion

## Background

For E-TM1.2, E-TM3.2, and E-TM3.3 [1], the PRBs are partitioned into two sets: the first Nboost RBs are boosted (attenuated) by 10 log10 pboost dB from the available resource blocks NRB while the remaining NRB-Nboost RBs are attenuated (boosted) by |10 log10 patten| dB where

patten=NRB-pboostNboostNRB-Nboost=NRB/Nboost-pboostNRB/Nboost-1

Typical values for the ratio of NRB/Nboost and pboost are 2.5 and 3 dB, respectively, for E-TM1.2. For E-TM3.2, the typical values for NRB/Nboost and pboost are 5/3 and -3 dB, respectively. For E-TM3.2, the typical values for NRB/Nboost and pboost are 2 and -6 dB, respectively.

One issue is identifying which Nboost RBs to boost. In general, there are NRBNboost possible patterns. Aggregating the patterns listed in 36.141 for all 10 slots for 100 RB FDD E-TM1.2 test, Figure 1 shows an unequal distribution of excited RBs in the test. Figure 2 shows which RBs are used in a particular subframe.

| Figure 1. E-TM1.2 100 RBs all subframes | Figure 2. Subframe 6 |
| --- | --- |

Appendix A shows distributions for E-TM1.2, FDD. Appendix B shows selected distributions (25 and 100 RBs) for E-TM1.2, TDD. Appendix C shows distributions for E-TM3.2, FDD. Appendix D shows selected distributions for E-TM3.2, TDD. Appendix E shows distributions for E-TM3.3, FDD. Appendix F shows selected distributions for E-TM3.3, TDD. Appendix G shows the distributions for 2 subframes of FDD E-TM1.2.

In examining the graphs in the appendices, the distributions tended to be uniform when averaged over all the subframes. Yet on individual subframes, the selected RBs tend to be clustered in small batches (e.g. Appendix G)

Observation: There tends to be uniform distribution of boosted / attenuated RBs in LTE test models.

## Considerations

The general problem is to excite all RBs on average. This problem is analogous of distributing a group of bits across a block (e.g. turbo interleaving). In looking at Table 5.1.3-3 in 36.212 [2], the interleaver is parameterized by coefficients f1 and f2 as well as the block size N. Bit k is placed at location (k) using a 1-to-1 mapping.

k=f1k+f2k2modN

It is proposed to apply the turbo interleaver concept to the RB selection. Suppose there are Nboost RBs (labeled 0 to Nboost-1) and the channel has NRB RBs. The following formula places the Nboost RBs across the NRB RBs according to

k=f1k+f2k2+gsmodNRB,  k=0,⋯,Nboost-1,  s=0,⋯,Nslot-1

The function g(s) is used to position the RBs at different locations for each slot. Conceptually, Nboost logically consecutive RBs are placed in mostly non-consecutive locations in frequency. The function g(s) just moves the locations from slot-to-slot.

For one example with NRB=100 and Nboost=40, f1=61, f2=40, g(s)=10s and Nslot=10. Figure 3 shows the distribution for 10 slots and that, on average, all RBs are equally excited. Figure 4 shows the distribution for slot 0. Appendix H shows more slots.

| Figure 3. 10 slot distribution | Figure 4. slot 0 |
| --- | --- |

## Parameter selection

Table 1 identifies the RBs and the number of parameter sets (f1 and f2) that have a unique mapping for all RBs from 11 to 275. The yellow shading indicates the RB sizes specified in 38.104. For 12 RBs, there are 4 possible sets to consider. For certain bandwidths there are no parameter sets (e.g., 11 RBs). To resolve this issue, some alternatives are presented.

Table 1. Number of candidates to consider for NRB. Yellow shading indicates defined RBs for the allowed bandwidths

| NRB | #Cand | NRB | #Cand | NRB | #Cand | NRB | #Cand | NRB | #Cand |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | None | 65 | None | 119 | None | 173 | None | 227 | None |
| 12 | 4 | 66 | 20 | 120 | 96 | 174 | 56 | 228 | 72 |
| 13 | None | 67 | None | 121 | 1100 | 175 | 480 | 229 | None |
| 14 | 6 | 68 | 32 | 122 | 60 | 176 | 560 | 230 | 88 |
| 15 | None | 69 | None | 123 | None | 177 | None | 231 | None |
| 16 | 56 | 70 | 24 | 124 | 60 | 178 | 88 | 232 | 336 |
| 17 | None | 71 | None | 125 | 2400 | 179 | None | 233 | None |
| 18 | 30 | 72 | 264 | 126 | 180 | 180 | 240 | 234 | 360 |
| 19 | None | 73 | None | 127 | None | 181 | None | 235 | None |
| 20 | 8 | 74 | 36 | 128 | 4032 | 182 | 72 | 236 | 116 |
| 21 | None | 75 | 160 | 129 | None | 183 | None | 237 | None |
| 22 | 10 | 76 | 36 | 130 | 48 | 184 | 264 | 238 | 96 |
| 23 | None | 77 | None | 131 | None | 185 | None | 239 | None |
| 24 | 24 | 78 | 24 | 132 | 40 | 186 | 60 | 240 | 448 |
| 25 | 80 | 79 | None | 133 | None | 187 | None | 241 | None |
| 26 | 12 | 80 | 224 | 134 | 66 | 188 | 92 | 242 | 2310 |
| 27 | 144 | 81 | 1404 | 135 | 576 | 189 | 864 | 243 | 12960 |
| 28 | 12 | 82 | 40 | 136 | 192 | 190 | 72 | 244 | 120 |
| 29 | None | 83 | None | 137 | None | 191 | None | 245 | 1008 |
| 30 | 8 | 84 | 24 | 138 | 44 | 192 | 1984 | 246 | 80 |
| 31 | None | 85 | None | 139 | None | 193 | None | 247 | None |
| 32 | 240 | 86 | 42 | 140 | 48 | 194 | 96 | 248 | 360 |
| 33 | None | 87 | None | 141 | None | 195 | None | 249 | None |
| 34 | 16 | 88 | 120 | 142 | 70 | 196 | 1092 | 250 | 4900 |
| 35 | None | 89 | None | 143 | None | 197 | None | 251 | None |
| 36 | 60 | 90 | 120 | 144 | 1104 | 198 | 300 | 252 | 360 |
| 37 | None | 91 | None | 145 | None | 199 | None | 253 | None |
| 38 | 18 | 92 | 44 | 146 | 72 | 200 | 1520 | 254 | 126 |
| 39 | None | 93 | None | 147 | 504 | 201 | None | 255 | None |
| 40 | 48 | 94 | 46 | 148 | 72 | 202 | 100 | 256 | 16256 |
| 41 | None | 95 | None | 149 | None | 203 | None | 257 | None |
| 42 | 12 | 96 | 480 | 150 | 360 | 204 | 64 | 258 | 84 |
| 43 | None | 97 | None | 151 | None | 205 | None | 259 | None |
| 44 | 20 | 98 | 546 | 152 | 216 | 206 | 102 | 260 | 96 |
| 45 | 48 | 99 | 120 | 153 | 192 | 207 | 264 | 261 | 336 |
| 46 | 22 | 100 | 360 | 154 | 60 | 208 | 672 | 262 | 130 |
| 47 | None | 101 | None | 155 | None | 209 | None | 263 | None |
| 48 | 112 | 102 | 32 | 156 | 48 | 210 | 48 | 264 | 240 |
| 49 | 252 | 103 | None | 157 | None | 211 | None | 265 | None |
| 50 | 180 | 104 | 144 | 158 | 78 | 212 | 104 | 266 | 108 |
| 51 | None | 105 | None | 159 | None | 213 | None | 267 | None |
| 52 | 24 | 106 | 52 | 160 | 960 | 214 | 106 | 268 | 132 |
| 53 | None | 107 | None | 161 | None | 215 | None | 269 | None |
| 54 | 306 | 108 | 612 | 162 | 2862 | 216 | 2520 | 270 | 1224 |
| 55 | None | 109 | None | 163 | None | 217 | None | 271 | None |
| 56 | 72 | 110 | 40 | 164 | 80 | 218 | 108 | 272 | 896 |
| 57 | None | 111 | None | 165 | None | 219 | None | 273 | None |
| 58 | 28 | 112 | 336 | 166 | 82 | 220 | 80 | 274 | 136 |
| 59 | None | 113 | None | 167 | None | 221 | None |  |  |
| 60 | 16 | 114 | 36 | 168 | 144 | 222 | 72 |  |  |
| 61 | None | 115 | None | 169 | 1872 | 223 | None |  |  |
| 62 | 30 | 116 | 56 | 170 | 64 | 224 | 1440 |  |  |
| 63 | 72 | 117 | 144 | 171 | 216 | 225 | 1680 |  |  |
| 64 | 992 | 118 | 58 | 172 | 84 | 226 | 112 |  |  |

Issue 1. How to select (f1 and f2)

- When N is even: i) f1 is odd (relatively prime to N), and ii) all prime factors of N are also factors of f2 [4].

- When N is odd and factorable, it must be divisible by q2 where q is a prime factor of N. This is why 11 has no sets but 121 has 1100 sets.

- Maximize distances [5]

- Compute searches as done above

Issue 2. How to select g(s)

- One approach is to make the function related to the bandwidth and number of slots in the measurement: floor(NRB/Nslots)s.

- Another is to use different (f1 and f2) as a function of slot number.

Issue 3. How to select (f1 and f2) if no sets are available for an NRB value

- Select the next NRB value with available set, such as NRB'=NRB+1 . Generate the mapping but skip over the mapping if that position is not defined

qk=f1k+f2k2+gsmodNRB',

πk=qk,qk≠NRBqk+1otherwise

## Example

Let NRB=11 and Nboost=5. Because there are no factors for 11, using the procedure in the previous section, the parameters for NRB=12 are (1,6), (5,6), (7,6), and (11,6). Selecting the (11,6) set, the interleaver (k) = { 0, 5, 10, 3, 8, 1, 6, 11, 4, 9, 2, 7}. Assuming g(s) = s, then the selected sequence is presented in Table 2. Notice the skip for position 11.

Table 2. Example

| Slot |  | Slot |  | Slot |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0, 5, 10, 3, 8 | 4 | 8, 1, 6, 4, 9 | 8 | 4, 9, 2, 7, 0 |
| 1 | 5, 10, 3, 8, 1 | 5 | 1, 6, 4, 9, 2 | 9 | 9, 2, 7, 0, 5 |
| 2 | 10, 3, 8, 1, 6 | 6 | 6, 4, 9, 2, 7 |  |  |
| 3 | 3, 8, 1, 6, 4 | 7 | 4, 9, 2, 7, 0 |  |  |

Although the above example is trivial, the procedure is extensible.

# Conclusion

In 36.141, the tables for RB boosting:

Observation: There tends to be uniform distribution of boosted / attenuated RBs in LTE test models.

This contribution presented a parameterized approach to identify RBs for testing. The approach is extensible and can account for skips if needed.

Proposal: consider a turbo-like interleaver for RB selection for the generation of the test models.

# References

36.141, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing

36.212, “Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding

R4-1808320, “WF on test models for NR conformance test”, Nokia, Nokia Shanghai Bell, RAN4#87, May 21-25, 2018.

A. Nimbalker, Y. Blankenship, B. Classon, T.K. Blankenship, “ARP and QPP Interleavers for LTE Turbo Coding”, WCNC 2008.

O.Y. Takeshita, “Permutation Polynomial Interleavers: An Algebraic-Geometric Perspective”, IEEE Trans. Inform. Theory, Vol. 53, Issue: 6, June 2007, pp 2116 – 2132.

# Appendix A: E-TM1.2 FDD, distribution of excited RBs

Figure 5-Figure 8 show the distributions for E-TM1.2, FDD for 15, 25, 50, and 75 RBs, respectively. There should be ~40% (4 subframes out of 10) in which a particular RB is excited.

| Figure 5. 15 RBs, LTE | Figure 6. 25 RBs, LTE |
| --- | --- |

| Figure 7. 50 RBs, LTE | Figure 8. 75 RBs, LTE |
| --- | --- |

# Appendix B: E-TM1.2 TDD Frames 1 and 2, distribution of excited RBs

Figure 9-Figure 12 show selected distributions (25 and 100 RBs) for E-TM1.2, TDD on a per frame basis for 25 and 100 RBs, respectively. There are two frames used for TDD (~6 subframes/frame). With 40% boosting, about on average each RB should be excited twice per frame.

| Figure 9. 25 RBs, LTE frame 1 | Figure 10. 25 RBs, LTE frame 2 |
| --- | --- |

| Figure 11. 100 RBs, LTE frame 1 | Figure 12. 100 RBs, LTE frame 2 |
| --- | --- |

# Appendix C: E-TM3.2 FDD Frame, distribution of attenuated RBs

Figure 13-Figure 16 shows the distributions for E-TM3.2, FDD for 15, 25, 50, and 100 RBs, respectively. Approximately each RB should be attenuated 6 times per frame.

| Figure 13. 15 RBs, LTE | Figure 14. 25 RBs, LTE |
| --- | --- |

| Figure 15. 50 RBs, LTE | Figure 16. 100 RBs, LTE |
| --- | --- |

# Appendix D: E-TM3.2 TDD Frames 1 and 2, excited RBs

Figure 17-Figure 20 show selected distributions for E-TM3.2, TDD.

| Figure 17. 25 RBs, LTE frame 1 | Figure 18. 25 RBs, LTE frame 2 |
| --- | --- |

| Figure 19. 100 RBs, LTE frame 1 | Figure 20. 100 RBs, LTE frame 2 |
| --- | --- |

# Appendix E: E-TM3.3FDD Frame, attenuated RBs

Figure 21-Figure 24 show the distributions for E-TM3.3, FDD for 15, 25, 50, and 100 RBs respectively. Approximately each RB should be excited 5 times per frame.

| Figure 21. 15 RBs, LTE | Figure 22. 25 RBs, LTE |
| --- | --- |

| Figure 23. 50 RBs, LTE | Figure 24. 100 RBs, LTE |
| --- | --- |

# Appendix F: E-TM3.3 TDD Frames 1 and 2, excited RBs

Figure 25-Figure 28 show selected distributions for E-TM3.3, TDD for 25 and 100 RBs respectively for both frames.

| Figure 25. 25 RBs, LTE frame 1 | Figure 26. 25 RBs, LTE frame 2 |
| --- | --- |

| Figure 27. 100 RBs, LTE frame 1 | Figure 28. 100 RBs, LTE frame 2 |
| --- | --- |

# Appendix G: E-TM1.2 FDD, selected subframes, 100 RBs

Figure 29 and Figure 30 show the distributions for 2 subframes of FDD E-TM1.2.

| Figure 29. Subframe 1 | Figure 30. Subframe 3 |
| --- | --- |

# Appendix H: Example Distribution of Excited RBs for NR

In Figure 31 and Figure 32, the 40% distribution (from E-TM1.2) is generated using a turbo interleaver for 2 slots.

| Figure 31. 100 RBs, NR, slot 0 | Figure 32. 100 RBs, NR slot 5 |
| --- | --- |