3GPP TSG RAN WG4 Meeting AH1807       		R4- 1809251

Montreal, Canada, July 2-6, 2018

Agenda item:	5.4.1.5

Source: 	AT&T

Title: 	DMRS assumptions for CSI reporting

Document for:	Discussion/Decision

# Introduction

In RAN #71, a new study item New Radio (NR) Access Technology was approved.  In the previous RAN4 meeting (RAN4#86bis), it was decided to define UE demodulation requirements for PDSCH, PDCCH [1] in FR1 and FR2.

In this contribution, we provide our views on CSI reporting requirements for NR

# CSI Reporting Requirements

For NR, the UE needs to report CQI, PMI and RI as in LTE. In addition, it should also report CRI, LI and L1-RSRP for beam management.  Hence we need to define requirements for each of these parameters.  Since each parameter is useful for scheduling PDSCH, we recommend RAN4 should define reporting requirements for CQI, PMI, RI, CRI, LI and L1-RSRP.

Proposal 1: RAN4 should define CSI reporting requirements for CQI, PMI, RI, CRI, LI and L1-RSRP

In addition, we would like to bring up another issue for discussion.  Unlike LTE, NR defines different DMRS configurations for different scenarios.

Table 1 Possible DMRS Configurations in NR

| DMRS Type | Number of front loaded symbols | Maximum number of additional symbols |
| --- | --- | --- |
| Type 1 | Single | 1, 2, and 3 |
|  | Two | 1 |
| Type 2 | Single | 1, 2 and3 |
|  | Two | 1 |

As shown in Table 1, NR defines two types of DMRS configurations where DMRS Type 1 has more RE density per RB than Type 2.  Each Type can be single symbol front loaded or two symbol front loaded. In addition, NR provides an option to configure additional DMRS symbols. The maximum number of additional DMRS symbols depends on the number of OFDM symbols per slot. For 14 OFDM symbols per slot, a maximum of 3 additional DMRS symbols are possible for single symbol front loaded DMRS.  For two symbol front loaded case, a maximum of 1 additional DMRS is possible for both Type 1 and Type 2.   Since some of these configurations can be changed dynamically using DCI (two symbol to one symbol and vice versa), we recommend that the UE should know the proper configuration whien computing CSI.  Since RAN1 specification TS 38.214 didn’t define the DMRS configuration that a UE should assume at the time of CSI computation, we request that RAN4 send an LS to RAN1 enquiring about the DMRS assumption for CSI computation.

Proposal 2: RAN4 should send an LS to RAN1 about the DMRS assumption for computing CSI

# Conclusions

In this contribution we outlined our views on CSI reporting requirements. Based on our observations we recommend

Proposal 1: RAN4 should define CSI reporting requirements for CQI, PMI, RI, CRI, LI and L1-RSRP

Proposal 2: RAN4 should send an LS to RAN1 about the DMRS assumption for computing CSI

# References

[1]. R4-1805549, “Way forward on NR UE performance”, Intel Corporation, Ericsson, Samsung, NTT   DOCOMO, Qualcomm, LGE, Spirent