3GPP TSG-RAN WG4 Meeting #AH-1807	R4-1809271

Montreal, Quebec, Canada, 2nd – 6th July 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On uncertainty contributions in TRP measurements

Agenda item:	4.2.1.1.4.1

Document for:	Discussion

# 1	Introduction

This document discusses uncertainty contributions in TRP measurements for in band TRP requirements in CATR chambers. It identifies a list of uncertainty contributors for computing the total measurement uncertainty.

# 2	Discussion

## 2.1 	Uncertainty budget format

The uncertainty budget format for EIRP measurement for CATR is outlined in TR 37.842, Section 10.3.1.1.3. The uncertainty budget for TRP measurement includes all EIRP uncertainty contributors and additional contributors attributed to TRP measurement as shown in Table 1.

Table 1: CATR uncertainty budget format for TRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | B2-1 |
| 2 | RF power measurement equipment | E |
| 3 | Standing wave between DUT and test range antenna | B2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | B2-4 |
| 5 | QZ ripple DUT | B2-5 |
| 6 | Coarse sampling grid | TBD |
| 7 | DUT offset from phase centre of axis of rotation | TBD |
| 8 | Polarization alignment | TBD |
| 9 | Test system frequency flatness | TBD |
| 19 | Miscellaneous uncertainty | B2-14 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | E |
| 7 | Mismatch of receiver chain | B2-7 |
| 8 | Insertion loss variation of receiver chain | B2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | B2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | B2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | E |
| 12 | Misalignment positioning system | B2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | B2-1 |
| 14 | Rotary Joints | B2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | B2-3 |
| 16 | Quality of quiet zone | B2-5 |
| 20 | Switching uncertainty | B2-15 |

### 2.1.1	Coarse sampling grid

In Table 1, the sampling grid contributor is due to using a finite number of angular samples when measuring 3-D patterns of TRP. This uncertainty is the difference in the TRP as a result when performing the measurement with finite angular steps in the azimuth and elevation directions. In TR 37.843, the uncertainty is referred to as TRP systematic error and is given as

TRPsyserr = | TRPEstimate – TRPreference |   (in dB)

where TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ for OTA far field measurement distance,

and  TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn for spherical equal angle grids.

The TRP systematic error is influenced by beam patterns for a given azimuth and elevation angular subinterval. If the sampling criteria outlined in Section 10.9.2, TR 37.843 are used to determine the azimuth and elevation angular subinterval, then the distribution of TRP systematic error for different beam patterns would follow normal distribution.

### 2.1.2	DUT offset from phase centre of axis of rotation

FFS.

### 2.1.3	Polarization alignment

FFS.

### 2.1.4	Test system frequency flatness

This uncertainty is due to variation of frequency response across the entire channel bandwidth. Any deviation of the rest of the channel form the signal level at the centre frequency will result in an error in the measured result. The error may be measured and corrected for, thereby minimizing the impact on measurement uncertainty. Where error correction is not possible or practical, the test chamber shall account for the total measurement uncertainty due to channel frequency response variation. Further details on the calculation of the error contribution is FFS.

## 2.2	Uncertainty calculation

The uncertainty calculation for EIRP measurements for CATR is outlined in TR 37.842, Section 10.3.1.1.3. The proposed uncertainty calculation for TRP measurements is given in Table 2.

Table 2: CATR uncertainty calculation for TRP measurement

| TRP uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
| 6 | Coarse sampling grid | TBD | TBD | Normal | 1 | 1 | TBD | TBD |
| 7 | DUT offset from phase centre of axis of rotation | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 8 | Polarization alignment | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 9 | Test system frequency flatness | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | TBD | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | TBD | TBD |

# 3	Conclusions

The document has discussed and identified a list of uncertainty contributors to calculating the total measurement uncertainty for TRP measurements in CATR chambers. The list of uncertainty contributors for TRP measurements is provided in Table 1 and their relationship with per point uncertainty contributors. The document also argues that the statistical distribution follows a normal distribution.

# References

- TR 37.843 v15.0.0, RF requirement background for AAS BS radiated requirements (Release 15)