3GPP TSG- RAN WG4 Meeting AH-1807 LTE/NR perf 	R4-1809431

# Montreal, Canada, 2th – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: MU for OTA receiver dynamic range requirement (10.3.4.x) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-06-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The MU for OTA receiver dynamic range requirement have yet to be filled in. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fill in the MU for OTA receiver dynamic range requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MU for OTA receiver dynamic range requirement in TR 37.843 remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 10.3.4 	OTA Dynamic range

#### 10.3.4.1	General

The OTA dynamic range is a measure of the capability of the receiver unit to receive a wanted signal in the presence of an interfering signal inside the received channel bandwidth or the capability of receiving high level of wanted signal.

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

#### 10.3.4.2	In-door anechoic chamber

##### 10.3.4.2.1	General

The measurement system setup is as depicted in figure 10.3.4.2.1-1.

Figure 10.3.4.2.1-1: In-door Anechoic Chamber measurement system setup for dynamic range

##### 10.3.4.2.2	Calibration

Calibration shall be done with the procedure shown in subclause 10.3.2.1.1.2 in TR 37.842 [4] for the wanted and AWGN interfering signals with power combiner/coupler.

LWanted_cal, A→D:  Calibration value for wanted signal between A and D in figure 10.3.4.2.1-1.

LAWGN_cal, A→E:  Calibration value for AWGN signal between A and E in figure 10.3.4.2.1-1.

##### 10.3.4.2.3 	Procedure

The measurement procedure is the same as those for the OTA sensitivity requirement in subclause 10.3.2.1.1.2 in TR 37.842 [4], except that step 11 is modified as follow.

11)	Set the test signal mean power at the RF signal source generator for wanted signal as the required level plus LWanted_cal, A→D and the reference measurement channel. Set the test signal mean power at the RF signal source generator for AWGN interfering signal as the required level plus LAWGN_cal, A→E and at the same frequency as wanted signal.

##### 10.3.4.2.4 	MU assessment

The MU for receiver dynamic range requirement is not impacted by OTA chamber related uncertainties; this is further discussed in subclause 10.3.4.x.

#### 10.3.4.3	CATR

##### 10.3.4.3.1	General

The measurement system setup is as depicted in figure 10.3.4.3.1-1.

Figure 10.3.4.3.1-1: CATR measurement system setup for dynamic range

##### 10.3.4.3.2 	Calibration

Calibration shall be done with the procedure shown in subclause 10.3.2.1.2.2 in TR 37.842 [4] for the wanted and AWGN interfering signals with power combiner/coupler.

LWanted_cal, A→B:  Calibration value for wanted signal between A and B in figure 10.3.4.3.1-1.

LAWGN_cal, A→E:  Calibration value for AWGN interfering signal between A and E in figure 10.3.4.3.1-1.

##### 10.3.4.3.3 	Procedure

- Set up AAS BS in place of SGH from calibration stage. Align AAS BS with boresight of transmitting antenna.

- Configure RF signal generator for wanted signal to set the signal at the required level plus LA→B and at reference measurement channel.

- Configure RF signal generator for AWGN interferer to set the signal at the required level plus LA→E and at the same frequency as wanted signal.

- Measure the throughput of the wanted signal according to annex E of the 3GPP TS 36.141 [??].

- Repeat steps 2-4 for all conformance test bands and directions.

##### 10.3.4.3.4 	MU assessment

The MU for receiver dynamic range requirement is not impacted by OTA chamber related uncertainties; this is further discussed in subclause 10.3.4.x.

#### 10.3.4.x	Summary

The receiver dynamic range requirement provides both a wanted signal and an in-channel AWGN interferer to the BS input. Both the wanted signal and interferer power levels are well above the receiver noise floor, to the extent that the SNR in the receiver is dependent only on the transmitter signal to AWGN level and is independent of the receiver noise level.

Small variations of the receiver power level due to uncertainty factors in the OTA environment will impact both the wanted signal and AWGN equally, and hence do not impact the SNR experienced by the receiver or change the measurement result. Thus, the receiver dynamic range requirement is not impacted by OTA chamber related uncertainties.

In 36.141 [22], the MU for receiver dynamic range is taken as the uncertainty in achieving the correct SNR at the test equipment transmitter. Considering the above argumentation that OTA chamber aspects will not impact MU for receiver dynamic range, the same MU of 0.3dB as for the conducted case can be assumed.

<End of change>