3GPP TSG-RAN WG4 Meeting AH-1807	R4-1809517

## Montreal, Canada, 2 – 6 July, 2018

Source: 	Huawei

Title:	TP to TS 38.141-1: NR BS conducted declarations corrections

Agenda Item:	5.1.4.3.6 (Declarations)

Document for:	Approval

# Introduction

In this TP to TS 38.141-1 further corrections to the conducted testing manufacturer declarations are provided.

The following modifications were introduced:

- Removal of MSR-specific declarations. There shall be no MSR related declaration in the TS 38.141-1/-2. Any MSR related declaration shall appear in 37 series, i.e. 37.141, and/or 37.145-1/-2

- Removal of “per RAT” related aspects from declarations

- Removal of Capability Set related declaration – shall be in the MSR test spec.

- D6.30 (Rated carrier output power (Prated,c,AC, or Prated,c,TABC)) correction: this declaration is not MSR related and shall be kept.

- Multiple text simplifications.

NOTE: The below used declaration IDs are re-used from AAS BS – those ID numbers are used for the referencing purposes and are subject for the optimisation.

This TP has been drafted on top of [1].

# Conclusions

Based on the above introduction, the following proposal is formulated:

Proposal 1:  approve the attached TP to TS 38.141-1 on improvements to the conducted testing manufacturer declarations.

# References

[1] 		Draft TS 38.141-1 v 0.3.0, shared after RAN4#87 on the RAN4 e-mail reflector

# TP to TS 38.141-1

------------------------------ Modified section ------------------------------

## 4.6	Manufacturer declarations

The following BS declarations listed in table 4.6-1 are required to be provided by the manufacturer for the conducted requirements testing of the BS type 1-C and BS type 1-H.

For the BS type 1-H declarations required for the radiated requirements testing, refer to 3GPP TS 38.141-2 [3].

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D6.2 | BS requirements set | Declaration of one of the NR base station requirement’s set as defined for BS type 1-C, or BS type 1-H. | x | x |
| D6.71 | BS class | BS class of the BS, declared as Wide Area BS, Medium Range BS, or Local Area BS. | x | x |
| D6.1 | NR operating bands | NR operating bands supported. Declared per single/multi-band connector (D6.13). | x | x |
| D6.3 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | x | x |
| D6.4 | Geographic area support | The manufacturer shall declare the regions the BS may operate in. e.g. CEPT. | x | x |
| [D6.5] | [Band n20 support, operating in geographical areas allocated to broadcasting (DTT)] | Editor’s note: this declaration is subject to technical discussion on the applicability of the DTT protection from NR BS. [If the BS has single band connector(s) or multi-band connector(s) declared to support Band n20 the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT).] | [x] | [x] |
| [D6.6] | [Band n20 support, emission level for channel N (PEM,N)] | Editor’s note: this declaration is subject to technical discussion on the applicability of the DTT protection from NR BS. [If the BS has sin single band connector(s) or multi-band connector(s) declared to support Band n20 and has been declared to operate in geographical areas allocated to broadcasting (DTT), the emission level for channel N (Annex G of 3GPP TS 36.104 [x]) shall be declared.] | [x] | [x] |
| [D6.7] | [Band n20 support, Maximum output Power in [10] MHz (P10MHz)] | Editor’s note: this declaration is subject to technical discussion on the applicability of the DTT protection from NR BS. [If the BS has single band connector(s) or multi-band connector(s) declared to support Band n20 and has been declared to operate in geographical areas allocated to broadcasting (DTT), the maximum output power in [10] MHz (Annex G of 3GPP TS 36.104 [11]) shall be declared.] | [x] | [x] |
| D6.10 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. | x | x |
| D6.11 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. | x | x |
| D6.13 | Single band connector or multi-band connector | Declaration of the single band or multi-band capability of single band connector(s) or multi-band connector(s), declared for every connector. | x | x |
| D6.14 | Contiguous or non-contiguous spectrum | Ability to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier. Declared per single-/multi-band connector, per operating band. | x | x |
| D6.15 | Contiguous and non-contiguous parameters | If contiguous and non-contiguous operation is declared (D6.14) then such parameters as maximum Base Station RF Bandwidth, rated output power declarations, or maximum number of supported carriers are declared as the same. | x | x |
| D6.16 | Maximum Radio Bandwidth | Maximum radio bandwidth that can be supported by the multi-band connector. May be different for transmit and receive. Declared for each supported operating band and operating bands combination (D6.41) supported for every multi-band connector. | x | x |
| D6.17 | Maximum Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band. Declared per supported operating band, per single-/multi-band connector. | x | x |
| D6.18 | Maximum Base Station RF Bandwidth for contiguous operation | Maximum Base Station RF Bandwidth for contiguous spectrum operation. Declared per supported operating band, per single-/multi-band connector. | x | x |
| D6.19 | Maximum Base Station RF Bandwidth for non- contiguous operation | Maximum Base Station RF Bandwidth for non-contiguous spectrum operation. Declared per supported operating band, per single-/multi-band connector. | x | x |
| D6.20 | NR supported channel bandwidths and SCS | NR channel bandwidth and SCS supported. Declared per supported operating band, per single-/multi-band connector. | x | x |
| D6.21 | single band connector / multi-band connector supported operating bands | List of operating bands and band combinations supported by each single-/multi-band connector. | x | x |
| D6.22 | CA only operation | Declaration of CA-only operation, declared per single-/multi-band connector. | x | x |
| D6.23 | Single or multiple carrier | Capable of operating with a single carrier (only) or multiple carriers. Declared per supported operating band, per single-/multi-band connector. | x | x |
| D6.24 | Maximum number of supported carriers per operating band | Maximum number of supported carriers per supported operation band. Declared per supported operating band, per single-/multi-band connector. | x | x |
| D6.25 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands. Declared for all connectors (D6.21). | x | x |
| D6.28 | Other band combination multi-band restrictions | Declare any other limitations under simultaneous operation in the declared band combinations (D6.41) for each multi-band connector which have any impact on the test configuration generation. Declared for every multi-band connector. | x | x |
| D6.30 | Rated carrier output power (Prated,c,AC, or Prated,c,TABC) | Conducted rated carrier output power, per single band connector or multi-band connector. Declared per supported operating band, per supported RAT, for all single band connector(s) or multi-band connector(s). | x | x |
| D6.31 | Rated carrier output power for contiguous spectrum operation | Conducted rated carrier output power for contiguous spectrum operation. Declared per supported operating band, per single-/multi-band connector. | x | x |
| D6.32 | Rated carrier output power for non-contiguous spectrum operation | Conducted rated carrier output power for non-contiguous spectrum operation. Declared per supported operating band, per single-/multi-band connector. | x | x |
| D6.34 | Rated total output power (Prated,t,AC, or Prated,t,TABC) | Conducted total rated output power. Declared per supported operating band, per single-/multi-band connector. For multi-band connectors declared for each supported operating band in each supported band combination. | x | x |
| D6.35 | Rated total output power for contiguous spectrum operation | Conducted total rated output power for contiguous spectrum operation. Declared per supported operating band, per single-/multi-band connector. | x | x |
| D6.36 | Rated total output power for non-contiguous spectrum operation | Conducted total rated output power for non-contiguous spectrum operation. Declared per supported operating band, per single-/multi-band connector. | x | x |
| [D6.37] | [Rated multi-band total output power, Prated,MB,TABC] | Editor’s note: FFS until the needs for Prated,MB,TABC in the specification is confirmed. [Conducted multi-band rated total output power. Declared per supported operating band combinations, per multi-band connector.] | [x] | [x] |
| D6.38 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band with transmission on all TAB connectors supporting the operating band. |  | x |
| D6.39 | Maximum supported power difference between carriers | Maximum supported power difference between carriers. Declared –per supported operating band, per single-/multi-band connector. | x | x |
| D6.40 | Maximum supported power difference between carriers is different operating bands | Supported power difference between any two carriers in any two different supported operating bands. Declared per supported operating band combination, per every multi-band connector. | x | x |
| D6.41 | Operating band combination support | List of operating bands combinations supported by the BS. | x | x |
| D6.42 | Total number of supported carriers for the declared band combinations of BS | Total number of supported carriers for the declared band combinations (D6.41) of the BS. | x | x |
| D6.43 | Intra-system interfering signal declaration list | List of single band connector(s) or multi-band connector(s) for which an intra-system interfering signal level is required to be declared. Declaration is required if the intra-system interfering signal level is larger than the co-location interfering signal level. | x | x |
| D6.44 | Intra-system interfering signal level | The interfering signal level in dBm. Declared per supported operating band, per single-/multi-band connector covered by D6.43. | x | x |
| D6.54 | [DL RS transmission groups] | [Groups of single band connector(s) or multi-band connector(s) which are declared to transmit DL RS. Declared per supported operating band.] | x | x |
| D6.58 | TAE groups | Set of declared TAB connector beam forming groups on which the TAE requirements apply. All TAB connectors belong to at least one TAB connector beam forming group (even if it's a TAB connector beam forming group consisting of one connector). The smallest possible number of TAB connector beam forming groups need to be declared such that there is no TAB connector not contained in at least one of the declared TAB connector beam forming groups. Declared per supported operating band. |  | x |
| D6.59 | Inter-band CA | Band combinations declared to support inter-band CA (per CA capable multi-band connector(s), as in D6.22). Declared for every multi-band connector which CA support. | x | x |
| D6.60 | Intra-band contiguous CA | Bands declared to support intra-band contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D6.22). Declared per single-/multi-band connector. | x | x |
| D6.61 | Intra-band non-contiguous | Bands declared to support intra-band non-contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D6.22). Declared per single-/multi-band connector. | x | x |
| D6.70 | Equivalent connectors | List of single band connector(s) or multi-band connector(s), which have been declared equivalent. Equivalent connectors imply that the single band connector(s) or multi-band connector(s), are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the single band connector(s) or multi-band connector(s) are identical and the transmitter unit and/or receiver unit driving the single band connector/ multi-band connector are of identical design. | x | x |
| D6.72 | TAB connector RX min cell group | Declared as a group of TAB connectors to which RX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  | x |
| D6.73 | TAB connector TX min cell group | Declared group of TAB connectors to which TX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  | x |

------------------------------ End of modified section ------------------------------