3GPP TSG-RAN WG4 Meeting AH-1807	R4-1809558

## Montreal, Canada, 2 – 6 July, 2018

Source: 	Huawei

Title:	TP to TS 38.141-1: General section for conducted Tx requirements (6.1)

Agenda Item:	5.1.3 (Conducted conformance testing (38.141-1))

Document for:	Approval

# Introduction

In this TP to TS 38.141-1 general section for conducted Tx requirements is provided for BS type 1-C and BS type 1-H.

Cross-references within the TP which require to be verified (e.g. referring to sections which are still empty in the latest TS 38.141-1) were marked highlighted.

# Conclusions

Based on the above introduction, the following proposal is formulated:

Proposal 1:  approve the attached TP to TS 38.141-1 on general section for conducted Tx requirements.

# References

[1] 		Draft TS 38.141-1 v 0.3.0, shared after RAN4#87 on the RAN4 e-mail reflector

# TP to TS 38.141-1

------------------------------ Modified section ------------------------------

# 6	Conducted transmitter characteristics

## 6.1	General

### 6.1.1	BS type 1-C

General test conditions for conducted transmitter tests are given in clause 4, including interpretation of measurement results and configurations for testing. BS configurations for the tests are defined in subclause 4.5.

If a number of single-band connectors, or multi-band connectors have been declared equivalent (see table 4.10-1, D6.70), only a representative one is necessary to demonstrate conformance.

### 6.1.2	BS type 1-H

General test conditions for conducted transmitter tests are given in clause 4, including interpretation of measurement results and configurations for testing. BS configurations for the tests are defined in subclause 4.5.

If a number of single-band connectors, or multi-band connectors have been declared equivalent (see table 4.10-1, D6.70), only a representative one is necessary to demonstrate conformance.

In subclause 6.6, if representative TAB connectors are used then per connector criteria (i.e. option 2 in TS 38.104 [2], subclause 6.6.3.4) shall be applied.

The manufacturer shall declare the minimum number of supported geographical cells (i.e. geographical areas). The minimum number of supported geographical cells (Ncells) relates to the BS setting with the minimum amount of cell splitting supported with transmission on all TAB connectors supporting the operating band. The manufacturer shall also declare TAB connector TX min cell groups. Every TAB connector supporting transmission in an operating band shall map to one TAB connector TX min cell group supporting the same. The mapping of TAB connectors to cells is implementation dependent.

The number of active transmitter units that are considered when calculating the emissions limit (NTXU, counted) for BS type 1-H is calculated as follows:

NTXU, counted = min(NTXU,active, 8·Ncells)

Further:

NTXU,countedpercell = NTXU,counted/Ncells

NTXU,countedpercell is used for scaling the basic limits as described in subclause 6.6.

NOTE:	NTXU,active depends on the actual number of active transmitter units and is independent to the declaration of Ncells.

------------------------------ End of modified section ------------------------------