3GPP TSG-RAN WG4 Meeting #71 RRM AH	R4-71AH-0025

Beijing, P.R. China, 24 - 26 June, 2014

Agenda Item:	3.1

Source: 	Alcatel-Lucent

Title: 	LC-MTC UE RSRP/RSRQ measurement requirements

Document for:	Discussion

# Introduction

LC-MTC UE RSRP/RSRQ measurement requirements were discussed in previous meetings without the conclusion. In RAN4#71, a WF was agreed to have a further investigation of the issue [1]. In this paper, we continue the discussion of the RSRP/RSRQ measurement requirements as well as corresponding test cases.

# RSRP/RSRQ measurement requirements

RSRP/RSRQ measurement performance requirements are defined in the TS 36.133 [2], including

- Intra-frequency RSRP Accuracy Requirements (absolute RSRP accuracy and relative RSRP accuracy)

- Inter-frequency RSRP Accuracy Requirements (absolute RSRP accuracy and relative RSRP accuracy)

- Intra-frequency RSRQ Accuracy Requirements (absolute RSRQ accuracy and relative RSRQ accuracy)

- Inter-frequency RSRQ Accuracy Requirements (absolute RSRQ accuracy and relative RSRQ accuracy)

For LC-MTC UE, the main factor that may impact RSRP/RSRQ measurement accuracy is that the LC-MTC UE only has one receive antenna while the current RSRP/RSRQ measurement performance requirements are defined based on the assumption that the UE has two receive antennas.

Based on the simulation results provided by a number of companies, the performance degradation due to one Rx antenna is about 1-2dB under the assumption that the measurement period of 200ms. The performance degradation could be diminished either with the measurement period is extended to 400ms with the same sampling rate, or let the UE to increase the sampling rate in the same measurement period with potential more power consumptions.

The main purpose of the RSRP/RSRQ measurements is helping the network to make the proper decision on mobility management. Thus, it would be very important for the LC-MTC UE to have the same RSRP/RSRQ accuracy as legacy UE in order to for ht network to make the correct decisions. Otherwise, the network would be forced to deal with the UEs with poor  RSRP/RSRQ reports, which in turn, may adversely, result in significant  degradation on the system performance. To avoid such an impact on the system performance, we like to propose:

Proposal 1: LC-MTC UE should follow the same RSRP/RSRQ accuracy performance requirements as currently defined in TS36.133.

The current common view for the usage of LC-MTC UEs is that they are mainly used for low mobility scenarios. Thus, it might be fine to relax the measurement period from 200ms to 400ms, if necessary, in order for the LC-MTC UE to meet the existing RSRP/RSRQ accuracy performance requirements. The extension of the measurement period also provide the potential for the UE to reduce the power consumption, since if most RF conditions, UE does not need to use whole 400ms for the RSRP/RSRQ measurements in order to meet the required accuracy.

Proposal 2: The measurement period for LC-MTC UE could be extended [FFS] ms with the consideration of L1 filtering effect for keeping the same RSRP/RSRQ accuracy performance requirements as currently defined in TS36.133.

# RSRP/RSRQ measurement test cases

RSRP and RSRQ measurement accuracy test cases are defined in Section A9.1 and A9.2 in TS 36.133. Currently, all these test cases are defined under AWGN channels. The previous simulation results show that under fading channels and with the same sampling period and sampling rate, LC-MTC UEs may have poor RSRP/RSRQ measurement performance than that of the legacy UEs due to the fact that LC-MTC UEs only have one Rx antenna. However, the degradation does not apply under AWGN channel. As a matter of fact, the reported RSRP and RSRQ measurement accuracy with 1Rx antenna should be better than those with two Rx antennas, if other conditions are the same.

Under AWGN channel, the received CRS signal yi in i-th receiving antenna can be modelled as:

yi=s+ni					(i = 1, 2)

Where s  is the CRS signal, and ni is the measurement noise whose magnitude is determined by the SNR at the each of the receiving antennas. The RSRP measurement error for i-th receiving antenna can be expressed as:

dRSRPi=RSRPi- RSRPtrue= [s**s+ni-s**s]=[s**ni]

According to TS 36.214 , “If receiver diversity is in use by the UE, the reported value shall not be lower than the corresponding RSRP of any of the individual diversity branches[ 3]”, it means the reported RSRP will be

RSRPreported= maxRSRP1, RSRP2

The measurement error for the reported RSRP with two antennas will be about

dRSRP=maxRSRP1, RSRP2-RSRPtrue

=maxdRSRP1, dRSRP2

=max⁡{[s**n1],[s**n2]}

As shown above, although the measurement error of the RSRP in each antenna dRSRPi is Gaussian, the measurement error for the reported RSRP does not follow the Gaussian distribution. Its covariance will be larger than the covariance of RSRP measurement error from each individual receiving antenna.

Based on above analysis as well the simulation results [9], under AWGN channel the RSRP and RSRQ measurement accuracy of the LC-MTC should not be degraded due to one Rx antenna. It implies, it would be easier for LC-MTC to pass the existing RSRP and RSRQ test cases than legacy UEs.

Proposal 3: It is worthy to investigate whether the RSRP/RSRQ accuracy test cases need to be modified or tightened up for LC-MTC, due to under AWGN channel, the measured RSRP/RSRQ with one Rx antenna would actually be better than the reported maximum RSRP/RSRQ selected from RSRP/RSRQ measurements obtained from two Rx antennas.

# Summary

In this paper, we provided a further discussion on the impact of dual connectivity on RRM Performance Requirements based on the latest agreements made by RAN2 with following observations:

Proposal 1: LC-MTC UE should follow the same RSRP/RSRQ accuracy performance requirements as currently defined in TS36.133.

Proposal 2: The measurement period for LC-MTC UE could be extended [FFS] ms with the consideration of L1 filtering effect for keeping the same RSRP/RSRQ accuracy performance requirements as currently defined in TS36.133.

Proposal 3: It is worthy to investigate whether the RSRP/RSRQ accuracy test cases need to be modified or tightened up for LC-MTC, due to the fact that under AWGN channel, the measured RSRP/RSRQ with one Rx antenna would actually be better than the reported maximum RSRP/RSRQ selected from RSRP/RSRQ measurements obtained from two Rx antennas.

# References

- R4-143834, Way forward on LC-MTC RSRP/RSRQ measurements and cell identification, Ericsson, Huawei, Hisilicon, Alcatel-Lucent, Samsung, Intel Ericsson

- 3GPP TS36.133, E-UTRA Requirements for support of radio resource management.

- 3GPP TS 36.214, E-UTRA –UTRA Physical layer Measurements.

- R4-143131, RSRP/RSRQ evaluation results for LC-MTC UE, MediaTek Inc.

- R4-143151,	Link level simulation for RSRP/RSRQ measurement for TDD low-cost MTC, Huawei, HiSilicon

- R4-143278,	RSRP and RSRQ simulation results for TDD low-cost MTC, Samsung

- R4-143378,	Simulation results for 1 Rx for RSRP/RSRQ measurements, Ericsson

- R4-143024,	Link level simulation results on cell measurement for low-cost MTC in TDD, Intel Corporation

- R4-71AH-0024, LC-MTC UE RSRP/RSRQ Simulation Results, Alcatel-Lucent