3GPP TSG-RAN WG4 NR AH Meeting	R4-1700006

Spokane, Washington, USA, 17 - 19 January, 2017

Agenda Item:	3.1.2.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Proposed BS ACLR/SEM/ACS values for coexistence study for WP5D on new radio access technology

Document for:	Approval

# 1.	Introduction

The revised study item on New Radio Access Technology was approved at TSG RAN#72 [1]. The objectives of this study item include identifying relevant RF parameters to be used for sharing and co-existence studies. Also a LS was received at TSG RAN#72 [2] from ITU-R WP5D asking for characteristics of terrestrial IMT systems for frequency sharing/interference analysis in 24.25 - 86 GHz. ITU-R WP5D sent another LS on “Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz” in [3]. On the other hand, a LS has been sent from ITU-R WP5D to 3GPP describing detailed modelling and simulation of IMT networks for use in sharing and compatibility studies [4]. This topic has been discussed at previous RAN4 meetings, and the way forwards on simulation assumptions were approved in RAN4#80bis [5-7] and RAN4#81 [8-10].

This contribution proposes the BS ACLR/SEM/ACS values for coexistence study for WP5D on new radio access technology, according to the DL ACIR values proposed in [11].

# 2.	ACLR/ACS values

The ACIR values proposed in [11] to be used to derive the ACLR and ACS values are provided in Table 1 below:

Table 1: Proposed ACIR values

| No. | Simulation frequency | Direction | Deployment Scenario | Proposed ACIR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 30 GHz | DL to DL | Indoor hotspot | 16 |
| 2 | 30 GHz | DL to DL | Urban macro (ISD=200m) | 20 |
| 3 | 30 GHz | DL to DL | Dense urban | 9 |
| 4 | 30 GHz | UL to UL | Indoor hotspot | 15 |
| 5 | 30 GHz | UL to UL | Urban macro (ISD=200m) | 16 |
| 6 | 30 GHz | UL to UL | Dense urban | 9 |
| 7 | 70 GHz | DL to DL | Indoor hotspot | 17 |
| 8 | 70 GHz | DL to DL | Dense urban | TBD |
| 9 | 70 GHz | UL to UL | Indoor hotspot | 15 |
| 10 | 70 GHz | UL to UL | Dense urban | TBD |

When the BS & UE ACLR/ACS were decided in the past in RAN4, the general principle was that the BS ACLR/ACS requirements should be around 10 dB tighter than the UE ACS/ACLR requirements, in order to ease UE implementation for mass market deployment. However, this may not be directly applicable to the new radio access technology, where the probably much wider channel bandwidth (e.g. 200 MHz) together with the potentially much higher frequency range (e.g. mmWave) will largely diminish the implementation margin of the BS compared to the UE, both in turn of the PA linearity and DPD implementation at the transmitter side, as well as the phase noise and ADC granularity at the receiver side. Note that additional RF filtering would not be effective to improve the ACLR/ACS to the adjacent channel which is next to the wanted signal.

In view of the above, it is proposed to evenly share the design and implementation loading between the BS and the UE, i.e. the BS ACLR and the UE ACS should both be equal to the DL ACIR plus 3 dB, while the UE ACLR and the BS ACS should both be equal to the UL ACIR plus 3 dB. The resultant BS ACLR/ACS values are provided in Table 2 below:

Table 2: Proposed BS ACLR/ACS values

| No. | Simulation frequency | Direction | Deployment Scenario | Proposed BS ACLR/ACS (dB) |
| --- | --- | --- | --- | --- |
| 1 | 30 GHz | DL to DL | Indoor hotspot | 19 (ACLR) |
| 2 | 30 GHz | DL to DL | Urban macro (ISD=200m) | 23 (ACLR) |
| 3 | 30 GHz | DL to DL | Dense urban | 12 (ACLR) |
| 4 | 30 GHz | UL to UL | Indoor hotspot | 18 (ACS) |
| 5 | 30 GHz | UL to UL | Urban macro (ISD=200m) | 19 (ACS) |
| 6 | 30 GHz | UL to UL | Dense urban | 12 (ACS) |
| 7 | 70 GHz | DL to DL | Indoor hotspot | 20 (ACLR) |
| 8 | 70 GHz | DL to DL | Dense urban | TBD |
| 9 | 70 GHz | UL to UL | Indoor hotspot | 18 (ACS) |
| 10 | 70 GHz | UL to UL | Dense urban | TBD |

It can be seen in Table 2 that the required BS ACLR/ACS values are quite different in different deployment scenarios, and are more relaxed in small cell (i.e. dense urban and indoor hotspot) cases. Therefore, it is proposed to consider using different BS ACLR/ACS values in different deployment scenarios, based on e.g. BS class or maximum transmit power. On the other hand, if a single BS ACLR/ACS value is preferred for coexistence study for WP5D, then the largest values among the different deployment scenarios should be used, i.e. 23 dB BS ACLR and 19 dB BS ACS. Preliminary investigations indicate that these levels of BS ACLR/ACS are flexible in the mmWave frequency range and the channel bandwidth of 200 MHz.

# 2.	SEM

It has been discussed in previous RAN4 meetings to adopt the emission limits in FCC 16-89 [12] as a baseline for the SEM for coexistence study for WP5D on new radio access technology. The FCC emission limits are extracted below from [12] for ease of reference:

Sec.  30.203  Emission limits.

(a) The conductive power or the total radiated power of any

emission outside a licensee's frequency block shall be -13 dBm/MHz or

lower. However, in the bands immediately outside and adjacent to the

licensee's frequency block, having a bandwidth equal to 10 percent of

the channel bandwidth, the conductive power or the total radiated power

of any emission shall be -5 dBm/MHz or lower.

(b)(1) Compliance with this provision is based on the use of

measurement instrumentation employing a resolution bandwidth of 1

megahertz or greater.

(2) When measuring the emission limits, the nominal carrier

frequency shall be adjusted as close to the licensee's frequency block

edges as the design permits.

(3) The measurements of emission power can be expressed in peak or

average values.

(c) For fixed point-to-point and point-to-multipoint limits see

Using the BS maximum transmit power and channel bandwidth assumed in the coexistence simulation [5], the corresponding ACLR to meet the FCC emission limits ae calculated in Table 3 below:

Table 3: Require BS ACLR values to meet FCC emission limits

| No. | Simulation frequency | Direction | Deployment Scenario | Required BS ACLR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 30 GHz | DL to DL | Indoor hotspot | 23-10log(200)-(-13)=13 |
| 2 | 30 GHz | DL to DL | Urban macro (ISD=200m) | 43-10log(200)-(-13)=33 |
| 3 | 30 GHz | DL to DL | Dense urban | 33-10log(200)-(-13)=23 |
| 7 | 70 GHz | DL to DL | Indoor hotspot | 23-10log(200)-(-13)=13 |
| 8 | 70 GHz | DL to DL | Dense urban | TBD |

It can be seen from Table 3 that for the urban macro and dense urban scenarios, the FCC emission limits already require more stringent BS ACLR values than those are derived from the coexistence simulation results (as shown in Table 2). For the indoor hotspot scenario, the required BS ACLR value to meet FCC emission limits is 7 dB less than that is derived from the coexistence simulation results (as shown in Table 2).

In view of the above, it is proposed to adopt the FCC limits for the SEM for coexistence study for WP5D for BS with maximum transmit power higher than 23 dBm (for urban macro and dense urban scenarios), and use the FCC limits minus 7 dB for the SEM for coexistence study for WP5D for BS with maximum transmit power lower than or equal to 23 dBm (for indoor hotspot scenario). The proposed SEM are provided in Table 4 below:

Table 4: Propose BS SEM

| BS maximum transmit power | Emission limit within immediate 10 percent of the channel bandwidth outside and adjacent to the carrier bandwidth | Emission limit otherwise |
| --- | --- | --- |
| > 23 dBm | -5 dBm/MHz | -13 dBm/MHz |
| <= 23 dBm | -12 dBm/MHz | -20 dBm/MHz |

# 3.	Conclusion

This contribution proposes the BS ACLR/SEM/ACS values for coexistence study for WP5D on new radio access technology, according to the proposed DL ACIR values. It is proposed to adopted the BS ACLR/SEM/ACS values as follows:

Proposal 1: To consider using different BS ACLR/ACS values in different deployment scenarios, or use 23 dB BS ACLR and 19 dB BS ACS if a single BS ACLR/ACS value is preferred for coexistence study for WP5D.

Proposal 2: To adopt the FCC limits for the SEM for coexistence study for WP5D for BS with maximum transmit power higher than 23 dBm, and use the FCC limits minus 7 dB for the SEM for coexistence study for WP5D for BS with maximum transmit power lower than or equal to 23 dBm.

# References

[1]	RP-161214, “Revision of SI: Study on New Radio Access Technology”, NTT DOCOMO, INC.

[2]	RP-160508, “LS on Characteristics of terrestrial IMT systems for frequency sharing/interf. analysis in 24.25 - 86 GHz”, ITU-R WP5D.

[3]	R4-165140, Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz, ITU-R WP5D

[4]	R4-165141, “Modelling and simulation of IMT networks for use in sharing and compatibility studies”, ITU-R WP5D.

[5]	R4-168953, “WF on Simulation assumptions of Co-existence study for WP5D”, NTT DOCOMO, INC.

[6]	R4-168794, “Further Way forward on Dense urban Layout and BS Beamforming Model”, Huawei, HiSilicon.

[7]	R4-167750, “Clarifications about UE antenna modeling for NR coexistence study”, Qualcomm Incorporated.

[8]	R4-1609497, “On throughput model of dense urban and indoor hotspot”, CATT.

[9]	R4-1610153, “Proposal on dense urban network layout for coexistence study for WP5D on new radio access technology”, Nokia, Alcatel-Lucent Shanghai Bell.

[10]	R4-1610634, “WF on urban macro scenario for coexistence study for WP5D on new radio access technology”, Nokia, Alcatel-Lucent Shanghai Bell, Qualcomm, Samsung, CATT, ZTE, NTT DOCOMO, Intel, Ericsson.

[11]	R4-1700005, “Proposed ACIR values for coexistence study for WP5D on new radio access technology”, Nokia, Alcatel-Lucent Shanghai Bell.

[12]	FCC Report and Order and Further Notice of Proposed Rulemaking, “Use of Spectrum Bands Above 24 GHz For Mobile Radio Services”, FCC 16-89, July 14, 2016.