3GPP TSG-RAN WG4 NR AH Meeting	R4- 1700042

November 17th ‒ 19th, 2017

Spokane, Washington, USA

Agenda item:	3.1.1.1

Source: 	Qualcomm Incorporated

Title: 	Simulation results for NR coexistence study: indoor deployment at 30GHz, 45GHz and 70GHz

Document for:	Discussion

# Introduction

This contribution presents a collection of simulation results for the NR adjacent channel coexistence study. The focus of this paper is on the indoor deployment test cases. Three different carrier frequencies are taken into account, namely 30GHz, 45GHz and 70GHz. Both downlink (DL) and uplink (UL) are analysed.

# Discussion

Compared to our previous results (summarized in [1]), this draft contains updated data obtained based on the latest assumptions and considering fine tuning of parameters based on online and offline discussions with other companies.

Table 1 contains a full list of the test cases analyzed in the NR adjacent channel coexistence study. The list is based on latest agreement in RAN4#81. Test cases 2b and 5b are optional, simulation at 45GHz (i.e. scenarios 11 to 14) are also optional. The complete list of parameters will be found in the latest version of TR 36 803 [2].

Table 1. Test cases for the NR adjacent channel coexistence study.

| No. | Aggressor | Victim | Carrier Freq | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Indoor hotspot |
| 2a | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Urban macro - ISD = 200m |
| 2b | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Urban macro - ISD = 300m |
| 3 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Dense urban |
| 4 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Indoor hotspot |
| 5a | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Urban macro - ISD = 200m |
| 5b | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Urban macro - ISD = 300m |
| 6 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Dense urban |
| 7 | NR, 200MHz | NR, 200MHz | 70 GHz | DL to DL | eMBB | Indoor hotspot |
| 8 | NR, 200MHz | NR, 200MHz | 70 GHz | DL to DL | eMBB | Dense urban |
| 9 | NR, 200MHz | NR, 200MHz | 70 GHz | UL to UL | eMBB | Indoor hotspot |
| 10 | NR, 200MHz | NR, 200MHz | 70 GHz | UL to UL | eMBB | Dense urban |
| 11 | NR, 200MHz | NR, 200MHz | 45 GHz | DL to DL | eMBB | Indoor hotspot |
| 12 | NR, 200MHz | NR, 200MHz | 45 GHz | DL to DL | eMBB | Dense urban |
| 13 | NR, 200MHz | NR, 200MHz | 45 GHz | UL to UL | eMBB | Indoor hotspot |
| 14 | NR, 200MHz | NR, 200MHz | 45 GHz | UL to UL | eMBB | Dense urban |

In this contribution we only focus on indoor scenarios, i.e. test cases 1, 4, 7, 9, 11 and 13.

The following sections are organized per carrier frequency, and both DL and UL data are considered. The plots in this contribution represent the throughput degradation due to adjacent channel interference as a function of ACIR, i.e. aggressor transmitter and victim receiver imperfections are taken into account jointly. For each carrier frequency two noise figures (NF) are analyzed:

- NF = 9dB and 11dB for 30GHz simulations

- NF = 11dB and 13dB for 45GHz simulations

- NF = 13dB and 15dB for 70GHz simulations

An overall summary of the simulation results for the indoor scenario will be presented in section 2.4.

## Carrier frequency = 30GHz

### Downlink

Figure 1. DL throughput degradation as a function of ACIR at 30GHz.

### Uplink

Figure 2. UL throughput degradation as a function of ACIR at 30GHz

## Carrier frequency = 45GHz

### Downlink

Figure 3. DL throughput degradation as a function of ACIR at 45GHz.

### Uplink

Figure 4. UL throughput degradation as a function of ACIR at 45GHz.

## Carrier frequency = 70GHz

### Downlink

Figure 5. DL throughput degradation as a function of ACIR at 70GHz.

### Uplink

Figure 6. UL throughput degradation as a function of ACIR at 70GHz.

## Summary of simulation results for indoor deployments

Table 2 shows as summary of the simulation results presented in this paper. The results in the last two columns are expressed in terms of ACIR needed (in dB) to keep the mean and 5%-tile degradation due to ACI within 5%.

Table 2. Summary of simulation results for indoor scenarios.

| Simulation results for indoor scenarios |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Case No. | Carrier Freq | Channel BW [MHz] | Direction | NF [dB] | Mean | 5%-tile |
| 1 | 30GHz | 200MHz | DL to DL | 9 | 16.18 | 13.48 |
|  |  |  |  | 11 | 16.18 | 13.48 |
| 4 | 30GHz | 200MHz | UL to UL | 9 | 9.93 | 13.18 |
|  |  |  |  | 11 | 8.96 | 12.98 |
| 11 | 45GHz | 200MHz | DL to DL | 11 | 16.49 | 13.91 |
|  |  |  |  | 13 | 16.5 | 13.91 |
| 13 | 45GHz | 200MHz | UL to UL | 11 | 9.09 | 13.07 |
|  |  |  |  | 13 | 8.01 | 12.78 |
| 7 | 70GHz | 200MHz | DL to DL | 13 | 16.39 | 13.89 |
|  |  |  |  | 15 | 16.35 | 13.91 |
| 9 | 70GHz | 200MHz | UL to UL | 13 | 8.01 | 12.77 |
|  |  |  |  | 15 | 6.78 | 12.37 |

The following observations can be made:

- The worst case DL ACIR, considering different noise figures and both mean and 5%-tile criteria, is 16.5dB

- The worst case UL ACIR, considering different noise figures and both mean and 5%-tile criteria, is 13.79dB

Results for urban macro and dense urban scenarios are reported in [3] and [4], respectively. An overall summary of the coexistence study results and specific proposals for ACLR and ACS requirements are presented in [5].

# Conclusions

In this contribution we presented simulation results for the NR adjacent channel coexistence study. We focused on indoor scenarios, analyzing both DL and UL cases for three different carrier frequencies, namely 30GHz, 45GHz and 70GHz.

# Reference

- R4-1610166, “Simulation of results for NR coexistence study: summary and ACIR proposals”, Qualcomm Incorporated.

- 3GPP TR 38.803, “Study on New Radio Access Technology; RF and co-existence aspects (Release 14)”.

- R4-1700043, “Simulation results for NR coexistence study: urban macro deployment at 30GHz”, Qualcomm Incorporated.

- R4-1700044, “Simulation results for NR coexistence study: dense urban deployment at 30GHz, 45GHz and 70GHz”, Qualcomm Incorporated.

- R4-1700045, “ACIR, ACLR and ACS proposals”, Qualcomm Incorporated.