3GPP TSG-RAN WG4 NR AH Meeting	R4- 1700043

November 17th ‒ 19th, 2017

Spokane, Washington, USA

Agenda item:	3.1.1.1

Source: 	Qualcomm Incorporated

Title: 	Simulation results for NR coexistence study: urban macro deployment at 30GHz

Document for:	Discussion

# Introduction

This contribution presents a collection of simulation results for the NR adjacent channel coexistence study. The focus of this paper is on the urban macro deployment at 30GHz. Two different inter-site distances are taken into account, namely 200m and 300m. Both downlink (DL) and uplink (UL) are analysed.

# Discussion

Compared to our previous results (summarized in [1]), this draft contains updated data obtained based on the latest assumptions and considering fine tuning of parameters based on online and offline discussions with other companies.

Table 1 contains a full list of the test cases analyzed in the NR adjacent channel coexistence study. The list is based on latest agreement in RAN4#81. Test cases 2b and 5b are optional, simulation at 45GHz (i.e. scenarios 11 to 14) are also optional. The complete list of parameters will be found in latest version of TR 36 803 [2].

Table 1. Test cases for the NR adjacent channel coexistence study.

| No. | Aggressor | Victim | Carrier Freq | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Indoor hotspot |
| 2a | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Urban macro - ISD = 200m |
| 2b | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Urban macro - ISD = 300m |
| 3 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Dense urban |
| 4 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Indoor hotspot |
| 5a | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Urban macro - ISD = 200m |
| 5b | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Urban macro - ISD = 300m |
| 6 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Dense urban |
| 7 | NR, 200MHz | NR, 200MHz | 70 GHz | DL to DL | eMBB | Indoor hotspot |
| 8 | NR, 200MHz | NR, 200MHz | 70 GHz | DL to DL | eMBB | Dense urban |
| 9 | NR, 200MHz | NR, 200MHz | 70 GHz | UL to UL | eMBB | Indoor hotspot |
| 10 | NR, 200MHz | NR, 200MHz | 70 GHz | UL to UL | eMBB | Dense urban |
| 11 | NR, 200MHz | NR, 200MHz | 45 GHz | DL to DL | eMBB | Indoor hotspot |
| 12 | NR, 200MHz | NR, 200MHz | 45 GHz | DL to DL | eMBB | Dense urban |
| 13 | NR, 200MHz | NR, 200MHz | 45 GHz | UL to UL | eMBB | Indoor hotspot |
| 14 | NR, 200MHz | NR, 200MHz | 45 GHz | UL to UL | eMBB | Dense urban |

In this contribution we only focus on urban macro scenarios, i.e. test cases 2a, 2b, 5a and 5b.

As it can be observed for urban macro deployment only 30GHz carrier frequency is considered. Also, two scenarios were identified based on inter-site distance (ISD), i.e. ISD=200m and ISD=300m.

The following two sections report results for the two ISD cases, and both DL and UL data are considered. The plots in this contribution represent the throughput degradation due to adjacent channel interference as a function of ACIR, i.e. aggressor transmitter and victim receiver imperfections are taken into account jointly. Two noise figures (NF) are analyzed, namely NF = 9dB and 11dB.

An overall summary of the simulation results for the urban macro scenario will be presented in section 2.3.

## ISD=200m

### Downlink

Figure 1. DL throughput degradation as a function of ACIR at 30GHz. ISD=200m.

### Uplink

Figure 2. UL throughput degradation as a function of ACIR at 30GHz. ISD=200m.

## ISD=300m

### Downlink

Figure 3. DL throughput degradation as a function of ACIR at 30GHz. ISD=300m.

### Uplink

Figure 4. UL throughput degradation as a function of ACIR at 30GHz. ISD=300m.

## Summary of simulation results for urban macro deployments

Table 2 shows as summary of the simulation results presented in this paper. The results in the last two columns are expressed in terms of ACIR needed (in dB) to keep the mean and 5%-tile degradation due to ACI within 5%.

Table 2. Summary of simulation results for urban macro scenarios.

| Simulation results for urban macro scenarios |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Case No. | Carrier Freq | Channel BW [MHz] | Direction | NF [dB] | Mean | 5%-tile |
| 2a | 30GHz | 200MHz | DL to DL | 9 | 12.33 | 13.49 |
|  |  |  |  | 11 | 12.31 | 13.16 |
| 2b | 30GHz | 200MHz | DL to DL | 9 | 12.47 | 9.55 |
|  |  |  |  | 11 | 12.44 | 9.15 |
| 5a | 30GHz | 200MHz | UL to UL | 9 | 5.31 | 12.22 |
|  |  |  |  | 11 | 4.19 | 9.79 |
| 5b | 30GHz | 200MHz | UL to UL | 9 | 4.88 | 13.79 |
|  |  |  |  | 11 | 3.82 | NA |

The following observations can be made:

- The worst case DL ACIR, considering different noise figures and both mean and 5%-tile criteria, is 13.49dB

- The worst case UL ACIR, considering different noise figures and both mean and 5%-tile criteria, is 13.79dB

Results for indoor and dense urban scenarios are reported in [3] and [4], respectively. An overall summary of the coexistence study results and specific proposals for ACLR and ACS requirements are presented in [5].

# Conclusions

In this contribution we presented simulation results for the NR adjacent channel coexistence study. We focused on urban macro scenarios deployed at 30GHz. We analyzed both DL and UL cases considering two ISD cases, i.e. ISD=200m and ISD=300m.

# Reference

- R4-1610166, “Simulation of results for NR coexistence study: summary and ACIR proposals”, Qualcomm Incorporated.

- 3GPP TR 38.803, “Study on New Radio Access Technology; RF and co-existence aspects (Release 14)”.

- R4-1700042, “Simulation results for NR coexistence study: indoor deployment at 30GHz, 45GHz and 70GHz”, Qualcomm Incorporated.

- R4-1700044, “Simulation results for NR coexistence study: dense urban deployment at 30GHz, 45GHz and 70GHz”, Qualcomm Incorporated.

- R4-1700045, “ACIR, ACLR and ACS proposals”, Qualcomm Incorporated.