3GPP TSG-RAN WG4 NR Ad-hoc	R4- 1700166

Spokane, Washington, USA, 17 – 19 January 2017

Agenda Item:	3.1.1.1

Source: 	Samsung

Title: 	Simulation results for NR coexistence study in urban macro scenario

Document for:	Discussion

- 1. Introduction

The ACIR is an important parameter for the design of RF circuits, and it imposes a constraint on adjacent channel interference so that throughput loss can be within a reasonable level. It was already observed in previous meetings that the required ACIRs in different scenarios can be quite different. Thus it is necessary to provide the ACIR simulation results for each specific scenario, i.e., urban macro, dense urban and indoor scenario. In RAN4#81, it was agreed to use ISD 200 m as the baseline case and ISD 300 m as the optional case for urban macro scenario.

This contribution presents the ACIR evaluation results in urban macro scenario at 30 GHz carrier frequency for both downlink and uplink. A brief analysis is also provided.

- 2. Discussion

- 2.1 Coexistence simulation case

The NR is assumed under synchronized network, where the aggressor and the victim have the same configuration, ISD=200 m, indoor UE percentage=0.2, BW=200 MHz.

Table 2.1-1 Simulation cases for urban macro scenario

| No. | Aggressor | Victim | Simulation frequency | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Urban macro |
| 5 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Urban macro |

- 2.2 Simulation parameters

Network layout, propagation model, etc. are the same with [2]. BS beamforming, UE beamforming are in agreement with [3] [4], respectively. Beamforming are employed at both the BS side and the UE side. Only the antenna element gain of BSs and UEs are considered in the cell selection process.

- 3. Simulation results

- 3.1 Downlink

This section presents the downlink simulation results, NF 9 dB and 11 dB. Table 3.1-1 and Figure 3.1-1 are the throughput loss versus ACIR results.

Table 3.1-1: Throughput loss at given ACIRs for downlink

| ACIR |  | 5 dB | 10 dB | 15 dB | 20 dB | 25 dB | 30 dB | 35 dB | 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 9 dB | mean | 7.90 | 4.17 | 2.11 | 0.99 | 0.33 | 0.05 | 0.01 | 0 |
|  | 5%-tile | 33.65 | 15.30 | 1.07 | 0.36 | 0.12 | 0.02 | 0.01 | 0 |
| NF 11 dB | mean | 7.88 | 4.16 | 2.11 | 0.99 | 0.33 | 0.05 | 0.01 | 0 |
|  | 5%-tile | 31.03 | 11.68 | 0.61 | 0.10 | 0..02 | 0 | 0 | 0 |
| NF 9 dB, different LSF | mean | 6.65 | 3.98 | 2.26 | 1.21 | 0.60 | 0.28 | 0.12 | 0.04 |
|  | 5%-tile | 32.38 | 20.23 | 10.58 | 5.43 | 1.86 | 0.41 | 0.22 | 0.14 |

Figure 3.1-1: Throughput loss versus ACIR for downlink.

From Figure 3.1-1, it is observed that the impact of NF on ACIR is trivial. The increase of NF from 9 dB to 11 dB almost has no influence on the ACIR when mean throughput is concerned, and only reduces the required ACIR from 13.6 dB to 13 dB when 5%-tile UE throughput is concerned.

Based on the assumption that victim BSs and aggressor BSs are collocated, large scale fadings (LSFs) between victim BSs and a UE and those between aggressor BSs and the same UE should be the same for the downlink. If they are generated independently, the ACIR results, in particular, the 5%-tile UE throughput is quite different, as seen by Figure 3.1-1. The required ACIR is around 20 dB with NF 9 dB, 6.4 dB larger than that with the same LSF.

Observation 1: If LSFs between victim BSs and a victim UE and those between aggressor BSs and the same UE are generated independently, the required ACIR would be much larger.

- 3.2 Uplink

This section presents the uplink simulation results, NF 9 dB and 11 dB are evaluated.

Table 3.2-1: Throughput loss at given ACIRs for uplink

| ACIR |  | 5 dB | 10 dB | 15 dB | 20 dB | 25 dB | 30 dB | 35 dB | 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 9 dB | mean | 2.46 | 1.16 | 0.48 | 0.17 | 0.06 | 0.02 | 0.01 | 0 |
|  | 5%-tile | 8.14 | 3.50 | 1.33 | 0.44 | 0.10 | 0 | 0 | 0 |
| NF 11 dB | mean | 2.42 | 1.14 | 0.47 | 0.17 | 0.06 | 0.02 | 0.01 | 0 |
|  | 5%-tile | 8.56 | 3.57 | 1.42 | 0.48 | 0.04 | 0 | 0 | 0 |

Figure 3.2-1: Throughput loss versus ACIR for uplink.

It can be seen from Figure 3.2-1 that even very small values of ACIR (less than 5 dB) can meet the 5% throughput loss requirement for mean throughput. When 5%-tile UE throughput loss is concerned, the required ACIRs with NF 9 dB and NF 11 dB are 8.4 and 8.6, respectively. In this case, the impact of NF is marginal as well.

- 4. Conclusions

This proposal presents ACIR evaluations for coexistence study in urban macro scenario at 30 GHz carrier frequency. Based on the simulation results, we have one observation and one proposal:

Observation 1: If LSFs between victim BSs and a victim UE and those between aggressor BSs and the same UE are generated independently, the required ACIR would be much larger.

- 5. References

- R4-168767 [5G NR] WF on SINR vs throughput mapping_v1, Huawei.

- R4-168953_r1, “WF on Simulation assumptions of Co-existence study for WP5D”, NTT Docomo.

- R4-167078, “WF on BS beamforming model for the ITU WP5D coexistence simulations”, Huawei, HiSilicon.

- R4-167123, “WF on UE beamforming model for the ITU WP5D coexistence simulations”, Qualcomm.

- R4-168795, “TPC_urban,” NTT Docomo, Qualcomm.