3GPP TSG-RAN WG4 Ad-hoc 	R4-1700229

Spokane, Washington, USA 17th – 19th January 2017

Agenda item:	3.1.2.1

Source: 	Qualcomm Incorporated

Title: 	A feasible ACLR metric and operating point for mm-wave UE’s

Document for:	Discussion

# Introduction

In [1], we presented co-existence analysis that shows that an ACLR performance of -16 dBc is required at the UE. This paper supports that conclusion with lab measurements on a mm-wave CMOS power amplifier. The device was designed on a 28nm CMOS process technology. The design extends ideas from prior work in [2]. All lab measurements were taken at 28 GHz, in line with our co-existence simulations.

Based on our RFIC power analysis, we are targeting an output power of 14 dBm. At this operating point, the PAE of the amplifier is 27-28% and our goal is to be as close as possible to PAE’s achieved in sub-6 GHz PA’s for LTE. In the following sections, it will become apparent that at this operating point of PAE and power, the ACLR of of the mm-wave PA will be -16 dBc.

# Test Setup

The lab setup and waveform parameters are outlined in the table below.

Table 1: Setup assumptions

| System bandwidth | 100MHz component carrier |
| --- | --- |
| Number of component carriers | 1 |
| Carrier frequency | 28 GHz |
| Guard band overhead | 10% (5MHz at each band edge) |
| Cyclic prefix overhead | 7% |
| Weighted overlap and add (WOLA) overhead | 2% for ACLR measurements |
| Modulation | QPSK |
| Waveform | OFDM: random bits modulated |
| Waveform bandwidth used | 90 MHz (all 1200 REs used) |
| ACLR measurement | Power in 90 MHz usable frequency of the two adjacent (upper and lower) channel bandwidths, relative to in-band power (dB) |

# Lab Measurements

## ACLR and PAE

Figure 1 shows the measured ACLR in the upper and lower channel bands adjacent to the band of interest. The data shows that, as power increases, the ACLR increases dramatically . At 14 dBm output power, the ACLR reaches a level of  -16 dBc.

Figure 1 ACLR vs output power

Fig. 2 shows the variation of ACLR versus PAE. We are targeting a minimum PAE of 27 to 28%, and the figure shows that the ACLR operating point is -16 dBc .

Figure 2 ACLR vs PAE

Meanwhile, Fig. 3 shows that the PAE of the PA increases with output power, and at 14 dBm of output power, a PAE of close to 27-28% is achieved.

Figure 3 PAE vs output power

# Conclusions

We have presented ACLR and PAE measurements to determine a feasible operating point based on lab measurements of a CMOS power amplifier operating at 28 GHz. The operating point is well aligned with the coexistence simulations in [1].

The analysis show that the ACLR level should be -16 dBc.

# References

- R4-1700045, “ACIR, ACLR and ACS proposals”, Qualcomm Incorporated, 3GPP RAN4, Spokane, January, 2016.

- "A 28GHz efficient linear power amplifier for 5G phased arrays in 28nm bulk CMOS", S. Shakib, H. C. Park, J. Dunworth, V. Aparin and K. Entesari, 2016 IEEE International Solid-State Circuits Conference (ISSCC), San Francisco, CA, 2016, pp. 352-353.