3GPP TSG-RAN WG4 Ad hoc	R4-1700231

Spokane, USA January 17th -19th, 2017

Agenda item:	3.1.2.1

Source: 	Qualcomm Incorporated

Title: 	A feasible ACS blocker specification for mm-wave UE’s

Document for:	Discussion

# Introduction

In [1], we proposed an ACS specification of -23 dBc for mm-wave UE’s based on co-existence studies.  In this paper, we provide a more detailed analysis of the ACS specification, taking into account link budget and RFIC losses. To simplify the analysis, we consider blockers in the spatial beam of the desired signal, without considering spatial rejection.

# Discussion

Consider the jammer scenario in Fig. 1, which has also been proposed in recent papers (cf. [2], [3] and [4]). The ACS blocker lies along the spatial direction of the desired signal, since this represents a worst-case analysis.

DLv

Aggressor

EIRP  = +75dBm (mWB)

Victim

Jammer

=  -77dBm

freq

DLa

ACL jammer

DLv

DLa

Figure 1:  In-band ACS blocker

Consider the major impairments or issues caused by strong in-band blockers:

- Inter-modulation terms in the RX path that de-sensitizes the LNA

- Adjacent channel leakage that appears in-band at the victim

- Reciprocal mixing of the aggressor into victim’s baseband due to phase noise

- Increased analog filtering requirements at the victim, or higher residual signal if blocker  is not adequately filtered

- Higher dynamic range in the digital data path.

Instead of studying the entire RF receive chain, which is strongly implementation dependent, we study the channel link budget as seen by the UE, and the major impairments of the first 1 or 2 elements in the RF chain, particularly the LNA.

The analysis is presented in Table 1. The analysis uses parameters that are consistent with the co-existence studies, including antenna configuration, path losses, shadowing effects, ACLR values and NF. The phase-noise mask is as proposed in [4].

Table 1

The analysis in Table 1 shows that an ACS level of -23 dBc is feasible, i.e., the inter-mods, adjacent-channel leakage, and reciprocally mixed terms will de-sensitize the RX path to a small extent, but not appreciably, as these terms are still lower than thermal noise floor.

The associated blocker level at an ACS of -23dBc is about -55 dBm.

# Conclusion

In this paper, we presented an analysis for a feasible ACS level that is aligned with the proposal in [1]. The proposed ACS value of -23 dBc, and associated blocker level of -55 dBm, satisfies RFIC constraints and link budget.

# References

- R4-1700045, “ACIR, ACLR and ACS proposals”, Qualcomm Incorporated, 3GPP RAN4, Spokane, January, 2016.

- R4-165770, “UE RF requirements for mmWave,” Qualcomm Incorporated, 3GPP RAN4 #80, Sweden, August 2016

- R4-168394,‘‘Consideration of ACS/blocking in mm Wave”, NTT Docomo, 3GPP RAN4, #80Bis, Slovenia, October, 2016.

- R4-1610006, “NR blocking requirements for mmWave”, 3GPP RAN4 #81, Reno, November, 2016.

- R1-165005, “On the evaluation of the PN model”, Nokia, Alcatel-Lucent Shanghai Bell, 3GPP-RAN1 #85, Nanjing, P.R. China, May , 2016.