3GPP TSG-RAN WG4 NR AH Meeting	R4-1700005

Spokane, Washington, USA, 17 - 19 January, 2017

Agenda Item:	3.1.1.2

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Proposed ACIR values for coexistence study for WP5D on new radio access technology

Document for:	Approval

# 1.	Introduction

The revised study item on New Radio Access Technology was approved at TSG RAN#72 [1]. The objectives of this study item include identifying relevant RF parameters to be used for sharing and co-existence studies. Also a LS was received at TSG RAN#72 [2] from ITU-R WP5D asking for characteristics of terrestrial IMT systems for frequency sharing/interference analysis in 24.25 - 86 GHz. ITU-R WP5D sent another LS on “Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz” in [3]. On the other hand, a LS has been sent from ITU-R WP5D to 3GPP describing detailed modelling and simulation of IMT networks for use in sharing and compatibility studies [4]. This topic has been discussed at previous RAN4 meetings, and the way forwards on simulation assumptions were approved in RAN4#80bis [5-7] and RAN4#81 [8-10].

This contribution proposes the ACIR values to be used to derive the ACLR and ACS values for coexistence study for WP5D on new radio access technology, according to the simulation results provided in [11].

2.	ACIR values

The ACIR values according to the simulation results in [11] are provided in Table 1 below:

Table 1: ACIR values from simulation results

| No. | Simulation frequency | Direction | Deployment Scenario | NF | ACIR (dB) for 5% throughput loss (average throughput) | ACIR (dB) for 5% throughput loss (5%ile throughput) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 30 GHz | DL to DL | Indoor hotspot | 9 | 15.7 | 14 |
|  |  |  |  | 11 | 15.7 | 14 |
| 2 | 30 GHz | DL to DL | Urban macro (ISD=200m) | 9 | 11.9 | 19.9 |
|  |  |  |  | 11 | 11.8 | 19.9 |
| 3 | 30 GHz | DL to DL | Dense urban | 9 | <5 | 7.4 |
|  |  |  |  | 11 | <5 | 8.5 |
| 4 | 30 GHz | UL to UL | Indoor hotspot | 9 | 10.8 | 14.8 |
|  |  |  |  | 11 | 9.6 | 14.7 |
| 5 | 30 GHz | UL to UL | Urban macro (ISD=200m) | 9 | 5.7 | 14.9 |
|  |  |  |  | 11 | <5 | 15.1 |
| 6 | 30 GHz | UL to UL | Dense urban | 9 | <5 | 8.9 |
|  |  |  |  | 11 | <5 | NA |
| 7 | 70 GHz | DL to DL | Indoor hotspot | 13 | 16.2 | 16.1 |
|  |  |  |  | 15 | 16 | 15.8 |
| 8 | 70 GHz | DL to DL | Dense urban | 13 | <5 | NA |
|  |  |  |  | 15 | <5 | NA |
| 9 | 70 GHz | UL to UL | Indoor hotspot | 13 | 8.6 | 14.5 |
|  |  |  |  | 15 | 7.4 | 14.2 |
| 10 | 70 GHz | UL to UL | Dense urban | 13 | 5.5 | NA |
|  |  |  |  | 15 | <5 | NA |

# 3.	Conclusion

This contribution proposes the ACIR values to be used to derive the ACLR and ACS values, according to the provided simulation results. It is proposed to adopted the ACIR values in Table 2 below:

Table 2: Proposed ACIR values

| No. | Simulation frequency | Direction | Deployment Scenario | Proposed ACIR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 30 GHz | DL to DL | Indoor hotspot | 16 |
| 2 | 30 GHz | DL to DL | Urban macro (ISD=200m) | 20 |
| 3 | 30 GHz | DL to DL | Dense urban | 9 |
| 4 | 30 GHz | UL to UL | Indoor hotspot | 15 |
| 5 | 30 GHz | UL to UL | Urban macro (ISD=200m) | 16 |
| 6 | 30 GHz | UL to UL | Dense urban | 9 |
| 7 | 70 GHz | DL to DL | Indoor hotspot | 17 |
| 8 | 70 GHz | DL to DL | Dense urban | TBD |
| 9 | 70 GHz | UL to UL | Indoor hotspot | 15 |
| 10 | 70 GHz | UL to UL | Dense urban | TBD |

# References

[1]	RP-161214, “Revision of SI: Study on New Radio Access Technology”, NTT DOCOMO, INC.

[2]	RP-160508, “LS on Characteristics of terrestrial IMT systems for frequency sharing/interf. analysis in 24.25 - 86 GHz”, ITU-R WP5D.

[3]	R4-165140, Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz, ITU-R WP5D

[4]	R4-165141, “Modelling and simulation of IMT networks for use in sharing and compatibility studies”, ITU-R WP5D.

[5]	R4-168953, “WF on Simulation assumptions of Co-existence study for WP5D”, NTT DOCOMO, INC.

[6]	R4-168794, “Further Way forward on Dense urban Layout and BS Beamforming Model”, Huawei, HiSilicon.

[7]	R4-167750, “Clarifications about UE antenna modeling for NR coexistence study”, Qualcomm Incorporated.

[8]	R4-1609497, “On throughput model of dense urban and indoor hotspot”, CATT.

[9]	R4-1610153, “Proposal on dense urban network layout for coexistence study for WP5D on new radio access technology”, Nokia, Alcatel-Lucent Shanghai Bell.

[10]	R4-1610634, “WF on urban macro scenario for coexistence study for WP5D on new radio access technology”, Nokia, Alcatel-Lucent Shanghai Bell, Qualcomm, Samsung, CATT, ZTE, NTT DOCOMO, Intel, Ericsson.

[11]	R4-1700004, “Summary of simulation results for coexistence study for WP5D on new radio access technology”, Nokia, Alcatel-Lucent Shanghai Bell.