3GPP TSG-RAN WG4 Meeting #1 NR	R4-1700008

Spokane, Washington, USA, January 17th -19th, 2017

Agenda item:	3.5.3 - Receiver characteristics

Source: 	Korea Testing Laboratory, ETRI

Title: 	Out-of-band blockers in the mm-wave spectrum

Document for:	Discussion

Introduction

Millimeter wave blockers (or jammers) can be the critical factor to degrade the receiver performance at mm-wave bands as discussed in [1]. Therefore, this paper provides preliminary surveys on the major sources of blocking in Korea where all NR spectrum bands have been recently announced according to the K-ICT Spectrum plan [2].

Discussion

The following table lists the major blockers in the mm-wave bands in the Korea.

| Start (GHz) | Stop (GHz) | Usage | Comment |
| --- | --- | --- | --- |
| 3.4 | 3.7 | Bandwidth 300 MHz, NR Spectrum plan by 2018 | K ICT spectrum plan |
| 19.7 | 21.2 | Satellite |  |
| 21.2 | 21.4 | Earth Exploration Satellite |  |
| 21.4 | 22 | Satellite |  |
| 22.1 | 23.6 | Point-to-Point fixed wireless |  |
| 23.6 | 24 | Earth Exploration Satellite |  |
| 24 | 24.05 | Amateur Station(24.025 GHz) | FACS(Flexible Access Common Spectrum) 3, [3] |
| 24.05 | 24.25 | Radar Motion Sensor | FACS(Flexible Access Common Spectrum) 3, [3] |
| 24.25 | 26.65 | Automotive Radar | FACS(Flexible Access Common Spectrum) 3, [3] |
| 27 | 27.5 | Satellite |  |
| 27.5 | 28.5 | Bandwidth 1 GHz, NR Spectrum plan by 2018, K ICT spectrum plan | 26.5 - 27.5 GHz, 28.5 - 29.5 GHz, Additional NR, K ICT spectrum plan |
| 28.5 | 29.1 | Satellite |  |
| 29.1 | 29.5 | Satellite |  |
| 29.5 | 29.9 | Satellite |  |
| 29.9 | 30 | Satellite |  |
| 30 | 31 | Satellite |  |
| 31 | 31.3 | HAPS(High Altitude Platform Station) |  |
| 31.3 | 31.5 | Experimental station (31.4 GHz) |  |
| 31.5 | 31.8 | Satellite |  |
| 31.8 | 33.4 | Aviation |  |
| 33.4 | 34.2 | Radio Detection |  |
| 34.2 | 35.2 | Intelligence Highway System |  |
| 35.2 | 35.5 | Meteorological Aid |  |
| 35.5 | 36 | Satellite |  |
| 36.5 | 42.5 | FWS(Fixed Wireless Systems) |  |
| 42.5 | 43.5 | Satellite |  |
| 43.5 | 47 | Satellite |  |
| 47 | 47.2 | Amateur station (47.1 GHz) |  |
| 47.2 | 50.2 | Satellite |  |
| 50.2 | 50.4 | Earth Exploration Satellite |  |
| 50.4 | 51.4 | Satellite |  |
| 51.4 | 52.6 | Satellite |  |
| 52.6 | 54.25 | Earth Exploration Satellite |  |
| 54.25 | 55.78 | Earth Exploration Satellite |  |
| 55.78 | 56.9 | Earth Exploration Satellite |  |
| 56.9 | 57 | Earth Exploration Satellite |  |
| 57 | 66 | FACS(Flexible Access Common Spectrum) 1 |  |
| 66 | 71 | Satellite |  |
| 71 | 76 | Point-to-Point fixed wireless |  |
| 76 | 77 | Automotive Radar |  |
| 81 | 86 | Point-to-Point fixed wireless |  |

Conclusion

This paper presents preliminary blocker data in mm-wave bands based on the K-ICT Spectrum plan [2].

References

- R4-1609756,” Out-of-band blockers in the mm-wave spectrum,” Qualcomm Incorporated, 3GPP RAN4 #81, Nov 2016

- Ministry of Science, ICT and Future Planning, K-ICT Spectrum plan, Dec. 22, 2016. (www.msip.go.kr)

- Institute for Future Radio Engineering, Spectrum Management, (http://www.ifre.re.kr/)