3GPP TSG-RAN WG4 NR AH Meeting	R4-1700014

Spokane, Washington, USA, 17 - 19 January, 2017

Source: 	China Telecom

Title: 	NR coexistence results for urban macro scenario

Agenda Item:	3.1.1.1

Document for:	Discussion

Introduction

In RAN4 #81 meeting, the WF [1] on urban macro scenario for coexistence study for WP5D was agreed, which decides to adopt the following assumptions and parameters for UMa DL and UL coexistence simulation in order to achieve a meaningful 5%-tile cell edge throughput metric, as well as a higher UL B/W efficiency.

- Assumptions and parameters (baseline):

  - 200 MHz B/W

  - 200 m ISD

  - 20% indoor user ratio (assuming indoor users are mostly served by small cells at mmWave frequency)

- Assumptions and parameters (optional):

  - 200 MHz B/W

  - 300 m ISD

  - 20% indoor user ratio (assuming indoor users are mostly served by small cells at mmWave frequency)

According to the calibration procedures in [2], we have provided the calibration results for UMa scenario via RAN4 email reflector before the AH meeting [3]..

In this contribution, we provide DL and UL ACIR results for unban macro scenario and the detailed source data can be found in the attachment.

Simulation assumptions

The simulation scenario for urban macro from WF [1] is given in the table below.

| No. | Aggressor | Victim | Simulation frequency | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Urban macro |
| 5 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Urban macro |

Details on urban macro scenarios are listed in the following table.

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Bandwidth |  | 200MHz |  |
| Inter-site distance |  | Baseline: 200m Optional: 300m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.900 |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa | Specified in TR38.900 |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Noise Figure |  | 9 or 11 dB | 30GHz |
| TPC model |  | CL-xile = 88 + 10*log10(200/X) | X: UL transmission BW |
| Evaluation metric |  | Based on [4] |  |

ACIR results for urban macro scenario

According to the simulation assumptions above, the ACIR results and source data for urban macro scenario are provided in the following tables.

DL ACIR results

- Urban macro DL with 30GHz, ISD=200m, NF=9dB

| Sourcing | Scenario No 2 | NR 30GHz, ISD=200m, NF=9dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL ACIR | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| China Telecom | Throughput loss (average) | 13.50 | 7.92 | 4.36 | 2.26 | 1.08 | 0.48 | 0.21 | 0.08 |
|  | Throughput loss (5%-tile) | 43.91 | 26.01 | 11.25 | 5.59 | 1.90 | 0.55 | 0.18 | 0.00 |

- Urban macro DL with 30GHz, ISD=200m, NF=11dB

| Sourcing | Scenario No 2 | NR 30GHz, ISD=200m, NF=11dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL ACIR | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| China Telecom | Throughput loss (average) | 13.43 | 7.88 | 4.34 | 2.25 | 1.07 | 0.48 | 0.20 | 0.08 |
|  | Throughput loss (5%-tile) | 42.60 | 23.64 | 12.15 | 5.03 | 1.97 | 1.19 | 0.32 | 0.00 |

- Urban macro DL with 30GHz, ISD=300m, NF=9dB

| Sourcing | Scenario No 2 | NR 30GHz, ISD=300m, NF=9dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL ACIR | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| China Telecom | Throughput loss (average) | 15.69 | 9.85 | 5.81 | 3.23 | 1.70 | 0.85 | 0.40 | 0.18 |
|  | Throughput loss (5%-tile) | 59.18 | 40.05 | 24.76 | 11.74 | 6.14 | 2.26 | 1.23 | 0.31 |

- Urban macro DL with 30GHz, ISD=300m, NF=11dB

| Sourcing | Scenario No 2 | NR 30GHz, ISD=300m, NF=11dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL ACIR | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| China Telecom | Throughput loss (average) | 15.60 | 9.80 | 5.79 | 3.21 | 1.69 | 0.85 | 0.40 | 0.18 |
|  | Throughput loss (5%-tile) | 58.18 | 39.98 | 24.17 | 13.65 | 5.99 | 1.60 | 0.55 | 0.00 |

- Summary of ACIR results for urban macro DL

| Sourcing | Scenario No 2 | NR 30GHz, ISD=200m, NF=9dB | NR 30GHz, ISD=200m, NF=11dB | NR 30GHz, ISD=300m, NF=9dB | NR 30GHz, ISD=300m, NF=11dB |
| --- | --- | --- | --- | --- | --- |
| China Telecom | DL ACIR for 5% throughput loss (average) | 14dB | 14dB | 17dB | 17dB |
|  | DL ACIR for 5% throughput loss (5%-tile) | 21dB | 21dB | 27dB | 27dB |

Observation 1: Considering the criterion of less than 5% cell average and cell edge throughput loss, the required ACIR for urban macro DL are summarized as:

- ACIR=21dB for ISD=200m, NF=9dB with 30GHz

- ACIR=21dB for ISD=200m, NF=11dB with 30GHz

- ACIR=27dB for ISD=300m, NF=9dB with 30GHz

- ACIR=27dB for ISD=300m, NF=11dB with 30GHz

UL ACIR results

- Urban macro UL with 30GHz, ISD=200m, NF=9dB

| Sourcing | Scenario No 5 | NR 30GHz, ISD=200m, NF=9dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL ACIR | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| China Telecom | Throughput loss (average) | 7.18 | 4.05 | 2.15 | 1.08 | 0.51 | 0.23 | 0.10 | 0.04 |
|  | Throughput loss (5%-tile) | 37.61 | 21.97 | 12.52 | 5.87 | 2.23 | 0.32 | 0.06 | 0.06 |

- Urban macro UL with 30GHz, ISD=200m, NF=11dB

| Sourcing | Scenario No 5 | NR 30GHz, ISD=200m, NF=11dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL ACIR | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| China Telecom | Throughput loss (average) | 6.34 | 3.52 | 1.83 | 0.89 | 0.42 | 0.18 | 0.07 | 0.03 |
|  | Throughput loss (5%-tile) | 32.46 | 20.67 | 11.88 | 3.72 | 0.82 | 0.27 | 0.00 | 0.00 |

- Urban macro UL with 30GHz, ISD=300m, NF=9dB

| Sourcing | Scenario No 5 | NR 30GHz, ISD=300m, NF=9dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL ACIR | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| China Telecom | Throughput loss (average) | 9.35 | 5.39 | 2.89 | 1.45 | 1.29 | 0.30 | 0.13 | 0.05 |
|  | Throughput loss (5%-tile) | NA | NA | NA | NA | NA | NA | NA | NA |

- Urban macro UL with 30GHz, ISD=300m, NF=11dB

| Sourcing | Scenario No 5 | NR 30GHz, ISD=300m, NF=11dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL ACIR | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| China Telecom | Throughput loss (average) | 8.29 | 4.68 | 2.46 | 1.21 | 0.56 | 0.24 | 0.10 | 0.04 |
|  | Throughput loss (5%-tile) | NA | NA | NA | NA | NA | NA | NA | NA |

- Summary of ACIR results for urban macro UL

| Sourcing | Scenario No 5 | NR 30GHz, ISD=200m, NF=9dB | NR 30GHz, ISD=200m, NF=11dB | NR 30GHz, ISD=300m, NF=9dB | NR 30GHz, ISD=300m, NF=11dB |
| --- | --- | --- | --- | --- | --- |
| China Telecom | UL ACIR for 5% throughput loss (average) | 10dB | 8dB | 11dB | 9dB |
|  | UL ACIR for 5% throughput loss (5%-tile) | 22dB | 19dB | NA | NA |

Observation 2: Considering the criterion of less than 5% cell average and cell edge throughput loss, the required ACIR for urban macro UL are summarized as:

- ACIR=22dB for ISD=200m, NF=9dB with 30GHz

- ACIR=19dB for ISD=200m, NF=11dB with 30GHz

- ACIR=11dB for average UEs, and 5%-tile UEs in outage for ISD=300m, NF=9dB with 30GHz

- ACIR=9dB for average UEs, and 5%-tile UEs in outage for ISD=300m, NF=11dB with 30GHz

Conclusion

In this contribution, we provide the simulation results for ACIR in urban macro scenario and have the following observations:

Observation 1: Considering the criterion of less than 5% cell average and cell edge throughput loss, the required ACIR for urban macro DL are summarized as:

- ACIR=21dB for ISD=200m, NF=9dB with 30GHz

- ACIR=21dB for ISD=200m, NF=11dB with 30GHz

- ACIR=27dB for ISD=300m, NF=9dB with 30GHz

- ACIR=27dB for ISD=300m, NF=11dB with 30GHz

Observation 2: Considering the criterion of less than 5% cell average and cell edge throughput loss, the required ACIR for urban macro UL are summarized as:

- ACIR=22dB for ISD=200m, NF=9dB with 30GHz

- ACIR=19dB for ISD=200m, NF=11dB with 30GHz

- ACIR=11dB for average UEs, and 5%-tile UEs in outage for ISD=300m, NF=9dB with 30GHz

- ACIR=9dB for average UEs, and 5%-tile UEs in outage for ISD=300m, NF=11dB with 30GHz

- References

- WF on urban macro scenario for coexistence study for WP5D on new radio access technology, Nokia, Alcatel-Lucent Shanghai Bell, Qualcomm, Samsung, CATT, ZTE, NTT DOCOMO, Intel, Ericsson

- R4-167128,	Work plan on NR co-existence study for WP5D, NTT DOCOMO

- R4-17xxxx, Calibration on NR coexistence study for WP5D_#81AH

- R4-168767,	WF on SINR vs throughput mapping, Huawei, Hisilicon