3GPP TSG-RAN WG4 #NR AH Meeting	R4-1700062

Spokane, USA, 17 - 19 January 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Discussion on bandwidth adaptation in NR

Agenda item:		3.2.2

Document for:		Discussion

# 1	Introduction

- At least for single carrier operation, NR should allow a UE to operate in a way where it receives at least downlink control information in a first RF bandwidth and where the UE is not expected to receive in a second RF bandwidth that is larger than the first RF bandwidth within less than X µs (FFS: value of X)

  - FFS the first RF bandwidth is within the second RF bandwidth

    - FFS the first RF bandwidth is at the center of the second RF bandwidth

  - FFS the maximal ratio of the first RF bandwidth over the second RF bandwidth

  - FFS detailed mechanism

- FFS RF bandwidth adaptation for RRM measurement

RAN1 has sent LS to RAN4 in [1]. The topic concerns UE RF bandwidth adaptation in relation to which RAN1 agreed following in the Reno meeting:

Based on this RAN1 had some further discussion related to the UE BW adaptation for DL data, DL measurements and UL control/data topic without further agreements. Based on the discussion RAN1 would like to ask RAN4 to study the following points for UE RF bandwidth adaptation in single and multiple carrier operation:

- How fast is the UE RF bandwidth adaptation

- How much power saving is possible for UE RF bandwidth adaptation

- Other benefits

- Whether any of the above depends on the conditions, such as

  - Whether or not first and second RF bandwidth are centered at the same frequency

  - Whether or not first RF bandwidth are partially or fully contained in the second RF bandwidth

  - The ratio of first and second RF bandwidth

  - Whether or not first and second RF bandwidth are in the same band

  - Dependency of modulation scheme

  - Whether or not neighbor cell synchronization signals are within first RF bandwidth

  - Whether or not first and/or second RF bandwidth are centered at the same frequency as neighbor cell synchronization signals

  - Whether or not additional reference signals are needed, for example for AGC settling

  - Whether it depends on transmission direction

The questions from RAN1 are partly related to RF and partly related to RRM (the last 5 questions). In the following we will address the topics which we consider relevant to RF. Additionally we will also discuss shortly on the RRM related topics but we expect those to be handled during the Febaruary ordinary meeting in Athens.

# 2	UE bandwidth adaptation – RF discussion

RAN4 was asked to study the UE RF bandwidth adaptation in single and multiple carrier operation. In the October meeting RAN1 decided following:

- At least for Phase 1, study mechanisms to support operation over e.g. around 1GHz contiguous spectrum from both NW and UE perspectives including the maximum single carrier bandwidth of at least 80 MHz

  - Carrier Aggregation/Dual Connectivity (Multi-carrier approach)

    - Details are FFS

    - FFS: non-contiguous spectrum case

  - Single carrier operation

    - Details are FFS

    - Maximum channel bandwidth continues to be studied in RAN1/4

      - Maximum bandwidth supported by some UE capabilities/categories may be less than channel bandwidth of serving single carrier

        - Note that some UE capabilities/categories may support channel bandwidth of serving single carrier

Which means that while study support for operation over 1GHz contiguous spectrum from both network and UE point of view, the assumed maximum single carrier bandwidth in phase 1 is at least 80MHz. Depending on the UE capability or category the supported bandwidth may be same or lower than the bandwidth of the serving single carrier.

From the LS text it is clear that at least for single carrier operation it is important to be able to adjust the UE operation bandwidth in a rather flexible manner depending on whether UE is receiving control and data. Such switching in bandwidth should be done within X µsec where the expected value of X is to be discussed in RAN4.

## 2.1 Bandwidth adaptation within the same band

When discussing the expected switching time RAN4 could initially discuss whether it is expected that the UE switching time would be the same for when increasing the bandwidth? I.e. UE switches from narrower bandwidth to larger bandwidth – and decreasing the bandwidth, within the bandwidth supported by the UE category – without changing the center frequency. From RF point of view the bandwidth adaptation would likely not be depend on whether the adaptation in an increase or decrease.

Observation 1: We expect that RF BW adaptation latency does not depend on whether the BW is increased or decreased.

Secondly RAN4 should discuss if it from RF switching point of view would impact the RF switching time if a change in the center frequency would have an effect? I.e. first RF bandwidth is partially or fully contained in the second RF bandwidth. E.g. if the UE supports 80MHz bandwidth and is currently operating at 20MHz at f1 would need to switch to operate at 40MHz at f2 where the 20MHz (at f1) is partially or fully within the second operating bandwidth.

It could be expected that a change in center center frequency would add some additional latency to the BW adaptation compared to the situation where the center frequency is not changed. Whether there is overlap between the BW before and after adapation is likely not having impact on the latency.

Observation 2: We expect that RF BW adaptation latency would be impacted if the adaptation includes change of center frequency.

RAN4 should also discuss whether any ratio conditions between the first and second bandwidths impacts the switching time. E.g. if the first RF bandwidth is 20MHz and second RF bandwidth is 40 MHz – would the switching time be same or different than if the first RF bandwidth is 20MHz and second RF bandwidth is 100 MHz?

## 2.2 Bandwidth adaptation within different bands

The three first question were related to RF switching within the same band. The final question related to RF, concerns whether bandwidth adaptation including switching between different bands, would have an impact on the UE RF adaptation lateny. E.g. if UE is operating in band 1 using 10MHz and would need to switch to band 2 using 20MHz would this bandwidth adaptation latency be same or different than if the bandwidth adaptation would be within same band like discussed in section 2.1?

In line with the discussion in section 2.1, a bandwidth adaptation which includes band switch would also include a change in center frequency. Based on this we expect that a bandwidth adaptation including band change would have longer latency than bandwidth adaptation which does not include band change.

Observation 3: We expect that a bandwidth adaptation including band change would have longer latency than bandwidth adaptation which does not include band change.

# 3	UE bandwidth adaptation – RRM discussion

In the former section we discussed the RF switching time related to bandwidth adaptation. RF switching time is of course an important and celtral factor when discussing RF bandwidth adaption and estimating the related UE delays. However, it is also important to look at the overall expected latency impact from bandwidth adaptations.

When it comes to system performance impact, one also have to look at the overall delay in terms of any effect on data transmissions. In this sense the delays occurring from UE RRM and demodulation would need to be accounted.

The LS asks RAN4 to study whether the UE BW adaptation would impact on DL data, DL measurements and UL control/data. RAN1 is asking specifically on a number of topics which we shortly address below although we recognize that they are not RF related as such:

  - Dependency of modulation scheme

  - Whether or not neighbor cell synchronization signals are within first RF bandwidth

  - Whether or not first and/or second RF bandwidth are centered at the same frequency as neighbor cell synchronization signals

  - Whether or not additional reference signals are needed, for example for AGC settling

  - Whether it depends on transmission direction

Changing the carrier center frequency or carrier bandwidth and the impact on the UE reception performance, has been discussed within the LTE CA and DC frame work. For LTE RAN4 RRM have defined UE requirements such as interruption time e.g. due to configuration and activation of SCells.

One could expect that RAN4 RRM would need to have a similar discussion for the same operations in 5G – i.e. adaptation of the UE RF bandwidth. The interruption requirements (for when the UE is physically changing or switching the BW) is one part impacting the system. Additionally we could expect an additional bandwidth adaptation latency is needed in order to allow the UE to prepare for reception and transmission following a bandwidth adaptation.

The detail related to an additional latency would need to be discussed in RAN4 and would likely depend on many parameters – among which some are not yet decided in RAN1 – e.g. synchronization design, synchronization signal transmission, availability of reference signals and likely transmission direction. Additionally it will likely also depend on which basic assumptions the UE can make during the bandwidth adaptation.

Whether modulation scheme would be impacting the overall latency which can be expected from a bandwidth adaption would need further discussions – once RAN1 has decided on which MCSs the 5G will support. RAN1#86bis agreed to support at least the modulation schemes in LTE (i.e. QPSK, 16QAM, 64QAM and 256QAM), while other enhanced modulation schemes are still under investigation.

As a summary our view is that it can be expected that additional latencies in UL and DL control and data transmissions will occur in connection with a bandwidth adaptation. Such RRM related latencies would be in addition to the latencies occurring from UE RF adaptation and what can be expected from the basic RF tuning.

# 4	UE bandwidth adaptation – Power Savings discussion

RAN1 also ask RAN4 about the UE power savings expected from from the UE RF bandwidth adaptation. Such potential power saving would depend on the UE implementation and architecture. Some initial discussions on power consumption savings in 5G has been ongoing in RRM – addressing both UE and gNB power savings [2, 3]. The discussions are still in the early phases.

In order to get some estimate on the potential power saving from bandwidth adaptation RAN4 would likely need to estimate whether there from RF point of view would be saving from operating on narrower bandwidth, and also estimate potential power saving gain. Such power saving gain could even be estimated in a relative manner. Input from  RRM and demod is also needed. E.g. RAN4 could try to estimate if there would be a noticeable power savings in terms of reduced processing on UE side from operating on narrower bandwidth.

Although exact estimates on potential power saving gain from adaptive bandwidth operation, we expect that there will be some power saving opportunities from enabling UE bandwidth adaptation.

Observation 4: We expect that there will be some power saving opportunities from enabling UE bandwidth adaptation.

# 3	Conclusions

RAN1 has sent LS to RAN4 in [1]. The topic concerns UE RF bandwidth adaptation in relation to which RAN1 made a number of agreements in the Reno meeting. RAN1 had some further discussion related to the UE BW adaptation for DL data, DL measurements and UL control/data topic without further agreements based on which an LS was sent to RAN4. In this paper we have discussed the topics which we consider relevant to RF based on which we make a number of observations:

Observation 1: We expect that RF BW adaptation latency does not depend on whether the BW is increased or decreased.

Observation 2: We expect that RF BW adaptation latency would be impacted if the adaptation includes change of center frequency.

Observation 3: We expect that a bandwidth adaptation including band change would have longer latency than bandwidth adaptation which does not include band change.

Observation 4: We expect that there will be some power saving opportunities from enabling UE bandwidth adaptation.

Additionally we also discussed on the RRM related topics but we expect those to be handled during the Febaruary ordinary meeting in Athens.

# 4	References

- R1-1613663, LS on UE RF Bandwidth Adaptation in NR, RAN1

- R4-1609329, Power consumption discussion, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1609565, Way forward for NR RRM, Ericsson, Nokia