3GPP TSG-RAN WG4 NR Ad-hoc	R4- 1700164

Spokane, Washington, USA, 17 – 19 January 2017

Agenda Item:	3.1.1.1

Source: 	Samsung

Title:	Simulation results for NR coexistence study in dense urban scenario

Document for:	Discussion

- 1. Introduction

The ACIR is an important parameter for the design of RF circuits, and it imposes a constraint on adjacent channel interference so that throughput loss can be within a reasonable level. It was already observed in previous meetings that the required ACIRs in different scenarios can be quite different. Thus it is necessary to provide the ACIR simulation results for each specific scenario, i.e., urban macro, dense urban and indoor scenario.

This contribution presents the ACIR evaluation results in dense urban scenario at 30 GHz and 70 GHz carrier frequency for both downlink and uplink.

- 2. Discussion

- 2.1 Coexistence simulation case

The NR is assumed under synchronized network, where aggressor and victim have the same configuration.

Table 2.1-1 Simulation cases of coexistence study for NR

| No. | Aggressor | Victim | Simulation frequency | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Dense urban |
| 6 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Dense urban |
| 8 | NR, 200MHz | NR, 200MHz | 70 GHz | DL to DL | eMBB | Dense urban |
| 10 | NR, 200MHz | NR, 200MHz | 70 GHz | UL to UL | eMBB | Dense urban |

- 2.2	Simulation settings

Detailed simulation assumptions, i.e., network layout, propagation model, etc. were agreed in [1]. BS beamforming, UE beamforming follow the agreements in [2] and [3], respectively. Beamforming are employed at both the BS side and the UE side. Only the antenna element gain of BSs and UEs are considered in the cell selection process.

- 3. Simulation results

  - 30 GHz

- 3.1.1 Downlink

The results of throughput loss with NF 9 dB and 11 dB are given below.

Table 3.1-1: Throughput loss at given ACIRs for 30 GHz, downlink

| Case |  | # 5 dB | # 10dB | # 15dB | # 20dB | # 25dB | # 30dB | # 35dB | # 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 9 dB | mean | 4.76 | 2.60 | 1.34 | 0.66 | 0.30 | 0.13 | 0.05 | 0.02 |
|  | 5%-tile | 9.33 | 3.82 | 1.34 | 0.60 | 0.04 | 0.04 | 0 | 0 |
| NF 11 dB | mean | 4.62 | 2.51 | 1.29 | 0.63 | 0.29 | 0.12 | 0.05 | 0.02 |
|  | 5%-tile | 7.19 | 2.82 | 1.20 | 0.46 | 0.18 | 0 | 0 | 0 |

Figure 3.1-1: Throughput loss versus ACIR for 30 GHz, downlink.

From Figure 3.1-1, it is seen that 5 dB is enough to guarantee a 5% or less loss for mean throughput. When 5%-tile UE throughput is concerned, ACIR should be no less than 8.9 dB and 7.5 dB for NF 9 dB and NF 11 dB, respectively.

- 3.1.2 Uplink

Table 3.1-2 and Figure 3.1-2 are the uplink throughput results.

Table 3.1-2: Throughput loss at given ACIRs for 30 GHz, uplink

| Case |  | # 5 dB | # 10dB | # 15dB | # 20dB | # 25dB | # 30dB | # 35dB | # 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 9 dB | mean | 1.67 | 0.84 | 0.41 | 0.19 | 0.08 | 0.03 | 0.01 | 0 |
|  | 5%-tile | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NF 11 dB | mean | 1.57 | 0.79 | 0.38 | 0.18 | 0.08 | 0.03 | 0.01 | 0 |
|  | 5%-tile | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Figure 3.1-2: Throughput loss versus ACIR for 30 GHz, uplink.

In this case, the 5%-tile UE throughput is not observed due to large interference, however, it is close to the turning point from zero to nonzero in the CDF of UE throughput. As indicated by Figure 3.1-2, 5 dB is enough to ensure a mean throughput loss less than 5%.

  - 70 GHz

- 3.2.1 Downlink

Downlink throughput loss results for 70 GHz are given below.

Table 3.2-1: Throughput loss at given ACIRs for 70 GHz, downlink

| Case |  | # 5 dB | # 10dB | # 15dB | # 20dB | # 25dB | # 30dB | # 35dB | # 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 9 dB | mean | 3.83 | 2.13 | 1.12 | 0.56 | 0.26 | 0.11 | 0.04 | 0.01 |
|  | 5%-tile | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NF 11 dB | mean | 3.74 | 2.08 | 1.10 | 0.55 | 0.26 | 0.10 | 0.03 | 0.01 |
|  | 5%-tile | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Figure 3.2-1: Throughput loss versus ACIR for 70 GHz, downlink.

Due to large path loss in 70 GHz, 5%-tile UE throughput is always zero. As indicated by Figure 3.2-1, 5 dB is enough to make the mean throughput loss less than 5%.

- 3.2.2 Uplink

Table 3.2-2 and Figure 3.2-2 are the 70 GHz uplink UE throughput loss results.

Table 3.2-2: UE throughput loss at given ACIRs for 70 GHz, uplink

| Case |  | # 5 dB | # 10dB | # 15dB | # 20dB | # 25dB | # 30dB | # 35dB | # 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 9 dB | mean | 0.90 | 0.47 | 0.24 | 0.11 | 0.05 | 0.02 | 0.01 | 0 |
|  | 5%-tile | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NF 11 dB | mean | 0.86 | 0.45 | 0.23 | 0.11 | 0.04 | 0.02 | 0.01 | 0 |
|  | 5%-tile | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Figure 3.2-2: UE throughput loss versus ACIR for 70 GHz, uplink.

In this case, the 5%-tile UE throughput is also zero. The impact of ACIR on the mean throughput is limited, as seen in Figure 3.2-2, even with 5 dB ACIR, mean throughput loss is less than 1%.

From Section 3.1 and Section 3.2, it is seen that only for 30 GHz downlink, 5%-tile UE throughput is nonzero, and hence, only for this case can we decide the appropriate ACIR. For other cases, if we decide the required ACIRs based solely on the mean throughput, then the throughput loss of cell edge UEs may not be satisfied. Therefore, it’s better to select a value 3~5 dB higher than that required by the mean throughput.

- 4. Conclusion

This document presents simulation results for coexistence study in dense urban scenario.

# References

- R4-168953_r1, “WF on Simulation assumptions of Co-existence study for WP5D”, NTT Docomo.

- R4-168794 [5G NR], “Further Way forward on Dense urban Layout and BS Beamforming Model_v4,” Huawei.

- R4-167123, “WF on UE beamforming model for the ITU WP5D coexistence simulations”, Qualcomm.

- R4-168767 [5G NR] “WF on SINR vs throughput mapping_v1”, Huawei.