3GPP TSG-RAN WG4 NR Ad-hoc	R4- 1700165

Spokane, Washington, USA, 17 – 19 January 2017

Agenda Item:	3.1.1.1

Source: 	Samsung

Title: 	Simulation results for NR coexistence study in indoor scenario

Document for:	Discussion

- 1. Introduction

The ACIR is an important parameter for the design of RF circuits, and it imposes a constraint on adjacent channel interference so that throughput loss can be within a reasonable level. It was already observed in previous meetings that the required ACIRs in different scenarios can be quite different. Thus it is necessary to provide the ACIR simulation results for each specific scenario, i.e., urban macro, dense urban and indoor scenario.

This contribution presents the ACIR evaluation results in indoor scenario at 30 GHz and 70 GHz carrier frequency for both downlink and uplink.

- 2. Discussion

- 2.1 Coexistence simulation case

The NR eMBB is assumed under synchronized network, where aggressor and victim have the same configuration.

Table 2.1-1 Simulation cases of coexistence study for NR

| No. | Aggressor | Victim | Simulation frequency | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Indoor hotspot |
| 4 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Indoor hotspot |
| 7 | NR, 200MHz | NR, 200MHz | 70 GHz | DL to DL | eMBB | Indoor hotspot |
| 9 | NR, 200MHz | NR, 200MHz | 70 GHz | UL to UL | eMBB | Indoor hotspot |

- 2.2 Simulation parameters

Network layout, propagation model etc. are the same with [1]. BS beamforming, UE beamforming are in agreement with [2], [3], respectively. SINR to Throughput mapping follows [4]. Beamforming are employed at both the BS side and UE side. In addition, only the antenna element gain of BSs and UEs are considered in the cell selection process.

- 3. Simulation results

- 3.1 30 GHz

- 3.1.1 Downlink

This section presents the 30 GHz downlink simulation results.

Table 3.1-1: Throughput loss at given ACIRs for 30 GHz downlink

| ACIR |  | 5 dB | 10 dB | 15 dB | 20 dB | 25 dB | 30 dB | 35 dB | 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 9 dB | mean | 17.52 | 10.21 | 5.39 | 2.53 | 1.02 | 0.33 | 0.10 | 0.03 |
|  | 5%-tile | 34.46 | 15.04 | 4.18 | 1.02 | 0.33 | 0.10 | 0.03 | 0.01 |
| NF 11 dB | mean | 17.51 | 10.21 | 5.39 | 2.53 | 1.02 | 0.34 | 0.10 | 0.03 |
|  | 5%-tile | 34.46 | 15.06 | 4.18 | 1.07 | 0.33 | 0.10 | 0.03 | 0.01 |

Figure 3.1-1: Throughput loss versus ACIR for 30 GHz, downlink.

From Figure 3.1-1, the impact of NF on the simulation results is negligible, NF 9 dB and 11 dB generates almost the same results. The required ACIR to guarantee a 5% throughput loss are 15.7 dB and 14.6 dB for mean throughput and 5%-tile UE throughput, respectively.

- 3.1.2 Uplink

This section presents the 30 GHz uplink simulation results. In this case, NF 9 dB and NF 11 dB generates exactly the same results.

Table 3.2-1: Throughput loss at given ACIRs for 30 GHz uplink

| ACIR |  | 5 dB | 10 dB | 15 dB | 20 dB | 25 dB | 30 dB | 35 dB | 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 9 dB | mean | 8.61 | 4.20 | 1.81 | 0.69 | 0.24 | 0.08 | 0.03 | 0.01 |
|  | 5%-tile | 31.18 | 13.37 | 3.72 | 0.52 | 0.22 | 0.08 | 0.05 | 0 |
| NF 11 dB | mean | 8.61 | 4.20 | 1.81 | 0.69 | 0.24 | 0.08 | 0.03 | 0.01 |
|  | 5%-tile | 31.18 | 13.37 | 3.72 | 0.52 | 0.22 | 0.08 | 0.05 | 0 |

Figure 3.2-1: Throughput loss versus ACIR for 30 GHz uplink.

From Figure 3.2-1, the required ACIR are 9.1 dB and 14.3 dB for mean throughput and 5%-tile UE throughput, respectively. In this case, NF has no impact on the throughput since it can be cancelled out in the SINR formula.

- 3.2 70 GHz

- 3.2.1 Downlink

Table 3.2-1: Throughput loss at given ACIRs for 70 GHz, downlink

| ACIR |  | 5 dB | 10 dB | 15 dB | 20 dB | 25 dB | 30 dB | 35 dB | 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 13 dB | mean | 8.26 | 4.57 | 2.33 | 1.07 | 0.41 | 0.12 | 0.03 | 0.01 |
|  | 5%-tile | 28.55 | 14.60 | 4.93 | 1.55 | 0.21 | 0 | 0 | 0 |
| NF 15 dB | mean | 8.26 | 4.58 | 2.34 | 1.08 | 0.42 | 0.12 | 0.03 | 0.01 |
|  | 5%-tile | 28.58 | 14.59 | 4.92 | 1.57 | 0.21 | 0 | 0 | 0 |

Figure 3.2-1: Throughput loss versus ACIR for 70 GHz, downlink.

From Figure 3.2-1, the required ACIR are 9.4 dB and 14.9 dB for mean throughput and 5%-tile UE throughput, respectively. The influence of noise figure on the simulation results can be ignored.

- 3.2.2 Uplink

Table 3.2-1: Throughput loss at given ACIRs for 70 GHz, uplink

| ACIR |  | 5 dB | 10 dB | 15 dB | 20 dB | 25 dB | 30 dB | 35 dB | 40 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF 13 dB | mean | 3.52 | 1.69 | 0.72 | 0.28 | 0.10 | 0.03 | 0.01 | 0 |
|  | 5%-tile | 21.39 | 10.31 | 3.58 | 0.76 | 0.06 | 0 | 0 | 0 |
| NF 15 dB | mean | 3.52 | 1.69 | 0.72 | 0.28 | 0.10 | 0.03 | 0.01 | 0 |
|  | 5%-tile | 21.39 | 10.31 | 3.58 | 0.76 | 0.06 | 0 | 0 | 0 |

Figure 3.2-2: Throughput loss versus ACIR for 70 GHz, uplink.

In this case, 5 dB ACIR is enough for mean throughput to be less than 5%, 13.9 dB is required to make the 5%-tile UE throughput loss less than 5%. Noise figure has no impact on the throughput loss results for this case.

- 4. Conclusions

This proposal presents ACIR evaluations for coexistence study in indoor scenario at 30 GHz and 70 GHz carrier frequency.

- 5. References

- R4-168953_r1, “WF on Simulation assumptions of Co-existence study for WP5D”, NTT Docomo.

- R4-168794 [5G NR], “Further Way forward on Dense urban Layout and BS Beamforming Model_v4,” Huawei.

- R4-167123, “WF on UE beamforming model for the ITU WP5D coexistence simulations”, Qualcomm.

- R4-168767 [5G NR] “WF on SINR vs throughput mapping_v1”, Huawei