3GPP TSG-RAN WG4 Meeting NR Ad-hoc                                                                                R4-1700170

Spoken, WA, USA, 17 – 19 Jan, 2016

Agenda Item:	3.8

Source: 	Samsung

Title: 	Text improvement of throughput vs SINR mapping

Document for:	Approval

Introduction

In the last RAN4 #81 meeting, text for throughput vs SINR mapping in TR 38.803 was proposed [1] and agreed. However, there are missing or duplicated points and some text error. This contribution is to improve the text for throughput vs SINR mapping in order to make understanding clear

Discussion

The missing or duplicated points are as follows:

- There are dis-continuities such as SNIRMIN and SNIRMAX

- SNIRMAX is not derived from ThrMAX in the section but provided from Modulation and Coding Scheme (MCS) level based on the some assumptions in the section. Therefore, definition for ThrMax is not necessary.

- SNR and SNIR are used without specific difference, so it is necessary to unify one of them.

Text Proposal

<Start of Text Proposal for TR 38.803>

### 5.2.2 Link level performance for 5G NR coexistence

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR                                         ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

S(SNIR)   Shannon bound, S(SNIR) =log2(1+SNIR)  bps/Hz
			Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the lowest MCS level, dB
	SNIRMAX Maximum SNIR of the highest MCS level, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 5.2.2-1 represent a baseline case, which assumes:

- 1:1 antenna configurations

- AWGN channel model

- Link Adaptation (see table 5.2.2-1 for details of the highest and lowestMCS level)

- No HARQ

Table 5.2.2-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Note that the parameters proposed in table 5.2.2-1 are targeted for eMBB coexistence scenario.

<End of Text Proposal>

Reference

[1] R4-1610570, “TP for TR 38.803 SINR vs throughput mapping”, Huawei