3GPP TSG-RAN WG4 meeting #81-NR-AH  	R4-1700223

# Spokane, Washington, USA, 17-19 January 2017

Source: 	Ericsson

Title:  	BS ACS and blocking for mm-wave frequencies

Agenda Item:	3.1.2.1

Document for:	Approval

# Introduction

The requested ITU-R compatibility parameter list contain both ACS and blocking. We believe that it is crucial to have a complete set of compatibility parameters in the LS response to ITU-R and thus given the limited time, we present here a best effort estimation for BS ACS and blocking levels. We believe that the values presented in this contribution are sufficient for responding to ITU-R, and responding with these values is much preferable to no response.  During the specification work, further discussion on spatial aspects, reference point for defining the ACS and blocking considering the OTA nature of the requirements as well as further consideration over degradation and bandwidths of wanted and interferer signal would be needed to have the detailed requirements settled.

Thus the discussion and proposed values in this paper are in the context of ITU-R related work assuming some simplifications.

To derive parameter levels, two approaches are examined. One is based on examining interferer statistics and the other examining ACIR and ACS levels provided by ongoing co-existence simulation work.

In addition, since 200 MHz is the agreed NR bandwidth for the ITU-R response, both ACS and in-band blocking are expressed with interferer bandwidth of 200 MHz.

As the RAN4#81AH is the last meeting to conclude on ITU-R parameters, a simplified and pragmatic approach could be the only possibility to conclude on the ITU-R compatibility parameters including ACS and blocking.

# Discussion

## Interferer statistics

In this section, interferer level statistics in terms of cdf plots are provided based on the existing RAN4 scenarios and simulation assumptions. For UMA case additional results for 100% grid shift is also presented.

FTP traffic with same load in both Aggressor and Victim Networks (high) was used all through the simulations. The UE output power statistics for each scenario is also included in the corresponding chapter.

### UMA

The UL interfere cdf results for 30 GHz, 45 GHz and 70 GHz are presented below:

### 30 GHz

Figure 1 UL interferer statistics for 0% and 100% grid shift

Figure 2	UE output power statistics for 0% and 100% grid shift

## UMI

### 30 GHz

Figure 3	UL interferer statistics and UE output power cdf

### 45 GHz

Figure 4  UL interferer statistics and UE output power cdf

### 70 GHz

Figure 5	UL interferer statistics and UE output power cdf

## INH

### 30 GHz

Figure 6	UL interferer statistics and UE output power cdf

### 45 GHz

Figure 7	UL interferer statistics and UE output power cdf

### 70 GHz

Figure 9	UL interferer statistics and UE output power cdf

At probability of 99%, the interferer levels for different scenarios and example frequency ranges is summarized in table 1:

Table 1	Interferer levels for 30 GHz

| 30 GHz | UMA 0% grid shift | UMA 100% grid shift | UMI | INH |
| --- | --- | --- | --- | --- |
|  | -90 dBm | -85 dBm | -85 dBm | -87 dBm |

Table 2	Interferer levels for 45 GHz

| 45 GHz | UMI | INH |
| --- | --- | --- |
|  | -85 dBm | -90 dBm |

Table 3	Interferer levels for 70 GHz

| 30 GHz | UMI | INH |
| --- | --- | --- |
|  | -90.5 dBm | -89.5 dBm |

It should be noted that, as discussed in [1], a full analysis of blocking scenarios should include an analysis of the joint probability of the blocker level and wanted signal level. This analysis is not provided in this paper, however even without considering joint probabilities, the derived blocking levels are less stringent than those derived from an ACS analysis. Thus it is proposed that levels from the ACS analysis are considered.

## ACS and blocking

In addition to examining interference statistics, a parallel approach to deriving a blocking requirement is to consider ACS.

The interferer statistics presented in the previous chapters is one element to estimate the ACS and blocking levels. The other element would be the ACIR results from the ongoing simulation work. Table 5 summarizes the needed UL ACIR simulation results presented in [2, 3 and 4] for ~2% degradation.

Table 5	UL Needed ACIR

| Needed ACIR | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | 14 dB | NA | NA |
| UMI | 16 dB | 16 dB | 16 dB |
| INH | 18 dB | 17 dB | 16 dB |

There are different possible approaches to break down the UL ACIR into UE ACLR and BS ACS. As extensively discussed in RAN4, for mm-wave frequencies, similar technology is used for BS and UE and thus similar expected performance. Thus given the assumption above an equal split approach would result in the BS ACS and UE ACLR as described in table 6 and table 7 respectively.

Table 6	BS ACS based on equal split

| BS ACS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | 17 dB | NA | NA |
| UMI | 19 dB | 19 dB | 19 dB |
| INH | 21 dB | 20 dB | 19 dB |

Table 7	UE ACLR based on equal split

| UE ACLR | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | 17 dB | NA | NA |
| UMI | 19 dB | 19 dB | 19 dB |
| INH | 21 dB | 20 dB | 19 dB |

Another approach would be to use asymmetric split where BS contribution by posing more stringent requirement would be less than UE to the ACIR. Using the asymmetric approach, the BS ACS and UE ACLR as described in table 8 and table 9 respectively.

Table 8	BS ACS based on asymmetric split

| BS ACS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | 22 dB | NA | NA |
| UMI | 24 dB | 24 dB | 24 dB |
| INH | 26 dB | 25 dB | 24 dB |

Table 9	UE ACLR based on asymmetric split

| UE ACLR | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | 15 dB | NA | NA |
| UMI | 17 dB | 17 dB | 17 dB |
| INH | 19 dB | 18 dB | 17 dB |

We would propose to use the asymmetric approach which has been the approach traditionally used in RAN4 for the benefit of UE and the need to have low battery consumption. Note that for NR, the UE ACLR is from technology performance point of view has been discussed in [2] and we would expect that NR should have better performance compared to incumbent technologies sharing the mm-wave ranges. In addition, there is a relation between ACLR and achievable EVM which can be further discussed during the WI phase.

The interferer levels for UMA, UMI and INH assuming BS ACS and degradation of 6 dB can be calculated as:

Interferer level = Noise floor + 4.7 dB (correspond to 6 dB degradation) + ACS

Where noise floor over bandwidth of 200 MHz for 30 GHz, and 45 GHz and 70 GHz is -81 dBm, -79 dBm and -77 dBm respectively.

Thus for different scenarios and different example frequencies, table 13 summarizes the ACS interferer level for example frequencies of 30 GHz, 45 GHz and 70 GHz.

Table 10 Interferer level for BS ACS

| BS ACS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | -54 dBm | NA | NA |
| UMI | -52 dBm | -55 dBm | -48 dBm |
| INH | -50 dBm | -56 dBm | -48 dBm |

As the calculated interferer levels for ACS given the asymmetric break-down of ACIR is lower than the interferer level statistics presented in chapter 2.1, the level of degradation can be kept open for further discussions in RAN4.

This presents the best estimated values for mm-wave BS ACS for ITU-R related discussions where having a complete response is crucial including the ACS and blocking.

It should be noted that here the blocking level is the level in the radio. For setting OTA blocking requirements, as described earlier, the spatial aspects for ACS/Blocking, carrier and interferer bandwidths, degradation level, methodology and considerations to drive the ACS and blocking more elaborated in [1] would require further work during the WI phase when specification work is getting started.

As the interferer levels for ACS turned out to be significantly higher than the interferer statics presented here, the same interferer level can be used towards ITU as the blocking level resulting in same interferer level for ACS and blocking.

Similar to in-band blocking levels which would require further work, the out-of-band blocking would also require further investigation and thus meanwhile the proposed in-band blocking value as described in this paper could serve as the value to submit to ITU-R at this stage.

Considering the analysis in this paper and the disclaimers described, the following is proposed as BS ACS and blocking for ITU-R response:

Proposal 1: The BS ACS for ITU-R response should be:

BS ACS based on asymmetric split

| BS ACS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | 22 dB | NA | NA |
| UMI | 24 dB | 24 dB | 24 dB |
| INH | 26 dB | 25 dB | 24 dB |

Which corresponds to interferer levels of:

Interferer level for BS ACS

| BS ACS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | -54 dBm | NA | NA |
| UMI | -52 dBm | -55 dBm | -48 dBm |
| INH | -50 dBm | -56 dBm | -48 dBm |

Proposal 2: The BS blocking levels (in-band and if necessary out-of-band) for ITU-R response for different deployments should be:

Interferer level for BS blocking

| BS ACS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | -54 dBm | NA | NA |
| UMI | -52 dBm | -55 dBm | -48 dBm |
| INH | -50 dBm | -56 dBm | -48 dBm |

Proposal 3: The estimated ACS and blocking parameters are at the radio and are based on simplifications and thus RAN4 need to continue the work on OTA blocking requirements during NR specification phase.

# Conclusion

In this paper, the BS ACS and in-band blocking levels for ITU-R response is further discussed. Both Interferer level statistics as well as different approached to bread-down the needed UL ACIR were thoroughly discussed.

As requested ITU-R compatibility parameter list contain both ACS and blocking. We believe that it is crucial to have a complete set of compatibility parameters in the LS response to ITU-R and thus given the limited time, we present the best estimation for BS ACS and blocking levels. Currently, as the approach for deriving the blocking requirement has not been discussed and decided, the aim of this paper is to make a first level estimate for the ITU response. Thus during the specification work, further discussion on spatial aspects, reference point for defining the ACS and blocking considering the OTA nature of the requirements as well as further consideration over degradation and bandwidths of wanted and interferer signal would be needed to have the detailed requirements settled.

Thus the discussion and proposed values in this paper are in the context of ITU-R related work assuming some simplifications. We believe that even though the simplifications presented in this paper are debatable and will need further discussion, the consequence of not replying to ITU with an ACS and blocking level is worse than the consequences of using this kind of first level estimate-.

Considering the analysis in this paper and the disclaimers described, the following is proposed as BS ACS and blocking for ITU-R response:

Proposal 1: The BS ACS should be:

BS ACS based on asymmetric split

| BS ACS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | 22 dB | NA | NA |
| UMI | 24 dB | 24 dB | 24 dB |
| INH | 26 dB | 25 dB | 24 dB |

Which corresponds to interferer levels of:

Interferer level for BS ACS

| BS ACS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | -54 dBm | NA | NA |
| UMI | -52 dBm | -55 dBm | -48 dBm |
| INH | -50 dBm | -56 dBm | -48 dBm |

Proposal 2: The BS blocking levels (in-band and if necessary out-of-band) for ITU-R response for different deployments should be:

Interferer level for BS blocking

| BS ACS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| UMA | -54 dBm | NA | NA |
| UMI | -52 dBm | -55 dBm | -48 dBm |
| INH | -50 dBm | -56 dBm | -48 dBm |

Proposal 3: The estimated ACS and blocking parameters are at the radio and are based on simplifications and thus RAN4 need to continue the work on OTA blocking requirements during NR specification phase.

# References

[1]   R4-1700215, “Spatial considerations for BS selectivity requirements”, Ericsson

[2]   R4-1610249, “On mm-wave ACLR for 45 and 70 GHz”, Ericsson

[3]   R4-1700086, “Simulations Results for Coexistence Studies in Dense Urban Micro Scenario for NR in mm-wave spectrum”,”, Ericsson

[4]   R4-1700087, “ Simulations Results for Coexistence Studies in Indoor hotspot Scenario for NR in mm-wave spectrum”, Ericsson

[5]   R4-1700085, “ Simulations Results on Coexistence Studies in Urban Macro Scenario for NR in mmwave spectrum”, Ericsson