3GPP TSG-RAN WG4 Meeting NR#2                                                                  R4-1706554

Qingdao, China, 27 - 29 May, 2017

Title:                     Proposal on new BS requirements for NR

Source: 	CMCC

Agenda Item:	3.4.2

Document for:	Approval

1    Introduction

In RAN4#83 meeting, A WF on new BS requirements was approved in R4-1706158 [1]. In order to guarantee the system performance, some declarations related to unwanted spatial emissions should be included in the scope of RAN4 specifications for 5G NR BS in Rel-15.

2    Discussion

According to the WF [1], there are three options for SLSR and FBR requirements.

- Option1: SLSR and FBR should be included as declarations for 5G NR BS in Rel-15.

- Option2: Side lobe level and front to back ratio potential requirements can be combined into a “emissions spatial mask” declaration

- Option3: Declared The total power that is radiated outside of the 3dB beamwidth within the context of the NR   declarations (assuming similar declarations to AAS with this addition)

In this contribution, we provide our proposal on unwanted spatial emissions BS requirements for NR.

- Option 1: Declaration for SLSR and FBR

We have presented contributions to discuss the necessity of SLSR and FBR, and how to define the new requirements for 5G NR BS [2] [3] [4] [5] [6]. Considering SLSR and FBR are associated with the kind of base station and antenna array, for each base station type defines a minimum requirement may be challenging in RAN4. SLSR and FBR should be included as declarations for 5G NR BS in Rel-15.

The declaration for option 1 could look like the following (exact wording to be clarified later):

| Unwante lobe ratio | The ratio of the beam peak EIRP to the largest EIRP that is radiated outside of the 3dB beamwidth. Declared for every beam identified in D9.3. |
| --- | --- |

- Option 2: Declaration for “Emissions spatial mask”

As stated in [7], a requirement which controls the radiation outside the design range of operation is a possibility. This requirement could be renamed as unwanted spatial emissions mask. Side lobe level and front to back ratio potential requirements can be combined into unwanted spatial emissions mask declaration.

The declaration for option 2 could look like the following (exact wording to be clarified later):

| Unwanted directions set | The set of directions where the AAS BS is intended not to radiated power, declared per EIRP accuracy directions set (D9.8) |
| --- | --- |
| Unwanted power level | Maximum power level radiated in the unwanted directions set |

- Option 3: Declaration for total radiated power outside of the 3dB beam width

As stated in [8], for some beam forming algorithms, spatial emissions might be traded against in beam wanted EIRP to optimize performance. The option3 is that declared the total power that is radiated outside of the 3dB beam width within the context of the NR declarations (assuming similar declarations to AAS with this addition).

The declaration for option 3 could look like the following (exact wording to be clarified later):

| Out of beam power | The total power that is radiated outside of the 3dB beamwidth. Declared for every beam identified in D9.3. |
| --- | --- |

We have already discussed the NR BS new requirements for many meetings, and we need to make some progress in this meeting. In order to carry out new BS requirements defining or declaration method, one of options related to unwanted spatial emissions should be selected for 5G NR BS in Rel-15, and RAN4 should start the discussion of the detail of declaration from NR#2.

Proposal 1: One of options related to unwanted spatial emissions should be selected for 5G NR BS in Rel-15, and RAN4 should start the discussion of the detail of declaration from NR#2.

3   Conclusions

In this paper, we summarize three options for SLSR and FBR, and put forward the defining way of new BS requirements for 5G NR.

Proposal 1: One of options related to unwanted spatial emissions should be selected for 5G NR BS in Rel-15, and RAN4 should start the discussion of the detail of declaration from NR#2.

- References

R4-1706158, WF on new BS requirements, NTT DOCOMO, CMCC

R4-167437, Consideration on metric for 5G NR BS output power, CMCC

R4-1609726, Discussion on side lobe suppression ratio defining for 5G NR, CMCC

R4-1700161, Discussion on NR BS specific new OTA requirements, CMCC

R4-1703338, Discussion on the necessity of SLSR and FBR, CMCC

R4-1705236, Proposal on BS specific requirements for NR, CMCC

R4-1705635, NR Range 2 - Beam specific requirememts, Huawei

R4-1704953, General discussion on average radiation pattern, Ericsson