3GPP TSG-RAN WG4 NR AH Meeting#2	R4-1706567

Qingdao, China, 27 - 29 June, 2017

Agenda Item:	3.4.4.2

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Proposal on below 6GHz NR BS Receiver In-band Blocking

Document for:	Approval

# 1.	Introduction

Below 6GHz NR BS receiver in-band blocking requirements have been discussed in RAN4, and it is for further study if it is possible to reuse the E-UTRA BS receiver in-band blocking requirements for the conducted requirements at antenna connector of NR BS below 6GHz [1].

This contribution provides a proposal to specify the below 6GHz NR BS receiver in-band blocking conducted requirements in the RAN4 specifications.

# 2.	Discussion

To investigate the received blocking signal power for below 6GHz NR BS from an adjacent E-UTRA system, UL simulation runs have been performed for 100MHz NR victim and 10MHz E-UTRA aggressor in urban macro scenarios (which is the most demanding case for the BS in-band blocking requirements as the UE transmit power will be the highest to compensate for the highest path loss) with 750m ISD (i.e. 500m cell range) at 3.5 GHz carrier frequency, and the simulation results are provided in Figures 1 to 6 below. The simulation assumptions and parameters used here are the same as those RAN4 used for NR coexistence simulation for WP5D reply, as recorded in TR 38.803 [1], except the following:

-	Besides coordinated site deployment (0% grid shift), un-coordinated site deployment (100% grid shift) between victim and interfering systems (which RAN4 has long been using in below 6GHz coexistence simulation).

-	ACLR and ACS models for asymmetric victim and aggressor bandwidths, as explained in TR 36.942 [2].

-	8x16 antenna array (with 3dB polarization gain) is only used in the NR BS, while passive directional antenna is used in the E-UTRA BS.

-	Omni-directional antenna is used in both NR and E-UTRA UE.

-	LOS path loss formula is slightly modified per the updated ones in TR 38.901 [3].

-	E-UTRA interfering UE with uplink power control parameters of gamma=1 and CLxile=112.

Figure 1: CDF of OTA received blocking signal power of victim BS (UMa 0% grid shift)

Figure 2: CDF of received blocking signal power with antenna element gain of victim BS (UMa 0% grid shift)

Figure 3: CDF of received blocking signal power with antenna element and array gain of victim BS (UMa 0% grid shift)

Figure 4: CDF of OTA received blocking signal power of victim BS (UMa 100% grid shift)

Figure 5: CDF of received blocking signal power with antenna element gain of victim BS (UMa 100% grid shift)

Figure 6: CDF of received blocking signal power with antenna element and array gain of victim BS (UMa 100% grid shift)

Comparing the simulation results in Figures 1 to 6 to those in [4], it can be seen the received blocking signal power level at the NR BS antenna connector would be higher with the E-UTRA interfering system without UE specific beamforming under the same deployment scenario. Hence coexistence with E-UTRA interfering system in the same geographical area would be the more demanding factor for the NR BS receiver in-band blocking conducted requirements comparing to the coexistence with NR system in the same geographical area. As such adopting the interfering signal power level for each E-UTRA BS class to that for the corresponding NR BS class (e.g. -43dBm for Wide Area BS) for the receiver in-band blocking conducted requirements would ensure sufficient protection of the NR BS receiver against E-UTRA or NR in-band uplink signal. Therefore, it is proposed to specify the below 6GHz NR BS receiver in-band blocking conducted requirement for each NR BS class with an interfering signal power equal to the corresponding E-UTRA BS class.

For the wanted signal power level, a 6dB desensitization (compared to the BS reference sensitivity) is allowed for both UTRA and E-UTRA BS in-band blocking requirements. It is proposed to adopt the 6dB desensitization (compared to the BS reference sensitivity) also for below 6GHz NR BS receiver in-band blocking requirement. The SNR for the BS reference sensitivity can be obtained at 95% relative throughput from link level simulations.

Moreover, the interfering signal should be defined as the same type as the wanted signal, with carrier frequency offset of two times the channel bandwidth of the wanted signal.

Besides, it is proposed to consider any extension on the lower and upper boundaries that would be agreed for the NR BS spectrum emission mask also for the in-band blocking requirement, as the front-end filter performance should be similar for the transmitter and receiver sides in below 6GHz.

# 3.	Conclusion and proposals

This contribution has provided simulation results and a proposal to specify the below 6GHz NR BS receiver in-band blocking requirement in the RAN4 specifications per the agreed way forward.

Proposal: To specify the below 6GHz NR BS receiver in-band blocking conducted requirement for each NR BS class with an interfering signal power equal to that for the corresponding E-UTRA BS class, and the wanted signal level calculated as the BS reference sensitivity plus 6dB. The SNR for the BS reference sensitivity can be obtained at 95% relative throughput from link level simulations. Moreover, the interfering signal should be defined as the same type as the wanted signal, with carrier frequency offset of two times the channel bandwidth of the wanted signal. Besides, any extension on the lower and upper boundaries that would be agreed for the NR BS spectrum emission mask should also be considered for the in-band blocking requirement.

# References

[1]	3GPP TR 38.803 v14.0.0, “Study on New Radio Access Technology; RF and co-existence aspects”.

[2]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[3]	3GPP TR 38.901 v14.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”.

[4]	R4-1706568, “Proposal on mmWave NR BS Receiver In-band Blocking”, Nokia, Alcatel-Lucent Shanghai Bell.