3GPP TSG-RAN WG4 Meeting NR#2                                   R4-1706650

Qingdao, China, 27th – 29th June, 2017

Agenda item:			3.5.8.2

Source:			Samsung

Title:	Discussion on the Feasibility of DC-based Mobility Enhancement

Document for:			Discussion

# 1	Introduction

In RAN2 #97bis meeting, an LS on the feasibility of DC-based mobility enhancement in NR was sent to RAN1 and RAN4 [1], and the following questions are provided on the technical issues for intra-frequency DC:

1. Overall Description: RAN2 is considering mobility enhancement based on DC framework to achieve 0ms interruption time. The target cell will be added as a PSCell which will be configured as PCell to complete the handover. This could be intra-frequency DC applicable for all supported frequency bands for NR for both synchronous and asynchronous cases (i.e. same as LTE dual connectivity). Therefore, RAN2 would like to ask RAN1 and RAN4 on the feasibility of simultaneous transmission and reception from two cells in NR. Q1: Is it feasible that the UE performs simultaneous reception from two intra-frequency cells in either synchronous or asynchronous network with single or dual RF chains? Q2: Is it feasible that the UE performs simultaneous transmission to two intra-frequency cells in either synchronous or asynchronous network with single or dual RF chains? Q3: Do the answers to Q1-Q2 change depending on the frequency bands? 2. Actions: To RAN1 and RAN4 groups. ACTION: RAN2 respectfully asks RAN1 and RAN4 groups to provide answers to the questions listed above.

In Hangzhou meeting, the above three questions are discussed but limited agreement achieved. In this paper, we will provide our views on this topic.

# Discussion

Based on the three questions from RAN2, the technical discussion is provided as below respectively.

Q1: Is it feasible that the UE performs simultaneous reception from two intra-frequency cells in either synchronous or asynchronous network with single or dual RF chains?

Firstly, with single RF chain in mmWave band, the impact of RX beamforming shall not be overlooked: i.e., in the cell edge region to request the handover, RX beamforming towards different directions could be inevitable in DC-based mobility between non-collocated gNBs, but aiming at two diverse directions is not obtainable especially considering digital + analogy beamforming is assumed by companies in the RF architecture discussion. Even with the potential advanced TDM/FDM solutions into account, the above-mentioned issue (two RX beams towards two different directions) is still hard to be addressed: (1) For FDM scheme, it requires two beams in two sub-bands but it is hard to be supported by current mmWave architecture agreement; (2) For TDM scheme, the beam switching time is inevitable which will be contradictory to the intention to introduce this DC-based mobility, i.e., 0ms interruption time.

With dual RF chains in mmWave band, two RX beamforming directions can be achieved for both synchronous and asynchronous network. However, we should further exam this assumption with RAN4’s agreement on mmWave UE architecture [2][3]. Based on our understanding, the “RX chain” concept is more comparable to the concept of “antenna arrangement” or “single polarization in one antenna arrangement” in [2]. If two antenna arrangements (assuming each include two polarizations) are utilized for two directions, the original Demod performance can be maintained, while if two polarizations in one antenna arrangement are utilized for two directions, the diversity gain will lost thus resulting degraded performance for both directions. It should be noted that, considering the direction matching between UE antenna arrangements and directions from source/target gNB, both above-mentioned situations could happen.

For below-6GHz, most technical difficulties have been well discussed for single RF chain, which will limit the implementation scenario into synchronous network with the limited power difference between target and source gNB. However, for dual RF chain in below 6GHz band, it is feasible to have simultaneous reception as we discussed in LTE eMobility discussion.

Observation 1: In mmWave band, UE performs simultaneous reception from two intra-frequency cells is not possible with one RF chain; two RF chain may enable the simultaneous reception but degraded performance may be expected.

Q2: Is it feasible that the UE performs simultaneous transmission to two intra-frequency cells in either synchronous or asynchronous network with single or dual RF chains?

Similar to the above analysis in RX part, in mmWave band, Tx beamforming simultaneously towards source and target cells will be required, especially considering in the handover region (more likely to be UL-limited area) reasonable Tx beamforming gains respectively toward source and target cells will be essential to maintain the wireless links. Similarly, the feasibility answer for single RF chain will still be negative. Similarly, advanced TDM/FDM scheme still cannot help for this Tx scenario.

For dual Tx RF chains utilized for mmWave band, the simultaneous intra-frequency transmissions toward two directions could be possible. However, considering the power sharing mechanism which will be decided by RAN1, the allocated power could be reduced thus limiting the cell edge performance to both directions.

For below-6GHz, similar to Rx discussion, we don’t observed any technical difficulty for two RF chain implementation. But for single RF chain implementation, the difficulty of Tx timing, Tx power imbalance between two transmissions will be expected.

Observation 2: In mmWave band, UE performs simultaneous transmission toward two intra-frequency cells is not possible with one RF chain; two RF chain may enable the simultaneous reception but degraded performance will be expected due to power allocation between two transmissions.

Q3: Do the answers to Q1-Q2 change depending on the frequency bands?

Considering the beamforming related issues are also band/frequency range related, the answer to Q3 could not be that straightforward, i.e, we should separate the discussion for mmWave and below 6GHz in the reply LS to RAN2.

Observation 3: Considering the beamforming related issues are also band/frequency range related, the answer to Q1 and Q2 will be band/frequency range dependent.

# Conclusion

In this paper, we provide our observations on RAN2’s questions on the feasibility of DC-based mobility enhancement in NR, i.e.,

Observation 1: In mmWave band, UE performs simultaneous reception from two intra-frequency cells is not possible with one RF chain; two RF chain may enable the simultaneous reception but degraded performance may be expected.

Observation 2: In mmWave band, UE performs simultaneous transmission toward two intra-frequency cells is not possible with one RF chain; two RF chain may enable the simultaneous reception but degraded performance will be expected due to power allocation between two transmissions.

Observation 3: Considering the beamforming related issues are also band/frequency range related, the answer to Q1 and Q2 will be band/frequency range dependent.

# Reference

[1] R2-1703971, “LS on the feasibility of DC-related mobility enhancements in NR”, Intel.

[2] R4-1704366, “WF on mmW antenna ref architecture”, Qualcomm, Sony and Skyworks.

[3] R4-1706067, “WF on UE Reference Architecture”, Sony, Skyworks Solutions Inc. and LG Electronics.

.