3GPP TSG-RAN WG4 Meeting NR Ad-hoc #2	 R4-1706656

Qingdao, China, 27-29 June, 2017

Agenda Item:	3.2.3

Source: 	Samsung

Title: 	Proposed spectrum utilization for NR

Document for:	Discussion

Introduction

For feasible spectrum utilization, following the agreements in last RAN4 meeting, generic spectrum utilization will be defined in Rel-15 NR time frame:

- Companies are encouraged to propose the spectral utilization values in terms of number of RBs - For below 6GHz, channel BW (5, 10, 15, 20, 40, 50, 60, 80, 100MHz) and SCS (15, 30, 60kHz) - For above 6GHz, channel BW (50, 100, 150, 200, 400MHz) and SCS (60, 120kHz) - Evaluations must be performed with agreed requirements - R4-1706073, mmW SEM - R4-1706063 WF on mmWave ACLR and ACS - R4-1706316 WF on sub 6GHz ACLR and ACS - Also relevant receiver blocking requirements must be considered (impact on both filtering and reciprocal phase noise mixing) - Companies are encouraged to provide info on spectral efficiency - Companies should provide relevant information used in the evaluation including EVM and Sub-6 SEM - Note: The BWs here are examples; a subset may be selected for the final set of BWs.

In companion paper on how to define transmission bandwidth configuration [1], we show the template on how to define NRB for in specification, and our preferred Channel Bandwidth sets under each SCS.

In this contribution, we give our preference values on achievable transmission spectrum utilization under different combinations of channel bandwidths and SCS (data).

Discussion

The primary assumptions are summarized here. For sub 6GHz, legacy LTE requirements for ACLR, ACS, emission mask, EVM were reused. For above 24GHz, related mm Wave agreements included SEM, ACLR and ACS were used.

Based on the evaluation and the feasibility consideration of transmitter implementation, initial proposals for achievable spectrum utilization under different combinations of SCS and channel bandwidths were given in below tables.

Note1: Below tables give achievable SU for all the possible combinations of SCS and CHBWs, final set of channel Bandwidths can be further down-selected, and we share our preferred CHBW sets in [2].

Note2: The proposed spectrum utilization in below tables was applicable for both downlink and uplink.

Note3: Below tables gives achievable SU based on the assumption of symmetrical guard-band on the upper-edge and lower-edge of channel bandwidths, if such assumption cannot be guaranteed as analysed in [3], proposed values maybe need to be revised.

Note4: The proposed spectrum utilization was target for single CC operation, regarding wideband operation with CA approach in UE side, it’s FFS as analysed in [4].

Proposed Spectrum utilization of sub 6GHz

Table 1: spectrum utilization for SCS=15 kHz (data)

| SCS (kHz) (Data) | 15 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW (MHz) | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
| FFT size | 512 | 1024 | 2048 | 2048 | 40296 | 4096 | Not Supported |  |  |
| # PRBs | 26 | 52 | NA | 106 | 214 | 268 |  |  |  |
| Spectrum utilization | 93.6% | 93.6% | NA | 95.4% | 96.3% | 96.48% |  |  |  |

Table 2: spectrum utilization for SCS= 30 KHz

| SCS (kHz) (Data) | 30 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW (MHz) | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
| FFT size | 256 | 512 | 1024 | 1024 | 2048 | 2048 | 4096 | 4096 | 4096 |
| # PRBs | TBD | 26 | NA | 52 | 106 | 134 | 162 | 216 | 272 |
| Spectrum utilization |  | 93.6% | NA | 93.6% | 95.4% | 96.48% | 97.2% | 97.2% | 97.92% |

Table 3: spectrum utilization for SCS= 60 KHz

| SCS (kHz) (Data) | 60 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW (MHz) | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
| FFT size | Not supported | 256 | 512 | 512 | 1024 | 1024 | 2048 | 2048 | 2048 |
| # PRBs |  | TBD | NA | 26 | 52 | 66 | 80 | 108 | 136 |
| Spectrum utilization |  |  | NA | 93.6% | 93.6% | 95.04% | 96.0% | 97.2% | 97.92% |

Proposed spectrum utilization for above 24GHz

Table 4: spectrum utilization for SCS=60 KHz

| SCS (kHz) (Data) | 60 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW (MHz) | 50 | 100 | 150 | 200 | 400 |
| FFT size | 1024 | 2048 | 4096 | 4096 | Not Supported |
| # PRBs | 66 | 132 | 198 | 264 |  |
| Spectrum utilization | 95.04% | 95.04% | 95.04% | 95.04% |  |

Table 5: spectrum utilization for SCS=120 KHz

| SCS (kHz) (Data) | 120 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW (MHz) | 50 | 100 | 150 | 200 | 400 |
| FFT size | 512 | 1024 | 2048 | 2048 | 4096 |
| # PRBs | 32 | 66 | 98 | 132 | 264 |
| Spectrum utilization | 92.16% | 95.04% | 94.08% | 95.04% | 95.04% |

Conclusion

In this contribution we share the preference on the achievable spectrum utilization under different combinations of CHBW and SCS (data). And it is suggested to be taken into account in corresponding requirement definition.

Reference

[1] R4-1706060, “Way Forward on Spectral Utilization”, Huawei, Qualcomm

[2] R4-1706657, “Discussion on CHBW sets”, Samsung

[3] R4-1706647, “Discussion on channel raster and SS raster”, Samsung

[4] R4-1706646, “Discussion on wideband operation”, Samsung, Samsung