3GPP TSG-RAN WG4 Meeting NR Ad-hoc #2	 R4-1706658

Qingdao, China, 27-29 June, 2017

Agenda Item:	3.2.1

Source: 	Samsung

Title: 	SCS for data and PBCH for mm Wave range

Document for:	Discussion

Introduction

For SCS of data, such agreements were reached in RAN4#82bis

Agreements in RAN4#82bis Table 1: RAN4 agreement on sub-carrier spacing on data SCS for Below 1GHz; SCS for 1GHz~6GHz; SCS for 24GHz-52.6GHz. data; 15KHz, 30KHz; 15kHz,30kHz, 60kHz; 60 kHz, 120kHz.

For SCS of SS/PBCH, such agreements reached in RAN1#88bis:

RAN1 agreements for PBCH and SCS for SS in RAN1#88bis: - Subcarrier spacings for PSS/SSS for difference freq. ranges: 15kHz/30kHz for below 6 GHz, and 120kHz/240kHz for above 6 GHz - Note: RAN1 assumes that RAN4 will decide it depending on frequency ranges - PSS/SSS sequence length: 127 sub carriers (12PRB) - PBCH BW: 288 subcarriers (24PRB)

In this contribution, we provide our views on related issues.

Discussion

SCS for data/control:

For sub 6GHz, considering different services purpose (eMBB, URLLC), multiple SCS may need to be supported for single band. At least from NW perspective, mixed numerology of data either FDM or TDM may be unavoidable.

For above 24GHz, considering specs simplicity, work load and system optimization, it’s better to down-select SCS for data as per-band basis in Rel-15 timeframe.

P1: SCS of data/control channel for above 24GHz should be per band/per frequency range specific, each band/frequency range should support a single value for SCS of data/control.

SCS for SS and PBCH

In last RAN1 meeting, such agreements were reached in RAN1:

Further RAN1 agreements for mini CHBW and SS SCS (LS R1-1709842): RAN1 would like to inform RAN4 that because of the inability of the 50MHz minimum carrier bandwidth to support SS Blocks with 240kHz SCS, RAN1 agreed that a minimum carrier bandwidth value of 100MHz should be supported in addition to 50MHz in order to accommodate SS Blocks with 240kHz SCS for bands, where appropriate in the frequency range from 24 GHz to 52.6 GHz. RAN1 would also like to inform RAN4 that because of the inability of the 5MHz minimum carrier bandwidth to support SS Blocks with 30kHz SCS, RAN1 agreed that a minimum carrier bandwidth value of 10MHz should be supported in addition to 5MHz in order to accommodate SS Blocks with 30kHz SCS for bands, where appropriate in the frequency range below 6 GHz.

Following RAN1 agreements, such interpretation can be observed:

RAN1 has already acknowledged the inability of supporting 30 kHz SS with 5MHz CHBW and 240 kHz with 50MHz CHBW with existing SS/PBCH BW design.

RAN1 has not revisit the PBCH design, on the contrary RAN1 ask RAN4 to consider the restriction of 30kHz SS and 240kHz SS applicable for bands:

- 30kHz SS only applicable for bands below 6GHz which minimum CHBW larger than or equal to 10MHz

- 240kHz SS only applicable for bands under mmWave range which minimum CHBW larger than or equal to 100MHz

Furthermore, following RAN1 agreements, SCS for SS should be per band or per frequency ranges specific. From RAN4 perspective, SCS of SS should be fixed at least for each band; otherwise UE needs to perform blind detection for SCS of SS for cell detection which increase UE complexity heavily and has extensive impact for RRM procedure as analysed by companies’ contribution.

For down-selection SCS for SS, both NW deployment flexibility, specification simplicity and system optimization need be considered.

Sub 6GHz SCS of SS

For sub 6GHz, 15kHz and 30kHz SCS of SS were potential options. 30kHz SCS of SS has more robust performance against phase noise then 15 kHz SS. Considering phase noise issue more critical under high frequency and larger CHBW case i.e. 50MHz and above CHBW, it’s preferred to select SCS of SS as 30kHz for CHBW larger than 50 MHz CHBW. For channel BW 5MHz, only option is 15 kHz of SS since PBCH bandwidth of 30 kHz conflict with 5MHz CHBW. Meanwhile, in order to down-selected SCS options in system operation, it’s better SCS of SS selected from the candidate sets of SCS of data under certain CHBWs.

P2: SCS of 30kHz is used for sub-6GHz bands that support maximum CHBWs larger than 50 MHz and minimum CHBW larger than or equal to 10MHz.

For bands which support 5MHz CHBW, two alternative ways can be considered:

- Alt1: Use a single SS SCS of 15 kHz if all the CHBWs defined for the band reside in 5MHz ~ 50MHz.

  - This has some impact on spectrum allocation, CA approach required for aggregated larger than 50MHz contiguous spectrum transmission

- Alt2: If band want to support channel bandwidths larger than 50MHz CHBW, then need to revise minimum CHBW as 10MHz above, then corresponding SCS of SS as 30kHz

  - For each band, channel bandwidth 5MHz and above 50MHz CHBW cannot be supported simultaneously either restrict with 5MHz ~50MHz CHBW in the set with 15kHz SS or alternatively taking at least 10MHz as minimum CHBW with 30kHz SS.

SCS of SS under mm Wave range

For mm Wave range above 24GHz, below options can be considered:

- Alt1: With full flexibility, down-selection from 120 kHz and 240 kHz per band specific

  - For bands support minimum CHBW as 50MHz, only option is 120kHz SS and for bands support minimum CHBW as 100MHz, SCS can be down-selected from 120kHz and 240 kHz

- Alt2: Choosing only one value from 120kHz and 240kHz for above 24GHz, considering the restriction of minimum CHBW, 120kHz SCS is only option to cover all bands

Backward compatibility issue for minimum CHBW and SCS of SS

If we determine SCS of SS per band basis, there may have some potential risks for backward compatibility if in later release we include small CHBW into this band.

Considering FCC spectrum allocation reality, 50MHz was the minimum spectrum block. This conflicts with current RAN1 assumption. Even we fixed minimum CHBW as 100MHz in current release, we cannot exclude the possibility some operators get 50 non-contiguous spectrum in future release, then we may have backward compatibility issue.

Taking 28GHz band as an example, in Rel-15, 100MHz was decided as minimum CHBW for 28GHz band based on operators’ feedback, and 240 SCS for SS selected. Then in later release, other operators may get 50MHz block spectrum on the same band, operators will require add 50MHz CHBW into this band. And this CHBW conflicts with 240 kHz SCS, which means 120kHz SCS of SS need to be supported for this band. And this will bring trouble for legacy UE and blind detection also needed for SCS.

In order to resolve this backward compatibility issue for minimum CHBW in mm Wave range, several options can be consider, one simple option is fixed SCS of SS as 120 kHz. However this will restrict NW deployment flexibility and 240 kHz is more robust against phase noise which is critical in mm Wave range considering for cell detection lack of phase noise tracking and compensation procedure. Another option, using 240 kHz for bands with minimum CHBW 100MHz and not allowed introduced 50MHz CHBW in later release for that band. This may be conflict with spectrum allocation reality especially in FCC. If operators want to have 50MHz CHBW and 240kHz in the same band, then further solutions were required in RAN1 i.e. revised existing PBCH design or introducing mechanism to facilitate CC with 50MHz CHBW without SS block transmission on that CC.

Conclusion

In this contribution, we provide initial analysis on SCS for data/control channel and SCS for SS/PBCH channel.

SCS of Data

P1: SCS of data/control channel for above 24GHz should be per band/per frequency ranges specific, each band/frequency range should support a single value for SCS of data/control.

SCS of SS for sub 6GHz

P2: SCS of 30 kHz is used for sub-6GHz bands that support maximum CHBWs larger than 50 MHz and minimum CHBW larger than or equal to 10MHz.

P3: For sub-6GHz bands that support minimum CHBW 5MHz, the following options can be considered:

-        Alt1: Use a single SS SCS of 15 kHz if all the CHBWs defined for the band reside in 5MHz ~ 50MHz.

-        Alt2: Use a single SS SCS of 30 kHz with defining the min CHBW as 10MHz or above and maximum CHBW larger than 50MHz.

SCS of SS for mm Wave range

P4: For mm Wave range, backward compatibility issue for minimum CHBW and SCS of SS need to be carefully considered.

- Alt1: With full flexibility, down-selection from 120 kHz and 240 kHz per band specific

- Alt2: Choosing only one value from 120 kHz and 240kHz for above 24GHz

Reference

[1] R1-1706717, LS on NR Initial access