3GPP TSG-WG RAN4 Meeting NR#2	R4-1706689

# Qingdao, China,  Date 27th – 29th June, 2017

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 					NR band numbering

Agenda Item: 			3.1.1

Document for:	Approval

# Introduction

NR band numbering has been discussed in previous RAN4 meeting and a WF was agreed in RAN4#83[1].

Based on the discussion, it is agreed to continue studying the following aspects of band numbering:

- For NR in LTE “refarming” bands, whether to re-use the LTE band numbers for NR.

- For new NR bands, consider different alternative options:

  - To put new NR bands above 6 GHz in a new separate numbering range within existing signaling capability.

  - To group new NR bands below 6 GHz with the LTE bands, resulting in separate band numbering ranges for below 6 GHz and above 6 GHz bands.

  - To assign unused numbers to all new bands on a “first come first served” basis, regardless of frequency range, duplex mode or RAT (LTE, NR, etc.)

- How to consider “Duplex mode” for different types of bands

# Discussion

## 2.1	n-prefix

It is essential that NR and LTE bands can be distinguished from each other easily and we propose to do this by assigning a n-prefix to NR band numbers. This is somewhat similar approach that was taken when LTE band number notation was chosen to be Arabic-numbering where as WCDMA band number notation was Roman-numbering. In all cases of cource only a number is signalled and the radio acces system is obvious anyways. Use of Arabic or Roman numbering or n-prefix is for specification and other documentation purposes which is necessary to avoid confusion about the radio acces system. Furthermore when thining further NR+LTE CA it is also essential that LTE and NR band numbering can be separated if LTE like CA acronym approach is adopted for LTE+NR CA. Hence in order to easily distinguish between LTE and NR bands we propose

Proposal 1: NR bands are distinguished from LTE bands by placing a n-prefix before band number. (n1,n2…)

## LTE and NR band number spaces

First we list some key principles on how new LTE and NR bands numbers are selected and Table 1 presents LTE, NR S6 and NR mmW band numbering spaces. Finally Table 2 presents some examples on setting the bands numbers.

Principles:

- When ever LTE and NR S6 have a band that has same frequency allocation then the band number is same

- NR bands are distinguished from LTE with n-prefix

- LTE and NR S6 has same numbering space 1 … 256 (current LTE signalling capability)

- If existing LTE band is re-farmed to NR same number is used with n-prefix. For example LTE band 28 would become NR bands n28

- New NR S6 or LTE FDD band number is first available from 65…256 space, note that 77 is curently first available

- New LTE TDD band number is first available from 33 … 64 space, note that 52 is curently first available. When all are used next available is taken from 65 … 256 space just like LTE FDD and NR S6 bands.

- If new band is standardized same time for LTE TDD and NR S6 band number is first available from 65…256 space

Table 1: LTE and NR band numbering spaces

|  |  | LTE |  | NR s6 |  | NR mmW |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Band # | Duplex | Band # | Duplex | Band # | Duplex |
| Legacy LTE FDD space |  | 1 | FDD | n1 | paired spectrum | N/A |  |
|  |  | 32 |  | n32 |  |  |  |
| Legacy LTE TDD space |  | 33 | TDD | n33 | unpaired spectrum |  |  |
|  |  | 64 |  | n64 |  |  |  |
| LTE/NR S6 space |  | 65 | all duplex modes | n65 | all duplex modes |  |  |
|  |  | 256 |  | n256 |  |  |  |
| NR mmW space |  | N/A |  |  |  | n256--> | all duplex modes |

Some examples are presented in Table 2

Table 2: LTE and NR S6 band numbering examples

| LTE | NR S6 | Comments |
| --- | --- | --- |
| 1 | n1 | LTE FDD band 1 is re-farmed for NR |
| 2 | - | LTE only |
| 3 | - | LTE only |
| 4 | - | LTE only |
| 5 | n5 | LTE FDD band 5 is re-farmed for NR |
| … |  |  |
| 36 | - | LTE only |
| 37 | - | LTE only |
| 38 | n38 | LTE TDD band 38 is re-farmed for NR |
| … |  |  |
| 52 | - | new LTE only TDD band. Can be re-farmed later for NR. |
| … |  |  |
| 65 | - | LTE only |
| 66 | n66 | LTE FDD band 66 is re-farmed for NR |
| … |  |  |
| 76 | - | new LTE only SDL band. Can be re-farmed later for NR. |
| - | n77 | new NR only band. Can be specified for LTE later. |
| - | n78 | new NR only band. Can be specified for LTE later. |
| - | n79 | new NR only band. Can be specified for LTE later. |
| - | N80 | new NR only band. Can be specified for LTE later. |
| 81 | N81 | new TDD band standardized for both LTE and NR in same WID. |

Proposal 2: Define NR and LTE band numbering spaces as in Table 1 and follow the principles presented in this paper.

# 3	Conclusion

In this contribution we have made following proposal

Proposal 1: NR bands are distinguished from LTE bands by placing a n-prefix before band number. (n1,n2…)

Furthermore we have also made proposal on how to define LTE and NR numbering spaces

Proposal 2: Define NR and LTE band numbering spaces as in Table 1 and follow the principles presented in this paper.

# 4	References

[1] R4-1706050, WF on NR band numbering, Ericsson, RAN4#83