3GPP TSG-RAN WG4 Meeting NR #2															R4-1706691

Qingdao, China, 27 - 29 June, 2017

Source: 	Nokia, Alcatel-Lucent Shanghai Bell, Skyworks Solutions, Inc.

Title: 					Simulation and measurement results for NR Sub-6 GHz CP-OFDM and DFT-s-OFDM waveforms

Agenda Item: 			3.3.3.1

Document for:	Discussion

# 1	Introduction

In this contribution we present simulated and measured MPR results for CP-OFDM and DFT-s-OFDM based waveforms. UE assumptions are as propsed in [4] and listed in this paper. Emission requirements SEM, ACLR, EVM and In-band emissions were as proposed in [5][6][7][8] respectively and also listed in this paper

# 2	Discussion

## 2.1	Evaluation assumptions

### 2.1.1	Tx chain assumtions

PA calibration point was 0 dB with 100 RB QPSK DFT-s-OFDM (15KHz SCS) signal as proposed in [4]

Transmittter assumptions are based on LTE 256-QAM UE assumptions [1] and proposal in [4] and presented in Table 1.

Table 1: NR transmitter assumptions for NR MPR simulations

| Tx EVM contributor | EVM | SNR(dB) |
| --- | --- | --- |
| PA | 1.85% | 34.7 |
| Transmitter | 1.19% | 38.5 |
| Phase noise | 1.78% | 35 |
| IQ imbalance | 2.06% | 33.7 |
| Total | 3.5% | 29.1 |

LO leakage used in the simulations was 25 dBc although we propose in [4] to use 28 dBc for NR.

### 2.1.3	Emission requirements

LTE SEM for 20 MHz bandwidth and NR SEM for 100 MHz as proposed in [5] and presented in Table 2

Table 2: SEM

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0-1 | -15 | -18 | -20 | -21 | -22 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 85-100 |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  | -25 |  |

NR ACLR as agreed in [3]

List..

LTE EVM as proposed in [7] and presented in Table 3

Table 3: EVM

| Modulation | EVM |
| --- | --- |
| QPSK | 17.5% |
| 16QAM | 12.5% |
| 64QAM | 8% |
| 256QAM | 3.5% |

Standard LTE Inband emissions as proposed in [8]

## 2.2	Simulation results

### 2.2.1	CP-OFDM

Signal parameters are as follows

- Spectrum utilization of 106 RB for 20 MHz signal

- WOLA with slope length of Ncp/8

QPSK

- Small allocations (up to 3RB at edge and up to 8RB in the middle) do not need MPR.

- Close to edge for RBsize 4 – 15 RB  SEM is limiting and when RBsize > 8 NRaclr is limiting factor RB

- Typically for RBsize < 80 allocations not at the edge of the channel, in-band emissions are limiting.

16-QAM

- Some MPR is always needed 1-3 dB

- Close to edge for RBsize 3 – 10 RB  SEM is limiting and when RBsize > 8 NRaclr is limiting factor RB

- Typically for RBsize < 70 allocations not at the edge of the channel, in-band emissions are limiting.

64-QAM

- 3 dB MPR always needed

- Typically in-band emission limited but can be also NRaclr is limited for some NRaclr > 50 RB allocations

### 2.2.2	DFT-s-OFDM

Signal parameters are as follows

- Spectrum utilization 106 RB for 20 MHz signal (max allocation size is 100 RB)

- WOLA with slope length of Ncp/8

QPSK

- QPSK does not need MPR which is logical as 0 dB MPR waveform is fully allocated QPSK signal

16-QAM

- 1 dB MPR is needed for some allocations

- RBsize 3 – 10 RB in the edge SEM is limiting factor and otherwise NRaclr is limiting factor

64-QAM

- 1 dB MPR is needed for all allocations

- NRaclr is limiting factor for RBsize >  25 RB otherwise in-band emissions are limitting

### 2.2.3	CP-OFDM with clipping

Signal parameters are as follows

- Spectrum utilization 106 RB for 20 MHz signal

- WOLA with slope length of Ncp/8

- Clipping threshold is 5 dB for QPSK, and 6 dB for 16-QAM. Clipping is done at the baseband before channel filter. The resulting baseband EVM is about 8% for QPSK and 7% for 16-QAM. Clipping not done for 64-QAM.

QPSK

- Small allocations (up to 3RB at edge and up to 5RB in the middle) do not need MPR.

- Close to edge for RBsize 4 – 10 RB  SEM is limiting and when RBsize > 8 NRaclr is limiting factor RB

- All allocations not at the edge are in-band emissions limited, due to the clipping

- Compared to unclipped signal maximum MPR is reduced from 3 dB to 2 dB

16-QAM

- Some MPR is always needed 1-3 dB

- Close to edge for RBsize 3 – 10 RB  SEM is limiting and when RBsize > 8 NRaclr is limiting factor RB

- All allocations not at the edge are in-band emissions limited, due to the clipping

- Compared to unclipped signal less waveforms need 3 dB MPR

64-QAM

- Clipping not feasible due to in-band emissions / EVM budget

## 2.3	Measurement Results

In parallel to extensive simulations done using an AM/AM and AM/PM based model extracted from measurement, a series of measuremenmt were performed on a 3.5GHz PA designed for LTE PC3 and PC2 (only using different supply voltages). A number of waveforms were measured covering:

- CP-OFDM and DFT-s-OFDM

- Full and Partial allocations (at channel edge and in-channel)

- 20MHz and 100MHz bandwidths

- QPSK, 16QAM and 64QAM

- Normal, clipped or low PAPR waveforms

ACLR and SEM measurements were performed in and measured output power is reported at the PA output while simulations are at antenna output assuming 5dB post PA losses.

### 2.3.1 PA operating point calibration

Table4 shows the LTE and NR PA calibration measurement. As expected the output power capability is virtualy the same with the main different being the 1dB higher NR linearity requirement.

Table 4: LTE and NR PA operating point calibration

| type | BW | RB | modulation | limit | Pout | MPR |
| --- | --- | --- | --- | --- | --- | --- |
| SC-FDMA | 20MHz | 100RB | QPSK | 30dB EUTRA ACLR | 28dBm | 1dB LTE |
| DFT-s-OFDM | 20MHz | 100RB3 | QPSK | 30dB NR ACLR | 27.8dBm | 0dB NR |

To compare simulation and measurement correlaltion and the validity of the AM/AM and AM/PM 3.5GHz PA model it was chosen to use a 100MHz CP-OFDM QPSK waveform as being representative of the larger bandwidth and higher PAPR ranges for the study. Figure 1 shows the QPSK OFDM 100MHZ spectrum at NR ACLR limit while Figure 2 shows the same simulated case.

Figure 1: QPSK CP-OFDM 273RB0 measured spectrum at 30dBc NR ACLR limit

Figure 2: QPSK CP-OFDM 273RB0 simulated spectrum at 30dBc NR ACLR limit

Table 5: measured versus simulated ACLRs for QPSK CP-OFDM 273RB0 waveform

| type | Pout (PA) | Pout (antenna) | NR ACLR L/U | EUTRA1 ACLR L/U | EUTRA1 ACLR L/U |
| --- | --- | --- | --- | --- | --- |
| measured | 26dBm | 21dBm | 31.0/30.3dBc | 34.6/34.1 dBc | 35.9/36.8 dBc |
| simulated |  | 20.7 dBm | 30 dBc | 34.1 dBc | 35.9 dBc |

As can be seen from Table 5 there is very good correlation between measurements and simulations.

### 2.3.2 20MHz full allocation measurement

Table 6: 20MHz fully allocated NR waveforms MPR or power boost

| type | BW [MHz] | nbRB-RBstart -SCS | modulation | NR ACLR | SEM | Pout¸ [dBm] | NR MPR /PB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | 20 | 100RB3-15kHz | QPSK | limit | OK | 27.8 | 0dB MPR |
| DFT-s-OFDM | 20 | 100RB0-15kHz | QPSK | limit | OK | 27.5 | 0.5dB MPR |
| CP-OFDM | 20 | 106RB0-15kHz | QPSK | limit | OK | 25.5 | 3dB MPR |
| CP-OFDM | 20 | 106RB0-15kHz | 16QAM | limit | OK | 25.6 | 3dB MPR |
| CP-OFDM | 20 | 106RB0-15kHz | 64QAM | limit | OK | 25.4 | 3dB MPR |
| CP-OFDM | 20 | 106RB0-15kHz | QPSK clipped | limit | OK | 26.3 | 1.5dB MPR |
| DFT-s-OFDM | 20 | 100RB0-15kHz | Pi/2 BPSK shaped | limit | OK | 28.7 | 1dB PB |

ACLR and SEM measurements were performed for a number of 20MHz fully allocated NR waveforms and are displayed in table 6. The following observation can be made:

- The non symmetric DFT-s-OFDM 100RB0 requires a slight MPR => fully allocated DFT-s-OFDM waveforms shall be centered

- All CP-OFDM require 3dB MPR for all QPSK/16QAM/64QAM modulations

- Clipping can improve MPR by 1.5dB but same improvement can be anticipated for clipping of DFT-s-OFDM waveforms

- Spectrally shaped PI/2 BPSK DFT-s-OFDM allows 1dB power boost (can be even higher with further PAPR reduction)

### 2.3.3	100MHz full allocation measurement

Table 7: 100MHz fully allocated NR waveforms MPR or power boost

| type | BW [MHz] | nbRB-RBstart -SCS | modulation | NR ACLR | SEM | Pout¸ [dBm] | NR MPR /PB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | 100 | 270RB0-30kHz | QPSK | limit | OK | 27.1 | 1dB MPR |
| CP-OFDM | 100 | 273RB0-30kHz | QPSK | limit | OK | 25.3 | 3dB MPR |
| CP-OFDM | 100 | 273RB0-30kHz | QPSK clipped | limit | OK | 26.1 | 2dB MPR |
| DFT-s-OFDM | 100 | 270RB0-30kHz | Pi/2 BPSK shaped | limit | OK | 28.3 | 0.5dB PB |

ACLR and SEM measurements were performed for a number of 100MHz fully allocated NR waveforms and are displayed in table 7. The following observation can be made:

- 100MHz DFT-s-OFDM waveform requires 1dB MPR compared to the reference 20MHz waveform => symmetrical allocation should be evaluated for lower MPR

- QPSK CP-OFDM require 3dB MPR making it constant versus BW

- Clipping can improve MPR by 1.5dB but same improvement can be anticipated for clipping of DFT-s-OFDM waveforms

- Spectrally shaped PI/2 BPSK DFT-s-OFDM allows 0.5dB power boost again reduced when compared to the 20MHz waveform

### 2.3.4	20MHz partial allocation measurement

Table 8: 20MHz partially allocated NR waveforms MPR or power boost

| type | BW [MHz] | nbRB-RBstart -SCS | modulation | NR ACLR | SEM | Pout¸ [dBm] | NR MPR /PB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | 20 | 18RB0-15kHz | QPSK | limit | OK | 31.7 | 1dB PB |
| CP-OFDM | 20 | 18RB0-15kHz | QPSK | limit | OK | 30.2 | 0.5dB MPR |
| CP-OFDM | 20 | 18RB0-15kHz | QPSK clipped | limit | OK | 30.7 | 0dB MPR |
| DFT-s-OFDM | 20 | 18RB0-15kHz | Pi/2 BPSK shaped | limit | OK | 33.7 | 3dB PB |
| DFT-s-OFDM | 20 | 18R18-15kHz | QPSK | OK | limit | 33 | 2dB PB |
| CP-OFDM | 20 | 18R18-15kHz | QPSK | OK | limit | 32.4 | 1.5dB PB |
| CP-OFDM | 20 | 18R18-15kHz | QPSK clipped | OK | limit | 32.1 | 1.5dB PB |

ACLR and SEM measurements were performed for a number of 20MHz partially allocated NR waveforms and are displayed in table 6. The following observation can be made:

- All partial waveform show reduced MPR and most of the time Power boost

- QPSK Partial allocations at channel edge are SEM limited and allow 1-2dB higher power than full allocations

- QPSK Partial allocations in channel are SEM limited and show 2-5dB higher power than full allocations

- Spectrally shaped PI/2 BPSK DFT-s-OFDM allows 3dB power boost

Since simulations have shown that in-band emission may limit the output power for these partial allocations the MPR reduction can be deducted from simulation results. Nevertheless power boost options may be studied further especially within the scope of setting in-band emission requirement for NR versus the different Sub Carrier spacing.

# 3	Conclusion

Simulation and measurement results presented in this paper show that there are QPSK CP-OFDM waveforms that do not require MPR or may enable power boost and can be used in extreme coverage situations if 256-QAM capable UE Tx assumptions are used and PA linearity is slightly increased compated to LTE.

Standard LTE In-band emissions requirement is limiting factor for many of the waveforms (and more specifically for partial allocations) and RAN4 need to consider proper definition for NR in-band emissions as also discussed in [8].

It was also observed that fully allocated DFt-s-OFDM waveforms shall be allocated with a symmetrical position to channel center and not aligned with the CP-OFDM RB0 position.

# 4	References

[1] R4-166954, WF on MPR\AMPR simulation assumptions for UL 256QAM, Huawei, Hisilicon, Qualcomm, CMCC, Nokia, Skyworks, Ericsson, RAN4#80

[2] R4-1705509, Power Class Definition for Sub-6GHz NR, Skyworks Solutions, Inc., RAN4#83

[3] R4-1706316, WF on sub-6GHz ACLR and ACS, Nokia, Alcatel-Lucent Shanghai Bell, RAN4#83

[4] 0 dB waveform and 256-QAM Tx

[5] R4-1706692,	NR Sub-6 GHz SEM, Nokia, Alcatel-Lucent Shanghai Bell, Skyworks Solutions, Inc.

[6] R4-1706613, Proposed ACLRs definition for NR UE Bellow 6GHz , Skyworks Solutions Inc. Nokia, Alcatel Lucent Shanghai Bell

[7] R4-1706612, Proposed EVM for NR UE Bellow 6GHz, Skyworks Solutions Inc. Nokia, Alcatel Lucent Shanghai Bell

[8] R4-1706693,	NR Sub-6 GHz In-band emission requirement, Nokia, Alcatel-Lucent Shanghai Bell, Skyworks Solutions, Inc.