3GPP TSG-RAN WG4 Meeting NR#2 Ad Hoc                                     R4-1706716

Qingdao, China, 27 - 29 Jun 2017

Source:		Ericsson

Title:			Requirements for NR UE timing advance

Agenda item:		3.5.3

Document for:	Approval

# 1 Introduction

In this contribution, we continue the discussion regarding timing advance for NR and examine some of the observations in [1], when it comes to developing requirements based on RAN1 decisions and comment on initial access and mobility.

# 2 Discussion

## 2.1 TA loop, steady state

In [1] we evaluated how accurately each UE’s timing can be set to the ideal timing TA0, from the gNB perspective, for the steady state case of an attached, non-mobile, UE that receives TA commands from the gNB.

The feedback loop consists of the following steps

- gNB determines a TA command to apply

- UE receives the TA command and applies it to its transmit timing

For each of these steps, an uncertainty budget is shown it table 1 for LTE

| Source of uncertainty | Reference | Budget LTE Ts = 1/(15000 x 2048) seconds |
| --- | --- | --- |
| 1. gNB determines a TA command to apply | 36.133 section 7.3 indicates “The timing advance command is expressed in multiples of 16* TS and is relative to the current uplink timing. “ | ±8Ts |
| 2. UE applies TA command | 36.133 section 7.3 indicates “The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to ±4* TS seconds to the signalled timing advance value compared to the timing of preceding uplink transmission.” | ±4Ts |
| TOTAL (ideal case) |  | ±12Ts |

Table 1: Uncertainty budget for timing advance setting

When different numerologies are considered (e.g. with shorter symbol duration/Ts), it could be expected that many of the uncertainties would scale according to the shorter Ts. In other words, in absolute time terms the gNB should be able to measure the signal with greater granularity, TA commands should support setting the UE timing advance with smaller steps, and the UE should also be able to implement finer adjustment for the uplink. Another way of stating this is simply that Ts becomes shorter as the subcarrier spacing is increased.

We get a total uncertainty of ±13 Ts. If we compare this with the LTE CP length of 144 Ts, we see that the fraction of CP used by TA uncertainty is 26/144 ≈ 20%. (There will be some added uncertainty from the UE extracting DL timing reference, especially in poor SNR, but it is not modelled here).

For NR, we will have several subcarriers spacing possibilities. They are listed in table 2.

The CP length is inversely proportional to Subcarrier Spacing as per RAN1 decision.

| Subcarrier spacing (kHz) | 15 | 30 | 60 | 120 | 240 |
| --- | --- | --- | --- | --- | --- |
| Slot duration (µs) | 500 | 250 | 125 | 62.5 | 31.25 |
| OFDM symbol, duration (µs) | 66.67 | 33.33 | 16.67 | 8.34 | 4.17 |
| Cyclic prefix, duration (µs) | 4.69 | 2.34 | 1.17 | 0.59 | 0.29 |
| OFDM symbol including cyclic prefix (µs) | 71.35 | 35.68 | 17.84 | 8.92 | 4.46 |

Table 2: CP lengths for normal CP.

If we assume that TA uncertainty is allocated the same fraction of the CP, 20%, as current LTE at 15 kHz subcarrier spacing, then we get the total TA uncertainty, for the higher subcarrier spacing of NR, 30, 60, 230 and 240 kHz, listed in table 3:

| Subcarrier spacing (kHz) | 15 | 30 | 60 | 120 | 240 |
| --- | --- | --- | --- | --- | --- |
| Cyclic prefix, duration (Ts) | 144 | 72 | 36 | 18 | 9 |
| TA uncertainty (Ts), for 20% of normal CP | ±13 | ±6.5 | ±3.25 | ±1.6 | ±0.81 |

Table 3: TA uncertainty

In RAN4#83 there was a discussion with regards to the DigRF specification (from MIPI) between RF and baseband will give an uncertainty that does not scale with subcarrier spacing. However, historically, MIMI specification has been updated every time a new technology has been introduced (GSM, WCDMA,LTE). What we do here is to specify the new 3GPP starting point as input to other specification groups like MIPI.

Observation 1: The TA error must scale with numerology to keep the TA error fraction of CP constant.

If we continue to scale the budget from table 1 we get table 4.

| Source of uncertainty | Reference | 15 | 30 | 60 | 120 | 240 |
| --- | --- | --- | --- | --- | --- | --- |
| 1. gNB determines a TA command to apply | 36.133 section 7.3 | 8 | 4 | 2 | 1 | 0.5 |
| 2. UE applies TA command | 36.133 section 7.3 | 4 | 2 | 1 | 0.5 | 0.25 |
| Total |  | ±12 | ±6.0 | ±3.0 | ±1.5 | ±0.75 |

Table 4: TA uncertainty budget examples

If we start to analyse table 4, based on the biggest contributors to the uncertainty we notice that the reporting granularity should improve. It is ±8 Ts today in LTE. The quantization uncertainty is scaled in table 4 above, but in an actual standard this would be defined by protocol and the strictest requirement would be used all over, i.e. ±1 Ts quantization granularity. Scaling requirements beyond 1 Ts in BS measurement and UE allocation of TA command leads to very strict requirements and might not be feasible.

Proposal 1: Scale TA command resolution with SCS, from 16 Ts for 15 kHz SCS.

Proposal 2: Scale UE relative error relative to ordered TA value, from 4 Ts for 15 kHz SCS.

There will be some added uncertainty from the UE extracting DL timing reference, especially in poor SNR. Time stamping normally depends on 1) SNR and 2) BW.  NR have higher minimum BW, so we can expect better time stamp accuracy.

## 2.2 Mobility

The gNodeB provides the UE with a configurable timer called timeAlignmentTimer. TimeAlignmentTimer is used to control how long the UE is considered uplink time aligned

The value of the timer is either UE specific (timeAlignmentTimerDedicated) and managed through dedicated signalling between the UE and the gNodeB, or cell specific (timeAlignmentTimerCommon) which is indicated in SIB2. In both cases, the timer is normally restarted whenever a new Timing Advance is given by the gNodeB. At the time of restart, the timer is restarted to a UE specific value if configured; otherwise it is restarted to a cell specific value

As discussed before, the gNodeB continuously measures timing of uplink signal from the UE and adjusts the uplink transmission timing by sending the Timing Advance Command to the UE. If the UE has not received a Timing Advance Command until the expiry of timeAlignmentTimer, the UE assumes that it has lost the uplink synchronization.

Observation 2: Worst case TA error is set by Initial error just after TA applied (measurement and reporting) + a drift part (mobility and TA periodicity)

It is up to gNB implementation to maintain accurate TA tracking and adjust by command to UE, with the shorter symbols of NR due to bigger subcarrier spacing of NR it is expected that this tracking has to be done in a more accurate way and with more commands per time unit to mitigate the drift part of the error.

## 2.3 TDD GP allocation and uplink transmission in the short-duration

The TA accuracy is also a factor when it comes related to adjacent in time UE UL transmission with transient discussion based on RAN 1 question in [3], the TA error would add to the issue (and probably be most significant), given that, for 15 kHz sub carrier spacing, we would have ±13 Ts which corresponds to ±0.4 µs.

LS R1-1703782, where RAN1 asked RAN4 the following questions,

RAN1 asks RAN4 to study transient period (including feasible values especially the one(s) smaller than that of LTE) considering uplink transmission in the short-duration.

RAN1 also asks RAN4 whether or not the transient period(s) relates to transmission characteristics e.g., bandwidth and frequency location in a bandwidth, PSD, frequency-hopping during the transmission, carrier frequency.

Finally, In the TDD time budget [3], we discuss margin for TA errors, if significant. In this contribution, we start at ±12 Ts which corresponds to ±0.4 µs and then scale. This keeps the impact of TA error to a minimum.

Observation 3: TA accuracy impacts budget for TDD GP allocation AND uplink transmission in the short-duration transient behaviour.

# 3 Conclusion

Observation 1: The TA error must scale with numerology to keep the TA error fraction of CP constant.

Proposal 1: Scale TA command resolution with SCS, from 16 Ts for 15 kHz SCS.

Proposal 2: Scale UE relative error relative to ordered TA value, from 4 Ts for 15 kHz SCS.

Observation 2: Worst case TA error is set by Initial error just after TA applied (measurement and reporting) + a drift part (mobility and TA periodicity)

Observation 3: TA accuracy impacts budget for TDD GP allocation AND uplink transmission in the short-duration transient behaviour.

# 4 References

- R4-167639, Timing advance for TDD NR. Ericsson

- R1-1703782, LS

- R4-1703013, TDD Time budget