3GPP TSG-RAN WG4 Meeting NR #2							R4-1706721

Qingdao, China, 27th – 29th June, 2017

Title: 	Expected RRM requirements for NSA in TS38.133

Source: 		Huawei, HiSilicon

Agenda item:		3.5.4

Document for:			Discussion

# Introduction

NSA operation is of high priority. It was encouraged in [1] in RAN4 #82bis that RAN4 should first discuss the expected measurement requirements needed for enabling NSA option 3 operation. This topic was initially discussed in last RAN4 #83 meeting. Some preliminary agreement on expected RRM requirements for NSA in TS38.133 was made in the evening ad-hoc and was captured in [2]:

| RRM requirements expected in TS38.133 for enabling dual connectivity option 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Types of requirements | Functionalities | Huawei | Ericsson | Agreement |
| RRC_IDLE state mobility | Cell selection/re-selection | N | N | No requirement |
| E-UTRAN RRC_INACTIVE state mobility |  |  | Pending RAN2 |  |
| E-UTRAN RRC_CONNECTED state mobility | Handover | N | N |  |
| RRC Connection Mobility Control | RRC re-establishment | Y | N |  |
|  | Random access | Y | Y | Requirement is needed |
| UE timing and signalling characteristics | UE transmit timing | Y | Y | Requirement is needed |
|  | UE timer accuracy | N |  |  |
|  | Timing advance | Y | Y | Requirement is needed |
|  | Radio link monitoring | Y | Y | Requirement is needed |
|  | Interruption with DC | Y | Y | Requirement is needed |
|  | Cell phase synchronization accuracy | TBD | Y for TDD |  |
|  | PSCell addition/release/change delay | Y | Y | Requirement is needed |
|  | Maximum Receive Timing Difference in Dual Connectivity | TBD |  |  |
|  | Maximum Transmission Timing Difference in Dual Connectivity | TBD |  |  |
|  | NR SCell Activation and Deactivation Delay | Y | Y |  |
| UE Measurements Procedures in RRC_CONNECTED State | [Intra/inter-frequency] measurement (including cell identification, beam management) | Y | Y | Requirement is needed |
| Measurements performance requirements for UE | Measurement accuracy | Y | Y | Requirement is needed |

As can be seen above, there are still some TBD in the table. In this contribution, we provide further discussion on the remaining issues.

# Discussion

In this section we will discuss the necessity of RRM requirements which have not been decided according to the above table.

- E-UTRAN RRC_INACTIVE state mobility

Inactive mode is a new RRC mode designed in NR. The relevant functionalities in this mode is being discussed in RAN2, including e.g. paging, PLMN selection, resume and etc. From RAN4 RRM perspective, the related UE behaviors at least include cell selection/reselection at this stage. Whether or not there is other RRM impact depends on RAN2 further progress.

The inactive mode is used for power saving compared with connected mode, and faster connection establishment compared with idle mode. In NSA operation option 3, UE first access an LTE PCell, and then will be configured with an NR PSCell. Note that the procedure belongs to connected mode. When the traffic is done UE would release the PSCell or even PCell. Then this is not NSA operation anymore. From mobility point of view, UE in RRC_INACTIVE state will perform cell selection/reselection according to RAN2 agreements. Since it was agreed in last meeting that there is no need to define cell selection/reselection. Thus we believe no need to define RRM requirement in RRC_INACTIVE state for NSA operation.

Proposal 1: RRC_INACTIVE state mobility requirements are not needed for enabling NSA option 3 operation.

- Handover

As mentioned in [3], corresponding handover requirements focus on the handover delay in handover procedure. No additional work is needed since in DC option 3 the PCell is a legacy LTE cell and UE needs to follow all the existing handover requirements in TS36.133. New handover schemes, e.g. DC-related mobility enhancement, are being discussed in RAN2. Note that all of the new handover schemes focus on SA scenarios (or PCell is an NR cell), irrelevant with NSA option 3 operation. Thus we propose:

Proposal 2: handover requirements are not needed for enabling NSA option 3 operation.

- RRC re-establishment

RRC re-establishment procedure occurs after RLF, which is triggered by RLM. In NSA operation, UE also needs to monitor the link quality of NR PSCell. In legacy LTE DC scenario, UE will perform RLM on both PCell and PSCell. Once RLF happens on PSCell, UE will inform PCell and then PCell might reconfigure another PSCell for the UE, rather than perform RRC re-establishment directly on PSCell. As for NR, it is unlikely that UE will trigger RRC re-establishment procedure on NR PSCell when RLF happens on the NR link whilst the link to E-UTRAN is still good, i.e. similar approach with LTE DC can be foreseen.

Proposal 3: RRC re-establishment requirements for NR PSCell are not needed for enabling NSA option 3 operation unless RAN2 decide that UE will perform RRC re-establishment on NR PSCell when RLF happens on the NR PSCell.

- Cell phase synchronization accuracy (Synchronized mode of dual connectivity)

- Maximum Receive Timing Difference in Dual Connectivity

- Maximum Transmission Timing Difference in Dual Connectivity

Before we conclude on whether these requirements are needed, we need to discuss the concept of “synchronous DC” in LTE and NR inter-working scenario. Counting that LTE and NR have different physical design, UE anyway may need two separated receivers and modules to process in parallel, irrespective of the timing difference between LTE and NR. However, the DL/UL operations should be aligned between the LTE cell and NR cell, and the cell phase synchronization requirements are needed to guarantee the DL/UL operation for each link would not be overlapped.

Another issue is on power control. How to share the UL TX power between the E-UTRAN link and NR link is not clear now. So the impact should be further studied once the RAN1 design become more stable.

- NR SCell Activation and Deactivation Delay

In future carrier aggregation will be supported in E-UTRAN link and NR link in NSA operations. Actually NSA with LTE CA, e.g. LTE 2DL/1UL + NR, has been discussed. Whether or not this requirement is needed in Rel-15 depend on operators input. There has been some discussion but no official agreements have been made. Thus we would like to keep it open at this stage, once we get official input we can further study.

# Conclusion

In this contribution we provide further discussion on the expected RRM requirements for NSA in TS38.133. After discussion the following conclusions are made:

Proposal 1: RRC_INACTIVE state mobility requirements are not needed for enabling NSA option 3 operation.

Proposal 2: handover requirements are not needed for enabling NSA option 3 operation.

Proposal 3: RRC re-establishment requirements are not needed for enabling NSA option 3 operation unless RAN2 decide that UE will perform RRC re-establishment on NR PSCell when RLF happens on the NR PSCell.

Besides, we also update the summary of table as below (yellow highlight):

| RRM requirements expected in TS38.133 for enabling dual connectivity option 3 |  |  |
| --- | --- | --- |
| Types of requirements | Functionalities | Agreement |
| RRC_IDLE state mobility | Cell selection/re-selection | No requirement |
| E-UTRAN RRC_INACTIVE state mobility | Cell selection/re-selection | No requirement |
| E-UTRAN RRC_CONNECTED state mobility | Handover | No requirement |
| RRC Connection Mobility Control | RRC re-establishment | No requirement |
|  | Random access | Requirement is needed |
| UE timing and signalling characteristics | UE transmit timing | Requirement is needed |
|  | UE timer accuracy | TBD |
|  | Timing advance | Requirement is needed |
|  | Radio link monitoring | Requirement is needed |
|  | Interruption with DC | Requirement is needed |
|  | Cell phase synchronization accuracy | Requirement is needed |
|  | PSCell addition/release/change delay | Requirement is needed |
|  | Maximum Receive Timing Difference in Dual Connectivity | TBD |
|  | Maximum Transmission Timing Difference in Dual Connectivity | TBD |
|  | NR SCell Activation and Deactivation Delay | TBD |
| UE Measurements Procedures in RRC_CONNECTED State | [Intra/inter-frequency] measurement (including cell identification, beam management) | Requirement is needed |
| Measurements performance requirements for UE | Measurement accuracy | Requirement is needed |

# Reference

[1] R4-1702888, “Way forward for NR RRM”, Ericsson, RAN4#82bis

[2] R4-1704806, “NR RRM way forward”, Ericsson, Nokia, ZTE

[3] R4-1704989, “RRM consideration on NSA requirements”, Huawei, HiSilicon