3GPP TSG-RAN WG4 Meeting #NR_AH2	R4-1706768

Qingdao, China, 27 – 29 June, 2017

Agenda item:	3.2.5

Source:	Dish Network

Title:	Considerations regarding WF on NR UE ACS for Sub-6GHz

Document for:	Approval

# 1	Introduction

WF on sub-6GHz ACLR and ACS was approved in RAN4#83 [1]. This contribution discusses implications of the WF that should be accounted in UE future work.

# 2	Discussion

UE ACS WF from [1] is as follows.

- ACS = 33dB for wanted channel bandwidth up to 20MHz.

  - ACS for other NR wanted channel bandwidth are FFS.

  - The adjacent channel bandwidth the same as the wanted signal.

  - ACS for different adjacent channel bandwidth are FFS.

  - ACS may be updated as SNR for Refsens is currently TBD.

- Case 1: Refsens+14dB for wanted signal and Refsens+45.5 dB for interferer.

- Case 2: -56.5dBm for wanted signal and -25dBm for interferer.

  - Case 1 and 2 is based on IM=2.5dB and SNR=-1dB.

Figure 1 below illustrates ACS for 20MHz channel. NR CC (according to WF) with ACS blocker is on the left-hand side and LTE CC with ACS blocker is on the right-hand side. PSD difference is 31.5dB for both RAT’s.

Figure 1ACS illustration for NR and LTE

In this contribution, discuss different topics related to the agreed WF [1]. In general, we are ok with it but we’d like to raise discussion on some important points as early as possible.

The WF didn’t treat new NR frequencies and NR bands re-farmed from LTE differently. Thus, the ACS requirements for NR bands that previously were LTE bands will change in terms of ACS level and ACS BW for 10, 15, and 20MHz CC BW’s. The PSD difference between the wanted signal and the blocker for 5MHz and 20MHz CC’s will remain the same while the PSD difference for 10 and 15MHz will be a bit smaller. The power difference between the wanted signal and the blocker for 5MHz and 10MHz CC’s will remain the same while the power difference for 15MHz and 20MHz will increase. Naturally, the change in ACS BW does not change adjacent blockers in real network deployments. The noise characteristics (spectrum regrowth, phase noise) of real ACS blocker are a bit different when comparing LTE ACS and NR ACS [1] but in testing there should be no difference because testers use very steep filters in blocker generation.

Observation 1: ACS requirements for NR bands re-farmed from LTE will change in terms of ACS level and ACS BW for 10, 15, and 20MHz CC BW’s

One of the open issues is ACS for CC’s larger than 20MHz. It is likely that ACS blockers narrower than the own carrier exist in NR, especially for bands re-farmed from LTE. For instance, operator A could own 40MHz spectrum and operator B could own 5MHz spectrum right next to operator A. In this case, it should be considered if the principle agreed in WF for CC’s up to 20MHz is still appropriate or if narrower ACS should be considered as well. The group should evaluate if narrow ACS (let’s say 5MHz like in LTE) should be defined in addition to wider ACS or if having only wider ACS requirement is enough to guarantee ACS performance in case of narrower adjacent carrier.

Proposal 1: 5MHz ACS should be considered as well for bandwidths larger than 20MHz

One of the implications of using wider ACS blocker is that it both widens and pushes also in-band blockers further away. As an example, it would be illogical if ACS BW for 20MHz CC would be 20MHz and in-band blockers BW would still be 5MHz like in LTE. It can be easily envisioned that IBB1 and IBB2 would also be as wide as the wanted signal. Once they are further away their levels should be shifted up accordingly.

Observation 2: In-band blockers become wider and are further away when ACS blocker is wider

Proposal 2: Observation 2 shall be accounted when specifying in-band blocking requirements

Finally, we consider one practical blocking requirement. There is a specific requirement for Band 71; a TV blocker, which is 12 MHz below the Band 71 DL. In LTE, this is specified as in-band blocker case 6 that is 5MHz wide blocker at -15dBm power level centered 12MHz below B71 DL. When wider ACS applied for this band in NR; for 10, 15, and 20MHz bandwidths this blocker is inside the ACS frequency range. That is not an issue as such, but the implication is that this blocker should still be verified somehow, in addition to the ACS defined in [1].

Observation 3: The nature of some band-specific blockers will change due to wider ACS blocker

Proposal 3: Specific blocker for NR band re-farmed from LTE B71 shall be addressed in NR specifications even if it would be inside the ACS range

# 3	Conclusions

Considerations based regarding WF on NR UE ACS for sub-6GHz are provided, with the following observations and proposals.

Observation 1: ACS requirements for NR bands re-farmed from LTE will change in terms of ACS level and ACS BW for 10, 15, and 20MHz CC BW’s

Proposal 1: 5MHz ACS should be considered as well for bandwidths larger than 20MHz

Observation 2: In-band blockers become wider and are further away when ACS blocker is wider

Proposal 2: Observation 2 shall be accounted when specifying in-band blocking requirements

Observation 3: The nature of some band-specific blockers will change due to wider ACS blocker

Proposal 3: Specific blocker for NR band re-farmed from LTE B71 shall be addressed in NR specifications even if it would be inside the ACS range

# 4	References

- R4-1706316 “WF on sub-6GHz ACLR and ACS”, Nokia, Alcatel-Lucent Shanghai-Bell, RAN4#83, Hangzhou, China