3GPP TSG-RAN WG4 Meeting NR#2	R4-1706813

Qingdao, China, 27-29 June 2017

Source: 	Ericsson

Title:  	Simulation results for receiver dynamic range of NR BS receiver without array gain for different deployment scenarios

Agenda Item:	3.4.4.3

Document for:	Approval

# Introduction

In RAN4#82bis, a way forward has been agreed in [2] on receiver dynamic range requirements for NR BS, where it is agreed:

- To further consider the following 2 options:

  - 1) Not to specify the mmWave NR BS RX dynamic range requirement in the RAN4 specifications.

  - 2) Specify the mmWave NR BS RX dynamic range requirement [5-15] dB over the BS noise floor.

For above 24GHz mmwave scenarios, we reused the existing simulation assumptions of WP5D coexistence study captured in the TR 38.803 for preliminary study.

In previous contribution, we provided our analysis on receiver dynamic range for NR in different deployment scenario [5]. In the contribution, we provided our analysis on receiver dynamic range for NR in different deployment scenario without the array gain, i.e. only considering the element gain.

# Noise Floor Rise for Receiver Dynamic Range Requirement in mmWave NR BS

As described in [3], for dynamic range requirement of E-UTRA BS, power level of interfering signal can be written as

PIntf [dB]= -174+10*log10(Bchannel) + NF[dB] + NFR[dB],                                         (1)

On the other hand, the power level of the wanted signal can be written as

Pwanted [dB]= -174+10*log10(Bwanted) + NF[dB] + NFR[dB]+𝛾[dB]+IM[dB].                         (2)

In equations (1) and (2), Bchannel is the bandwidth of the channel in Hz, NF is the noise figure in dB, NFR is the noise floor rise which is set to 20 dB, and derived according to the system level simulation results considering both inter-cell interference and RSRP measurement accuracy for uplink power control. As explained in [3], IM is implementation margin reserved for BS manufacturer in dB; For high MCS level, IM is assumed to be 2.5dB. 𝛾 is the Singal-to-Noise Ratio that satisfies 95% throughput of measurement channel.

The interference over thermal (IoT) can be calculated using

IoT= IK×T×B×NF ,                                                                                     (3)

With I being the UL interference level, K being Poltzmann’s constant, and T is the thermal noise temperature.

For NR, the NFR should be estimated using system level simulations. Usually, in these simulations, only in-channel interference is considered. However, in the following simulation results, we have also included adjacent channel interference, which is a more conservative approach. To add adjacent channel interference, we use the UL ACIR according to agreements in 38.803.  For the noise floor rise, we consider both 50% and 99% UL interference at victim system receiver. Given I, we can calculate the NFR by

NFR(I)[dB]=(I+CS)[dB]-CS[dB],                                                                       (4)

where, CS refers to the Conducted Sensitivity, which can be formulated as

CS[dB]= -174 + NF[dB] + 10 log(BW) + SINR[dB].                                                    (5)

# IoT Levels at Victim Network Without Array Gain

In this section we present the IoT levels at the victim network for UMa, UMi, and InH scenarios without the array gain included.

### IoT for UMa scenario without array gain

In Table 1 and Figure 1 we present the IoT levels for UMa scenario, GS refers to the grid shift, i.e. 0% GS represents the collocated case, while 100% GS refers to the non-collocated scenario.

Table 1: IoT levels for UMa scenario.

| Scenario | Grid Shift (GS) | Utilization | 50% IoT [dB] | 99% IoT [dB] |
| --- | --- | --- | --- | --- |
| UMa@30GHz 16dB ACIR | 0% | 50% | -37,7 | -26,3 |
|  | 0% | 90% | -33,9 | -20,7 |
|  | 100% | 50% | -37,4 | -16,8 |
|  | 100% | 90% | -30,0 | -9,0 |

Figure 1: IoT levels for UMa scenario.

### IoT for UMi scenario without array gain

In Table 2 and Figure 2 we present the IoT levels for UMi scenario.

Table 2: IoT levels for UMi scenario

| Scenario | Utilization | 50% IoT [dB] | 99% IoT [dB] |
| --- | --- | --- | --- |
| Umi@30GHz 16dB ACIR | 50% | -64,1 | -34,9 |
|  | 90% | -64,1 | -34,9 |
| Umi@45GHz 14dB ACIR | 50% | -64,4 | -36,0 |
|  | 90% | -64,4 | -36,0 |
| Umi@70GHz 14dB ACIR | 50% | -68,0 | -39,1 |
|  | 90% | -68,0 | -39,1 |

Figure 2: IoT levels for UMa scenario.

### IoT for InH scenario without array gain

Table 3 and Figure 3 we present the IoT levels for InH scenario.

Table 3: IoT levels for InH scenario.

| Scenario | Utilization | 50% IoT [dB] | 99% IoT [dB] |
| --- | --- | --- | --- |
| InH@30GHz 16dB ACIR | 50% | -33,4 | -18,1 |
|  | 90% | -29,8 | -14,7 |
| InH@45GHz 14dB ACIR | 50% | -31,7 | -15,5 |
|  | 90% | -28,5 | -11,6 |
| InH@70GHz 14dB ACIR | 50% | -31,4 | -18,4 |
|  | 90% | -28,3 | -16,5 |

Figure 3: IoT levels for InH scenario.

# NFR at Victim Network Without Array Gain

We have explained this in more detail in one of our companion papers [4]. Using equations (3) and (4) and interference levels in Tables 1 and 2, we can calculate the noise floor rise for 50% UL interference as in Tables 5, and 6.

### NFR for UMa scenario without array gain

The NFR levels for UMa scenario are presented in Table 4.

Table 4: NFR levels for UMa scenario

| Scenario | GS | Utilization | NFR @ 50% UL Interference [dB] | NFR @ 99% UL Interference [dB] |
| --- | --- | --- | --- | --- |
| UMa@30GHz 16dB ACIR | 0% | 50% | 0,0007 | 0,0102 |
|  | 0% | 90% | 0,0018 | 0,0371 |
|  | 100% | 50% | 0,0008 | 0,0902 |
|  | 100% | 90% | 0,0043 | 0,5231 |

### NFR for UMi scenario without array gain

The NFR levels for UMi scenario are presented in Table 5.

Table 5: NFR levels for UMi scenario

| Scenario | Utilization | NFR @ 50% UL Interference [dB] | NFR @ 99% UL Interference [dB] |
| --- | --- | --- | --- |
| Umi@30GHz 16dB ACIR | 50% | 0,000002 | 0,001411 |
|  | 90% | 0,000002 | 0,001411 |
| Umi@45GHz 14dB ACIR | 50% | 0,000003 | 0,001756 |
|  | 90% | 0,000003 | 0,001756 |
| Umi@70GHz 14dB ACIR | 50% | 0,000002 | 0,001354 |
|  | 90% | 0,000002 | 0,001354 |

### NFR for InH scenario without array gain

The NFR levels for InH are presented in Table 6.

Table 6: NFR levels for InH scenario

| Scenario | Utilization | NFR @ 50% UL Interference [dB] | NFR @ 99% UL Interference [dB] |
| --- | --- | --- | --- |
| InH@30GHz 16dB ACIR | 50% | 0,002 | 0,067 |
|  | 90% | 0,005 | 0,146 |
| InH@45GHz 14dB ACIR | 50% | 0,005 | 0,191 |
|  | 90% | 0,010 | 0,461 |
| InH@70GHz 14dB ACIR | 50% | 0,008 | 0,156 |
|  | 90% | 0,016 | 0,240 |

In the above results, we see that, InH case has higher noise rise in 99% UL interference, comparted to 50% UL interference level. It is worth noting here that, dynamic range is usually measured with respect to 50% interference levels. In our simulations, we have seen 0dB noise rise for InH case when 50% interference level is considered.

# Conclusion

We have presented noise floor rise for three different scenarios for mmWave spectrum without including the array gain. As it is seen for both 99% and 50% interference levels, the noise floor rise is very small, never exceeds 1 dB.

As reported in [3], the noise rise could be very small for NR, effectively considering it 0dB, thus our results are inline with [3].

Based on the simulation results, we observe the following:

Observation-1:

Noise floor rise for dynamic range of NR receiver without including the array gain is ~0dB for all scenarios at 50 and 99 percentile UL interference.

Thus, we propose the following:

Proposal: There is no need to specify dynamic range requirement for NR BS.

# References

RP-170847, New WID on New Radio Access Technology, NTT DOCOMO, INC.

R4-1704357, WF on mmWave NR BS RF receiver dynamic range requirements, Nokia, Alcatel-Lucent Shanghai Bell

R4-1702040, WF on noise floor rise of ICS and dynamic range, Source: ZTE Corporation

R4-1705094, Joint probability for receiver blocking, Ericsson

R4-1705097, Simulation results for receiver dynamic range of NR BS receiver with different deployment scenarios, Ericsson