3GPP TSG-RAN WG4 Meeting NR Ad Hoc #2                                                       R4-1706826

# Qingdao, China, 27 - 29 June, 2017

Source: 	ZTE Corporation

Title:	Further consideration on non-contiguous allocation in NR

Agenda Item:	3.3.3.1

Document for:	Discussion

# Introduction

In RAN4 #83, a proposal on how to handle non-contiguous resource allocation in NR[1], and one concern was also launched on  a larger signaling overhead in large BW case in order to support non-continous allocation [2].

In this contribution, we make a calculation of signalling overhead for resource allocation in NR by assuming the enumeration method similar in LTE, and based on the calculation we further propose that non-contiguous allocation should not be specified in Rel-15 NR.

# Discussion

## Signaling overhead for type 2 resource allocation in LTE

In LTE, signaling for resource allocation of both DL and UL is carefully designed and optimized since this signaling is one of the most frequently transmitted over the air from BS to UE, up to one signaling per UE per TTI. In type 2 resource allocation, contiguous RBs are allocated, and an enumeration method is introduced in order to reduce the signaling overhead. In this method, the total number of possible different contiguous resource allocation is enumerated as:

K=NRB22+NRB2

Where NRB is the maximum number of RBs. The following table shows K values of DL and UL respectively for different channel bandwidth.

|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| NRB | 6 | 15 | 25 | 50 | 75 | 100 |
| K for UL | 10 | 91 | 276 | 1176 | 2701 | 4851 |
| Num of bits required for UL resource allocation | 4 | 7 | 9 | 11 | 12 | 13 |
| K for DL | 21 | 120 | 325 | 1275 | 2850 | 5050 |
| Num of bits required for DL resource allocation | 5 | 7 | 9 | 11 | 12 | 13 |

## Signaling overhead for resource allocation in NR

Compared with the resource allocation in LTE, there are mainly two differences: 1) multiple sub-carrier spacing introduced: it is agreed that that sub-carrier spacing 15kHz /30kHz /60kHz should be supported for sub-6GHz; 2) larger bandwidth supported, although spectrum utilization is still under discussion in RAN4.

One example for the possible spectrum utilization for different combinations of SCS and channel bandwidth can be found in [3]:

|  | CH BW [MHz] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS [kHz] | 5 |  | 10 |  | 15 |  | 20 |  | 30 |  |
|  | NRB | K | NRB | K | NRB | K | NRB | K | NRB | K |
| 15 | 25 | 325 | 52 | 1378 | 79 | 3160 | 106 | 5671 | 161 | 13041 |
| 30 | [12] | 78 | 25 | 325 | 38 | 741 | 52 | 1378 | 79 | 3160 |
| 60 | N.A |  | N.A | N.A | N.A | N.A | 25 | 325 | 38 | 741 |

| SCS [kHz] | 40 |  | 60 |  | 80 |  | 100 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | K | NRB | K | NRB | K | NRB | K |
| 15 | 216 | 23436 | N.A. | N.A | N.A | N.A | N.A | N.A |
| 30 | 106 | 5671 | 162 | 13203 | 217 | 23653 | 273 | 37401 |
| 60 | 52 | 1378 | 80 | 3240 | 107 | 5778 | 135 | 9180 |

The maximum K value is 37401 which correspond to 16 bits, and if considering a 3-bit SCS choice, the total number of signaling bits in the resource allocation could be up to 19 bits in NR.

Observation 1: Due to the support of multiple SCS and larger bandwidth, signaling overhead for resource allocation in NR may be up to 19 bits.

From signaling overhead prospective, each non-contiguous allocation could be split into multiple independent contiguous allocations; therefore the straightforward estimate on the signaling overhead for non-contiguous allocation is multiple times of that for a contiguous allocation.

Observation 2: Each non-contiguous allocation could be split into multiple independent contiguous allocations thus the signaling overhead for non-contiguous allocation is multiple times of that for a contiguous allocation.

As discussed above, the total required signaling overhead for supporting non-contiguous allocation could be huge. Further study should be conducted aiming at an effective signaling compressing method. Obviously it seems not feasible in Rel-15 time frame. Therefore we propose that non-contiguous allocation should not be specified in Rel-15 NR.

Proposal 1: Non-contiguous allocation should not be specified in Rel-15 NR.

# Conclusion

Based on the above discussions, we have the following observations and proposal:

Observation 1: Due to the support of multiple SCS and larger bandwidth, signaling overhead for resource allocation in NR may be up to 19 bits.

Observation 2: Each non-contiguous allocation could be split into multiple independent contiguous allocations thus the signaling overhead for non-contiguous allocation is multiple times of that for a contiguous allocation.

Proposal 1: Non-contiguous allocation should not be specified in Rel-15 NR.

# References

R4-1705206, “Non-contiguous allocations for NR”, Qualcomm

RAN4 #83 Chairman notes

R4-1705207, “Way Forward on Spectral Utilization”, Qualcomm