3GPP TSG-RAN WG4 NR AH#2 Meeting	R4-1706850

Qingdao, China, 27 - 29 June 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Channel bandwidth and subcarrier spacing in Rel-15

Agenda item:		3.2.1

Document for:		Approval

# 1	Introduction

In this paper, we discuss the open issues on subcarrier spacing and channel bandwidth as summarized in WF [1], in which the following summary table is presented. Min and Max channel bandwidth agreed in [2,3] are already reflected, but there are still options for minimum channel bandwidth for each SCS.

| Frequency range | SCS (kHz) | Min CHBW (MHz) | Max CHBW (MHz) |
| --- | --- | --- | --- |
| Range 1 | 15 | 5 | 50 |
| Range 1 | 30 | Option 1: 5MHz Option 2: 10MHz | 100 |
| Range 1 | 60 | Option 1: 10MHz Option 2: 20MHz | 100MHz |
| Range 2 | 60 | 50 | 200 |
| Range 2 | 120 | 50 | 400MHz |

NOTE: These agreements and the following open issues agree to the data channel, not the PSS/SSS

# 2	Discussion

The following subcarrier carrier spacings have already been agreed for NR [4].

- SCS supported for bands below 1 GHz: 15 kHz, 30 kHz (support of 60 kHz pending RAN1 check)

- SCS supported for bands between 1GHz and 6GHz: 15 kHz, 30 kHz, 60 kHz

- SCS supported for bands above 24GHz and below 52.6GHz: 60 kHz, 120 kHz, 240 kHz for control (e.g. SS signals) but not for data

LS reply [5] indicates that the support of 60kHz for bands below 1GHz is not confirmed in RAN1. Thus, we exclude it in the following discussion.

For the minimum channel bandwidth of Range 1, two options remain either 5 or 10MHz for 30kHz SCS and 10 or 20MHz for 60kHz SCS in WF [1]. To support better spectrum utilization than LTE, it is not recommended to use the wide SCS for narrow channel bandwidths [6]; thus, we propose the option 2. This is also consistent with the request from RAN1 [7] to support minimum carrier bandwidth of 10MHz to accommodate SS Block with 30kHz SCS.

Therefore, we propose the following minimum channel bandwidths for 30 and 60kHz SCS.

Proposal 1: Minimum channel bandwidth for 30kHz data SCS is 10MHz for the frequency range below 6GHz.

Proposal 2: Minimum channel bandwidth for 60kHz data SCS is 20MHz for the frequency range below 6GHz.

The LS from RAN1 [7] also requests the minimum carrier bandwidth of 100MHz to accommodate SS Block with 240kHz SCS for the frequency range from 24 to 52.6 GHz. So far, RAN4 has agreed 50MHz minimum channel bandwidth as summarised in Section 1. Also 60 and 120kHz are the assumption for data SCS for the bands 24 to 52.6GHz [4].

As already agreed in RAN4, the set of supported NR bandwidths for each frequency band will be band specific [2] and 50MHz is the absolute minimum applied to the frequency range 24-52.6GHz. However, the minimum supported bandwidth is specific to each band and can be 100MHz.

Observation 1: Minimum channel bandwidth for each NR band is a band specific decision in RAN4.

For the bands with 240kHz SS block SCS, the channel bandwidth shall be larger than 69.12MHz because PSS/SSS/PBCH occupies 288 subcarriers [9]. For the bands with 30kHz SS block SCS, the channel bandwidth shall be larger than 8.64MHz.

SS block SCS is also a band specific decision in RAN4; each band shall be specified with a single SS block SCS. If these SS block SCS suggested by RAN1 are selected, i.e., 240kHz or 30kHz, then we propose that the minimum channel bandwidth is at least 100MHz or 10MHz, respectively, for these bands. RAN1 requests can be supported in this way and therefore the agreement in [1] does not need to be modified.

Observation 2: SS block SCS for each NR band is a band specific decision in RAN4.

Proposal 3: Minimum channel bandwidth for NR bands with 30kHz SS block SCS shall be at least 10MHz.

Proposal 4: Minimum channel bandwidth for NR bands with 240kHz SS block SCS shall be at least 100MHz.

We earlier proposed the supported channel bandwidth sets for Rel-15 [8] and further discussion took place in RAN4#83 meeting. The following channel bandwidth set is proposed as the starting point for Rel-15 so that the set of RF requirements for UE and BS are specified.

Proposal 5: We propose to use the following table as a starting point for defining NR channel bandwidths in Rel-15.

|  | Frequency range | SS block SCS (kHz) | Data SCS (kHz) | Supported CHBW set (MHz) |
| --- | --- | --- | --- | --- |
| Range 1 | 0 – 6 GHz | 15 | 15 | 5, 10, 15, 20, 25, 40 |
|  | 0 – 6 GHz | 30 | 15 | 10, 15, 20, 25, 40 |
|  | 0 – 6 GHz | 15 | 30 | 10, 15, 20, 25, 40, 50, 80, 100 |
|  | 0 – 6 GHz | 30 | 30 | 10, 15, 20, 25, 40, 50, 80, 100 |
|  | 1 – 6 GHz | 15 | 60 | 20, 25, 40, 50, 80, 100 |
|  | 1 – 6 GHz | 30 | 60 | 20, 25, 40, 50, 80, 100 |
| Range 2 | 24 – 52.6 GHz | 120 | 60 | 50, 80, 100, 200 |
|  | 24 – 52.6 GHz | 240 | 60 | 100, 200 |
|  | 24 – 52.6 GHz | 120 | 120 | 50, 80, 100, 200, 400 |
|  | 24 – 52.6 GHz | 240 | 120 | 100, 200, 400 |
|  | FFS | 240 | 240 | 100, 200, 400 |

Noted that the 240kHz data SCS is not a working assumption for 24-52.6GHz bands and may be used for higher frequency bands to support wider contiguous spectrum, thus, it would be reasonable to set the wider minimum channel bandwidth such as 100MHz. This can be included in Rel-15 specifications as a place holder, even though we do not introduce a corresponding NR frequency band.

# 3	Conclusion

Proposal 1: Minimum channel bandwidth for 30kHz data SCS is 10MHz for the frequency range below 6GHz.

Proposal 2: Minimum channel bandwidth for 60kHz data SCS is 20MHz for the frequency range below 6GHz.

Observation 1: Minimum channel bandwidth for each NR band is a band specific decision in RAN4.

Observation 2: SS block SCS for each NR band is a band specific decision in RAN4.

Proposal 3: Minimum channel bandwidth for NR bands with 30kHz SS block SCS shall be at least 10MHz.

Proposal 4: Minimum channel bandwidth for NR bands with 240kHz SS block SCS shall be at least 100MHz.

Proposal 5: We propose to use the following table as a starting point for defining NR channel bandwidths in Rel-15.

|  | Frequency range | SS block SCS (kHz) | Data SCS (kHz) | Supported CHBW set (MHz) |
| --- | --- | --- | --- | --- |
| Range 1 | 0 – 6 GHz | 15 | 15 | 5, 10, 15, 20, 25, 40 |
|  | 0 – 6 GHz | 30 | 15 | 10, 15, 20, 25, 40 |
|  | 0 – 6 GHz | 15 | 30 | 10, 15, 20, 25, 40, 50, 80, 100 |
|  | 0 – 6 GHz | 30 | 30 | 10, 15, 20, 25, 40, 50, 80, 100 |
|  | 1 – 6 GHz | 15 | 60 | 20, 25, 40, 50, 80, 100 |
|  | 1 – 6 GHz | 30 | 60 | 20, 25, 40, 50, 80, 100 |
| Range 2 | 24 – 52.6 GHz | 120 | 60 | 50, 80, 100, 200 |
|  | 24 – 52.6 GHz | 240 | 60 | 100, 200 |
|  | 24 – 52.6 GHz | 120 | 120 | 50, 80, 100, 200, 400 |
|  | 24 – 52.6 GHz | 240 | 120 | 100, 200, 400 |
|  | FFS | 240 | 240 | 100, 200, 400 |

# 4	References

- R4-1706055	WF on supporting channel bandwidth and SCS, Ericsson

- R4-1703100	LS Reply on NR minimum carrier bandwidth

- R4-1706321	WF on Maximum Channel Bandwidth, Qualcomm Incorporated

- R4-1704223	LS on SCS for NR

- R1-1709850	LS response on SCS for NR

- R4-1705342	Spectrum Utilization simulation results, Nokia, Alcatel-Lucent Shanghai Bell

- R1-1709842	LS response on NR minimum carrier bandwidth

- R4-1705341	Definition of NR Channel Bandwidths and Spectrum Utilization requirements, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1704312	LS on NR Initial access