3GPP TSG-RAN WG4 Meeting NR#2                                                              R4-1706932

Qingdao, China, 27 - 29 June, 2017

Agenda item:	3.2.5

Source: 	CMCC

Title: 	Discussion on NR co-existence study for 3.5GHz NR HPUE

Document for:	Approval

Introduction

A WF on high power UE for 3.5GHz was proposed in RAN #83 [1]. In this paper, we analyze NR HPUE co-existence in NR-LTE and NR-NR scenarios and discuss whether the LTE B41 HPUE co-existence results can be reused for 3.5GHz NR HPUE.

Discussion

Deployment scenarios

- 2.1.1	NR-LTE

LTE B41 HPUE co-existence simulation parameters (as specified in 36.886[2]) and NR parameters in 3.5GHz were compared in table 1.  It is observed that the main differences are frequency ranges and channel bandwidth,   which will result in different path-loss, ACLR and HPUE transmit power in system level simulation.

- The path-loss difference between the operating bands 3.5GHz and the simulation bands 2.6GHz in TR 36.886 is 2.7dB.  The larger path-loss compared with that in [2] will decrease wanted power while the interference level will also be decreased at the same time.

- For the same maximum output power, larger channel bandwidth (100MHz for single carrier) means a lower aggressor PSD, which will results in a lower ACLR in the victim.

- In addition, the power control modelling in [2] is adopted for NR HPUE and the transmit power is fine-tuned by power control factor 10*log(200MHz/bandwidth) considering the larger bandwidth. The parameter CLx-ile for 100MHz bandwidth is 105 compared with 115 for 10MHz bandwidth when γ = 1.

Observation 1: Different path-loss, power control factor and HPUE transmit power are the main differences between the LTE B41 HPUE co-existence study and NR-LTE co-existence in 3.5GHz.

The summary of simulation parameters for coexistence is shown below.

Table 1 Summary of simulation parameters for coexistence study

|  |  | NR in 3.5GHz |  |  | LTE in 36.886 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Base Station |  | HPUE | Base Station | HPUE |
| Carrier frequency |  | 3500 MHz |  |  | 2600MHz |  |
| Channel bandwidth |  | 100 MHz |  |  | 10MHz |  |
| Inter-site distance |  | 0.75km for urban |  |  | 0.75km for urban |  |
| Cell layout |  | Wrap-around 19 tri-sector cells, uncoordinated |  |  | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Lognormal fading |  | 10 dB |  |  | 10 dB |  |
| Shadowing correlation |  | Between cells: 0.5, between sites: 1.0 |  |  | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) |  | 70 dB (urban areas) |  |  | 70 dB (urban areas) |  |
| Antenna gain and horizontal antenna pattern |  | up to 64 elements [3] | Omni-directional antenna with -3.5 dBi. |  | 17 dBi, = 65 degrees, Am = 20 dB | Omni-directional antenna with -3.5 dBi. |
| Noise figure |  | 5 dB | 9 dB |  | 5 dB | 9 dB |
| Transmit power |  | 73dBm(EIRP) | 26 dBm |  | 46 dBm | 26 dBm |
| Antenna height |  | 20 m | 1.5 m |  | 45 m | 1.5 m |
| ACLR |  | 45dB | Use Table 5.2 in TR 36.942 ACLR1: 20+X, Where X is 1 dB |  | 45 dB | Use Table 5.2 in TR 36.942 ACLR1: 30+X, Where X is 1 dB |
| ACS |  | 45 dB | 27 dB |  | 45 dB | 27 dB |
| SINR to throughput mapping |  | Scaled Shannon's formula. |  |  | Scaled Shannon's formula. |  |
| UL to DL interference |  | no |  |  | Not capture in the TR because synchronous deployment is considered. |  |
| Traffic |  | Full buffer |  |  | Full buffer |  |
| ACI leakage model |  | ACIR = 1/(1/ACS+1/ACLR) |  |  | ACIR = 1/(1/ACS+1/ACLR) |  |
| Performance metric |  | throughput degradation compared to single operator case, i.e. no ACIR. Both mean and 5%-tile are considered. |  |  | throughput degradation compared to single operator case, i.e. no ACIR. Both mean and 5%-tile are considered. |  |
| UL power control model |  | Same as TR 36.886 |  |  | TR 36.886 |  |
| power control algorithm parameters for set 1 | CLx-ile | 105 for 100MHz bandwidth |  |  | TR 36.886 |  |
|  | gamma | 1 |  |  | 1 |  |

- 2.1.2	NR-NR

HPUE co-existence between NR and NR corresponds to the coexistence of two commercial networks operating in adjacent channel with similar deployment parameters. Different from LTE BS, beam forming is deployed for NR BS which will produce directional and high-gain beamformings.

Observation 2: beam forming is deployed for NR BS in NR-NR co-existence scenario.

Coexistence study

In this section, we only focus on UL/UL interference from the aggressor NR HPUE to victim LTE/NR BS in adjacent channel because DL/UL interference can be avoided by using dynamically assigned DL/UL sub-frames. The explanation for the parameters used in the simulations can be found in clause 2.1.1.

- 2.2.1	NR-LTE coexistence

Table 2 shows the summary of additional ACLR for NR-LTE co-existence based on the ACLR for LTE HPUE with 10MHz bandwidth. Furthermore, Table 3 shows the summary of throughput loss.

Table 2 Summary of additional ACLR simulations

| Additional ACLR/dB | 100 MHz |  |
| --- | --- | --- |
| ISD | Avg | 5th percentile |
| 750 meter | -3 | 0 |

Table 3 Summary of throughput loss simulations

| Throughput Loss/% | ACLR offset /100 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ISD | Avg |  |  | 5th percentile |  |
|  | -3dB | 0dB | 1dB | 0dB | 1dB |
| 750 meter | 4.49 | 3.33 | 3.03 | 4.95 | 4.62 |

The simulation results indicate that there will be negligible impact to the ACLR for LTE B41 HPUE co-existence when considering all the differences analyzed in 2.1.1. Therefore, LTE B41 HPUE co-existence results can be reused for 3.5GHz NR HPUE.

Observation 3: For NR HPUE co-existence in NR-LTE scenarios, LTE B41 HPUE co-existence results can be reused for 3.5GHz NR HPUE.

- 2.2.2	NR-NR coexistence

Table 4 and table 5 show the summary of ACLR for HPUE co-existence and the summary of throughput loss in NR-NR scenario.

Table 4 Summary of ACLR simulations

| ACLR/dB | Bandwidth/100 MHz |  |
| --- | --- | --- |
| ISD | Avg | 5th percentile |
| 750 meter | 20 | 27 |

Table 5 Summary of throughput loss simulations

| Throughput Loss/% | Bandwidth/100 MHz |  |
| --- | --- | --- |
| ISD | Avg | 5th percentile |
| 750 meter | 4.2 | 4.9 |

The simulation results indicate that at least 27dB ACLR is needed for HUPE in NR – NR co-existence compared with the 31dB ACLR for LTE HPUE. As analyzed in section 2.1, directional beamformings with higher gain at NR BS are the main reasons for the 4dB less ACLR.

Observation 4: For NR – NR co-existence scenario, at least 27dB ACLR is needed for NR HPUE.

Considering both the LTE-NR co-existence and NR-NR co-existence, 31dB ACLR for NR HPUE is more reasonable to achieve the co-existence between NR and the other systems at adjacent channel. Therefore the ACLR for LTE B41 HPUE co-existence can be reused for 3.5GHz NR HPUE.

Proposal 1: Considering both the LTE-NR co-existence and NR-NR co-existence, 31dB ACLR for NR HPUE is more reasonable. Therefore, LTE B41HPUE co-existence results can be reused for 3.5GHz NR HUPE.

Conclusions

In this paper, we analyze HPUE co-existence in NR-LTE and NR-NR scenarios in 3.5GHz. We have the following observations and proposal:

Observation 1: Different path-loss, power control factor and HPUE transmit power are the main differences between the LTE B41 HPUE co-existence study and NR-LTE co-existence in 3.5GHz.

Observation 2: beam forming is deployed for NR BS in NR-NR co-existence scenario.

Observation 3: For NR HPUE co-existence in NR-LTE scenarios, LTE B41 HPUE co-existence results can be reused for 3.5GHz NR HPUE.

Observation 4: For NR – NR co-existence scenario, at least 27dB ACLR is needed for 3.5GHz NR HPUE.

Proposal 1: Considering both the LTE-NR co-existence and NR-NR co-existence, 31dB ACLR for NR HPUE is more reasonable. Therefore, LTE B41 HPUE co-existence results can be reused for 3.5GHz NR HPUE.

- Reference

- R4-1706068, “WF on high power UE for 3.5GHz”, CMCC

- 3GPP TR 36.942 version 8.2.0 Release 8, “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

- 3GPP TR 37.842, “Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”