3GPP TSG-RAN WG4 NR AH#2	R4-1709304

Nagoya, Japan, 18 – 21 September, 2017

# Source:	Intel Corporation

# Title:	On REFSENS for NR Sub-6 GHz

# Agenda Item:	3.3.4.1

# Document for:	Discussion

# 1	Introduction

In RAN4#84, WF on SA and NSA UE REFSENS for NR Sub-6GHz were agreed. In this contribution, we show our view on SA UE REFSENS for NR Sub-6 GHz range – Bands n77, n78, and n79.

# 2	Discussion

The reference sensitivity level is the minimum mean received signal strength applied to all UE antenna ports at which there is sufficient SINR for the specified modulation scheme to meet a minimum throughput requirement of 95% of the maximum possible.

Reference Sensitivity (REFSENS) is a range of values that can be calculated using the Formula [1]:

REFSENS (dBm)  = -174 dBm + NF + 10log(RxBW) + SNR + IM – Diversity Gain ----- (eq. 1)

- RxBW is the receiver bandwidth per channel BW and SCS

- NF is the prescribed maximum noise figure for the receiver

- SNR is the requirement for the chosen modulation and coding scheme

- Diversity gain represents the diversity gain (2RX)

- IM is the implementation margin including diversity impairment

In our understanding, NF includes RF implementation margin, SNR assumes ideal BB for a fixed code rate of 1/3 which we should revisit after the decision by RAN1. IM includes only BB margin. RAN4 might need to have further clarification on these definitions.

Observation 1: The REFSENS equation still need further clarification which margins, i.e. RF and BB margins, are supposed to be accounted in each of NF, SNR and IM.

The RxBW per channel BW is given in the following table:

Table 1. RxBW per channel BW

Note: Highest RxBW is always for the lowest valid SCS

|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kTB/Hz [dBm] | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| 10log(RxBW) [dB] | 66.5 | 69.7 | 71.5 | 72.8 | 73.8 | 75.9 | 76.9 | 77.7 | 78.9 | 79.9 |
| Diversity Gain [dB] | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| SNR [dB] | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| NF [dB] | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |
| IM [dB] | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| REFSENS (2Rx) [dBm] | -98 | -94.8 | -93 | -91.7 | -90.7 | -88.6 | -87.6 | -86.8 | -85.6 | -84.6 |

Table 2. Derived REFSENS for NR 3.3 – 4.2 GHz (n77 & n78) and 4.4 – 5 GHz (n79)

In the REFSENS calculation in Table 2, -1 dB SNR (QPSK with 1/3 code rate), 11 dB NF, and 2.5 dB IM are assumed across all n77, n78, and n79 bands. NF includes RFFE implementation margin and IM includes BB implementation margin.

Observation 2: Table 2 was derived based on -1 dB SNR (QPSK with 1/3 code rate), 11 dB NF, and 2.5 dB IM assumptions for across all Bands n77, n78, and n79.

# 3	Conclusions

In this contribution, the following observations were made:

Observation 1: The REFSENS equation still need further clarification which margins, i.e. RF and BB margins, are supposed to be accounted in each of NF, SNR and IM.

Observation 2: Table 2 was derived based on -1 dB SNR (QPSK with 1/3 code rate), 11 dB NF, and 2.5 dB IM assumptions for across all Bands n77, n78, and n79.

# 4	Reference

- R4-179149, WF on SA and NSA UE REFSENS for NR Sub-6GHz, Berlin, August 2017